3GPP RAN4 Meeting #112bis		R4-2415393

Hefei, China, October 14-18, 2024

Agenda item:	5.3.4

Source:	Nokia

Title:	Correction of TR 38.719-03-01

Document for:	Approval

# 1	Introduction

In RAN4#112 new TRs with updated guidelines, table structures and rules were introduced. In this contribution we would like to suggest minor changes to the table structures so that they serve the right purpose and eliminate potential confusion. The proposed changes are for TR 38.719-03-01.

# 2	Discussion

## 2.1	Table structure for coexistence studies

A new table structure in both 38.719-02-01 and 38.719-03-01 was introduced to study 2UL with 2 CCs, often called triple-beat. The triple beat analysis is based on an uplink configuration that combines inter-band with intra-band CA. In the table there is a clear section for the intra-band and the inter-band, but whereas the table is fine for 38.719-02-01 it does not serve 38.719-03-01. An excerpt of the section of the TR is shown in the following.

- - - - - - - Start excerpt from TR 38.719-03-01 - - - - - - -

##### 5.x.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Editor’s Note: Since the triple beat tables have already been calculated for the different two band fallbacks, the tables below may skip the IMD calculations and refer to the relevant two band TP tables. Nonetheless, the IMD issues should be stated, and the related TPs referenced. Only one example table is provided in this template but there may be up to three cases for the victim DL band and as many tables may be needed.

Table 5.x.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band nZ interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXX MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | – |  | – |  |
| fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the two UL band with intra-band UL CA triple beat shall be included here, even if no issues are identified. |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

- - - - - - - End excerpt from TR 38.719-03-01 - - - - - - -

First it is noted that the TR list the maximum instantaneous bandwidth as “XXX” and it is believed that that should be updated/clarified in the TR.

Observation 1:	The maximum instantaneous bandwidth should be updated from XXX in the TR.

Another more important observation is that this table does not work for 3CA combinations, since fDL is not of nX, but nZ as written in the text above the table. The check in 38.719-03-01 is for the 3rd band always, while the check in 38.719-02-01 is always for the single carrier band (nX). If fDL is populated for nX, it is the same analysis as conducted in the 2CA fallback.

Observation 2:	If fDL is populated for nX, it is the same analysis as conducted in the 2CA fallback.

We propose a solution to correct the single carrier information in 38.719-03-01 with a minimal of impact to table structure as presented in TP 1.

- - - - - - - Start TP 1 - - - - - - -

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | single carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| nX fUL | – |  | – |  |
| nZ fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the two UL band with intra-band UL CA triple beat shall be included here, even if no issues are identified. |  |  |  |

- - - - - - - End TP 1 - - - - - - -

Proposal 1:	Correct the single carrier information in 38.719-03-01 with a minimal of impact to table structure as presented in TP 1.

Additionally, Note 1 does not work for 3CA combinations, since it is no longer a check between nX and CA_nYB/C, but now involves nZ, so even if the two uplink bands are of same or adjacent band groups, it doesn’t consider the placement of the downlink location of the third band.

Observation 2:	Note 1 does not work for 3CA combinations, since it is no longer a check between nX and CA_nYB/C, but now involves nZ, so even if the two uplink bands are of same or adjacent band groups, it doesn’t consider the placement of the downlink location of the third band.

Note 1 of Table 5.x.2.1.2-1 must be rewritten, or a new Note must be added, that serves 3CA with impact on the third downlink band. In this relation we would like an agreement between companies if also for the 3CA case we must consider all bands to be of a same, or adjacent band group, or if it only considers the uplink bands.

- - - - - - - Start TP 2 - - - - - - -

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | single carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| nX fUL | – |  | – |  |
| nZ fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the two UL band with intra-band UL CA triple beat shall be included here, even if no issues are identified. |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in table A.1, of band one (nX) or two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

- - - - - - - End TP 2 - - - - - - -

Proposal 2:	Update Note 1 of Table 5.x.2.1.2-1, such it serves 3CA with impact on the third downlink band, as presented in TP 2

These two TPs are to be implemented into TR 38.719-03-01 to ensure correct guidance for the 3CA combinations.

Proposal 3:	Implement TP 1 and TP2 into the revised version of TR 38.719-03-01.

# 3	Conclusions

This contribution describes the relevant changes to TR 38.719-03-01 triple beat analysis:

Observation 1:	The maximum instantaneous bandwidth should be updated from XXX in the TR.

Observation 2:	If fDL is populated for nX, it is the same analysis as conducted in the 2CA fallback.

Observation 2:	Note 1 does not work for 3CA combinations, since it is no longer a check between nX and CA_nYB/C, but now involves nZ, so even if the two uplink bands are of same or adjacent band groups, it doesn’t consider the placement of the downlink location of the third band.

Proposal 1:	Correct the single carrier information in 38.719-03-01 with a minimal of impact to table structure as presented in TP 1.

Proposal 2:	Update Note 1 of Table 5.x.2.1.2-1, such it serves 3CA with impact on the third downlink band, as presented in TP 2

Proposal 3:	Implement TP 1 and TP2 into the revised version of TR 38.719-03-01.

# 4	References