3GPP TSG-RAN WG4 Meeting # 112-bis					R4-2415423

Hefei Meeting, Oct. 14th – Oct 18th, 2024

Title:	TP to TR 38.719-03-01 CA_n1-n71-n78 w ULCA

Source:	Nokia, Etisalat UAE

Agenda item:	5.3.4

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-03-01 to add CA_n1-n71-n78 with intra- and -interband uplink CA.

************************************* Start of TP*****************************************

## 5.x	CA_n1-n71-n78

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920-1980 | 2110-2170 | FDD |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n71A-n78A | CA_n1A-n71A CA_n1A-n78A CA_n71A-n78A | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | 10,15,20,25,30,40,50,60,70,80,90,100 |  |
| CA_n1A-n71A-n78C | CA_n78C CA_n1A-n71A CA_n1A-n78A CA_n1A-n78C CA_n71A-n78A CA_n71A-n78C | n1 | 5,10,15,20,25,30,40,45,50 | 0 |
|  |  | n71 | 5,10,15,20 |  |
|  |  | n78 | CA_n78C_BCS 4 and 5 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n71-n78, the TIB,c and RIB,c values are re-used from CA_n1-n78-n105 given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | 0.3 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n1-n71-n78 | - | 0.2 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

##### 5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

Table 5.x.2.1.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n71A.

Table 5.x.2.1.1-1: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5166 - 5356 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.x.2.1.1-2 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n1A-n78A.

Table 5.x.2.1.1-2: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 - 1880 |  | 5220 - 5780 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 - 660 |  | 4620 - 5680 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 - 7760 |  | 8520 - 9580 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 2640 |  | 7920 - 9480 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3760 - 2640 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 - 9740 |  | 11820 - 13380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10440 - 11560 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 - 11220 |  | 4620 - 3880 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 - 5940 |  | 660 - 1840 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 - 17180 |  | 10980 - 11720 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 - 15360 |  | 12360 - 13540 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Table 5.x.2.1.1-3 provides the two UL bands with one CC per band IMD interference analysis for CA_n1A-n71A-n78A with UL CA_n71A-n78A.

Table 5.x.2.1.1-3: Two UL bands IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 - 3137 |  | 3963 - 4498 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2474 - 1904 |  | 5902 - 6937 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 - 5196 |  | 7263 - 8298 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1811 - 1206 |  | 9202 - 10737 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6274 - 5204 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5289 - 5894 |  | 10563 - 12098 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7926 - 8996 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14537 - 12502 |  | 508 - 1148 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10074 - 8504 |  | 4506 - 5611 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 - 15898 |  | 5952 - 6592 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 - 12796 |  | 8589 - 9694 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the analysis in Table 5.x.2.1.1-1 there is IMD3 from n1 and n71 into RX band n78.

Based on the analysis in Table 5.x.2.1.1-2 there is IMD3 from n1 and n78 into RX band n71.

Based on the analysis in Table 5.x.2.1.1-3 there is IMD3 from n71 and n78 into RX band n1.

##### 5.x.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.x.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n71 interference analysis for CA_n1A-n78C with n1/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n1 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n1 fUL | 1920-1980 |  | 3300-3800 |  |
| n71 fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1720-1920 |  | 1980-2180 |  |
| Analysis | Note 1 applies. The bands are not of adjacent band groups |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.x.2.1.2-2 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n1 interference analysis for CA_n71A -n78C with n71/78C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-2: Two UL band with intra-band ULCA Triple beat analysis

| Band / CA1 | n71 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| n71 fUL | 663-698 |  | 3300-3800 |  |
| n1 fDL | 2110-2170 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | Note 1 applies. The bands are not of adjacent band groups |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 5.x.2.2	REFSENS requirements

The MSD requirements are re-used from CA_n1-n78-n105.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71-n78 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | N/A | 635 | 15.2 | FDD | IMD3 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3342 | 15.7 | TDD | IMD3 |
|  | n1 | N/A | 5 | N/A | 2160 | 15.7 | FDD | IMD3 |
|  | n71 | 686 | 5 | 25 | 640 | N/A | FDD | N/A |
|  | n78 | 3532 | 10 | 50 | 3532 | N/A | TDD | N/A |

************************************* End of TP******************************************