3GPP TSG-RAN WG4 Meeting # 112bis								    							 R4-2415433

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item: 6.1.1.3.2

Source: MediaTek Inc.

Title: Feasibility evaluation of MIMO layer for 6Rx UE

Document for: Discussion

# Introduction

The WI NR_ENDC_RF_Ph4 [1] was approved in RP#103 meeting with one objective “6Rx for handheld and FWA UE” within which one task to RAN4 is to study the gain and feasibility of 6 MIMO layers, targeting at support of handheld UEs.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112 meeting, the discussions continued. An WF [2] was agreed with the relevant part captured as below. In [2], some of the link-level simulation assumptions were agreed. We copied the detail table in the Appendix.

# Topic 3: MIMO layer evaluation for 6Rx UE ## Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Agreement: - Do not tighten the minimum requirements of BS Tx EVM - Further discuss the assumption of Tx EVM for 6-layer performance evaluation. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-4: Performance requirements for 6Rx Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. ## Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

In this paper, we conducted link-level evaluations for the feasibility of 6 layer as well as more evaluated scenarios in system-level simulation.

# Link-level simulation

## 2.1 Discussion for simulation assumptions

MIMO correlation and antenna efficiency

Based on the WF [2], RAN4 agreed to use link level simulation for performance evaluations based on the assumptions in Appendix A. There were two options for correlation matrices provided from OPPO (Option 1) [3] and HW [4] (Option 2) [4] in the lasting meeting. The values provided by OPPO is ECC (envelop correlation coefficient) values, which measures how correlated two different antennas’ radiation patterns are [5]. According to [6], the correlation coefficient for antenna pairs depends on ECC and also antenna efficiency, i.e., ρpq=ηpηqECCp, q, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q. Therefore, ECC values are not the correlation coefficients used in RAN4 demodulation simulation. We recommend RAN4 to further study how to link between the ECC values and the MIMO correlation model in TS 38.101-4.

Observation 1: The values in UE correlation matrix of Option 1 is ECC, not correlation coefficient.

Observation 2: The correlation coefficient for antenna pairs depends on ECC and also antenna efficiency, i.e., ρpq=ηpηqECCp, q, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

Proposal 1: RAN4 to further discuss how to link between the ECC values and the MIMO correlation model in TS 38.101-4

In RAN4 simulation, the diagonal terms in the MIMO correlation are always 1, which is fine when the test is carried out in a conducted manner. Now in the current feasibility study of 6 layer, companies propose the ECC measurement results of each antenna and suggest using the measurement results for evaluation. In this case, the assumption of all-1 diagonal terms does not hold anymore because the antenna efficiency is not possible to be perfect for all antennas. Therefore, it is fair to also consider the antenna efficiency in this feasibility study.

Observation 3: As RAN4 is considering using real antenna performance by adapting the measured ECC values into the link-level simulation, the antenna efficiency needs to be considered as well.

Proposal 2: RAN4 to consider using realistic antenna efficiency in the link-level simulation.

In TS38.101-4, the spatial correlation coefficient in correlation matrices, rij, between the ith and the jth antenna element is expressed as (reflect the nonlinear decaying rate of the spatial correlation)

rij=ρdijΔ2,                                (1)

Where dij is the antenna element separation, ρ is the correlation at a reference element separation of Δ [7]. Based on (1), the gNB correlation and UE correlation for one antenna, two antennas and four antennas are defined in Clause B.2.3.1.1 with the number of antennas N and dij=(i-j)Δ(N-1).

Table 1. gNB correlation matrix in TS38.101-4

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| gNB Correlation |  |  |  |

Table 2. UE correlation matrix in TS38.101-4

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| UE Correlation |  |  |  |

Observation 4: In TS38.101-4, the spatial correlation coefficient in correlation matrices, rij, between the ith and the jth antenna element is expressed as rij=ρdijΔ2, where dij=(i-j)Δ(N-1) is the antenna element separation, ρ is the correlation at a reference element separation of Δ and N is the number of antenna.

Based on Observation 1, we derived the MIMO correlation matrix for 6Rx (N = 6) as follows.

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β425*β925*β425*β125*1

Also, the pragmatic approach was used in RAN4 to introduce MIMO correlation matrix for performance test, where the pragmatic approach means that no array details (except the number of elements) are considered in the model, and the correlation matrices, based on the Kronecker product  of the correlation matrices at gNB  and UE , are specified for distinct levels in Table B.2.3.1.2-1 in TS38.101-4.

Table 3. The α and β parameters for ULA MIMO correlation matrices in TS38.101-4

| Correlation Model |  |  |
| --- | --- | --- |
| Low correlation | 0 | 0 |
| Medium Correlation | 0.3 | 0.9 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |
| High Correlation | 0.9 | 0.9 |

The justification to define β = 0.005154 for 8Rx in Medium Correlation B is to attain the same correlation value for 4Rx in Medium Correlation A between two adjacent antennas, i.e., β4Rx1/9=β8Rx1/49. This implies that 8Rx in Medium Correlation B would double the size/form factor compared to 4Rx in Medium Correlation A. Considering 6Rx foldable phones, we assume that size/form factor of 6Rx UE is larger than 4Rx UE and introduce “Medium Correlation C” with β = 0.00718 for 6Rx to attain the same correlation value for 4Rx in Medium Correlation A between two adjacent antennas, i.e., β4Rx1/9=β6Rx1/25.

Observation 5: Following the approach defining MIMO correlation matrix in TS38.101-4, we can assume that size/form factor of 6Rx UE is larger than 4Rx UE and introduce “Medium Correlation C” with β = 0.00718 for 6Rx to attain the same correlation value as 4Rx in Medium Correlation A between two adjacent antennas.

Proposal 3: RAN4 to further discuss whether and how to follow the approach in TS38.101-4 to introduce MIMO correlation for 6Rx.

DMRS configuration

There are three options for DMRS configuration in the table of simulation assumptions, which are

- Option 1: 1+1 (4L), 2+0 (6L)

- Option 2: 2+2 (4L), 2+2 (6L)

- Option 3: 1+1 (4L), 2+2 (6L)

The additional DMRS can improve the accuracy of channel estimation and results in higher throughput, especially for high-speed mobility conditions where channel conditions vary rapidly. In [7], it was agreed to use 1 additional DMRS for low (5/10Hz) and medium (100 Hz) speed scenarios.

- DMRS - Type 1 single symbol front loaded - 1 additional DMRS for Low (5/10Hz) and Medium (100 Hz) Speed scenarios - i.e. for full DL slots OFDM symbols with DMRS: #2 and #9 - additional DMRSs for High (400 Hz) Speed scenarios - i.e. for full DL slots OFDM symbols with DMRS: #2, #6 and #9 - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers

Therefore, we prefer to use DMRS configuration with additional DMRS for performance evaluation. Besides, for SU-MIMO transmission with 4 layers, the typical configuration is using antenna port 1000, 1001, 1002 and 1003 for PDSCH and DMRS transmission. Hence, we propose to use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

Observation 6: The additional DMRS can improve the accuracy of channel estimation and results in higher throughput

Observation 7: In TS38.101-4, all PDSCH requirements are configured with additional DMRS except PDSCH mapping type B

Observation 8: In TS38.101-4, antenna ports indexes {1000-1003} are used for Rank 4 requirements

Proposal 4: Use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

MCS table

The TBS and the maximum achievable throughput is related to MCS. If we go for some fixed MCS, it means that the TBS is fixed during the simulation. However, it is more realistic to have dynamic MCS selection to achieve higher throughput by considering channel quality under different SNRs.

Also, MCS Table 1 limits the modulation order to be up to 64QAM. However, DL 256QAM is already widely used in all FR1 deployments. It is important to consider 256QAM in the feasibility study of 6-layer MIMO. Therefore, we suggest using adaptive MCS (target BLER 10%) with MCS Table2.

Observation 9: Fixed MCS assumptions will restrict the maximum throughput that UE can achieve and difficult to tell whether 6L can provide higher throughput than 4L.

Proposal 5: Instead of using some fixed MCS, we propose to use adaptive MCS with MCS Table 2 for feasibility evaluation.

Tx EVM

It was agreed in [2] that the BS Tx EVM should not be tightened. According to Table 6.5.2.2-1 of TS 38.104, the BS EVM requirements is 3.5% for 256QAM.

Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

In RAN4 PDSCH and SDR requirements, we typically pick the Tx EVM smaller than the requirement in TS 38.104. In [7], different Tx EVM values for different modulation order were agreed:

- Use the following RF impairments models to define the minimum UE performance requirements for FR1 - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM

Following the previous agreement, we propose to use Tx EVM = 3% for all of modulation order in MCS Table 2.

Proposal 6: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation.

## 2.2 Simulation results

We provide simulation results in Figure 1 based on assumptions in Table 4. As we mentioned in previous section, we suggest RAN4 to study how to link the ECC values to the MIMO correlation model in TS 38.101-4 and to consider antenna efficiency in the link-level simulation. At this moment, since there is no new consensus on how to handle these issues, we tentatively adopted the UE correlation matrix Option 1 and Option 2 for evaluation. However, new simulations need to be conducted again after we conclude the ECC and antenna efficiency issues.

Table 4. Simulation assumptions for 6Rx performance evaluation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | 8Tx, 6Rx |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank 4, 5, 6 |
| MCS |  | Adaptive MCS (target BLER 10%) |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | 1+1 (4L), 2+2 (6L) |
|  | Precoding | Random |
| BS Correlation matrix |  | Identity matrix with dimension of 8x8 (uncorrelated) |
| UE Correlation matrix |  | Option 1: from HW (R4-2412572) 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 2: Medium C 1.000.900.901.000.660.90 0.660.390.900.661.000.90 0.190.070.390.190.660.390.390.660.190.390.070.19 0.901.000.660.900.390.66 0.900.661.000.900.901.00 Note: Assume antenna efficiency = 1. This needs to be changed based on realistic values |

With correlation matrix form HW and “Medium C”, it shows that the throughput of rank 5 and 6 is lower than that of rank 4 considering 3% Tx EVM.

Figure 2-1. Link level simulation results

Observation 10: With correlation matrix form HW and “Medium C”, our simulation results show that the throughput of rank 5 and 6 is lower than that of rank 4 considering 3% Tx EVM.

# System-level simulation

On system-level simulation aspect, we provide further evaluations with more different scenarios based on our previous contribution [8]. The simulation assumptions are provided in Table 3-1.

Table 3-1 Parameters for system-level simulation

| Scenario |  | UMa |  | UMi | InH |
| --- | --- | --- | --- | --- | --- |
| Parameters |  |  |  |  |  |
| Inter-site distance (ISD) |  | 500m | 200m | 200m | 60m |
| site/cell number |  | (7,21) |  |  | (3,3) |
| TRxP within 1 site |  | 3 |  |  | 1 |
| UE distribution |  | - 80% indoor UE, UE speed 3km/h - 20% indoor UE, UE speed 30km/h |  |  | UE speed 3km/h |
| Carrier freq |  | 3.5GHz |  |  |  |
| Traffic Model |  | FTP traffic model with resource utilizations (RU) rates around 30% |  |  |  |
| Transmission scheme |  | - Max MIMO layer: 4 or 6 - SU-MIMO, up to 256QAM |  |  |  |
| PDSCH DMRS |  | - 1-symbol overhead for MIMO layer |  |  |  |
| CSI |  | - CSI period 30ms, SRS period 10ms - OLLA with target BLER=0.1 |  |  |  |
| BS | Height/tilting angle | 25m/15° |  | 19m/12° | 3m/0° |
|  | Antenna configuration M,N,P,Mg,Ng,Mp,Np | 32 TxRU (8,8,2,1,1,2,8) |  |  | 16 TxRU (8,4,2,1,1,2,4) |
|  | Tx Power/Ant gain | 43dBm/8dBi |  |  | 23dBm/5dBi |
|  | EVM | 0% |  |  |  |
| UE | Ant. type & config. | ULA, 6Rx |  |  |  |
|  | Ant. distance | 0.5λor 0.3λ |  |  |  |

Figures 3-1(a) provides the throughput of ‘up to 6 layer’ (6L) over throughput of ‘up to 4 layer’ (4L) for UMa with ISD=500m. We considered UE antenna distances of 0.5λ and 0.3λ. The percentages of >4L usage are provided in Figure 3-1(b).

| (a) | (b) |
| --- | --- |

Figure 3-1. Evaluation results for UMa, ISD=500m

Figures 3-2(a) and 3-2(b) provides the corresponding results for UMa with ISD=200m. Figures 3-3(a) and 3-3(b) provides the corresponding results for UMi with ISD=200m. Figures 3-4(a) and 3-4(b) provides the corresponding results for InH with ISD=60m.

| (a) | (b) |
| --- | --- |

Figure 3-2. Evaluation results for UMa, ISD=200m

| (a) | (b) |
| --- | --- |

Figure 3-3. Evaluation results for UMi, ISD=200m

| (a) | (b) |
| --- | --- |

Figure 3-4. Evaluation results for InH, ISD=60m

From above results, we can observe:

- In marco cell there is almost zero benefit to support up to 6 layers.

- With UE antenna distance 0.3λ, there is almost zero benefit to support up to 6 layers in non-InH scenario. The only scenario with cell average throughput gain by allowing 6 layers for 6Rx is InH with antenna distance 0.5λ. However, this requires the indoor BS to equipped with at least 8 TxRU.

- The gain of supporting up to 6L is only limited to InH or cell-centre UEs of medium cells with UE antenna distance 0.5λ. The usage ratio of 5 and 6 layer is around 14% in ISD 500m cases, 23% in ISD 200m and 31% in InH. However, such a usage ratio does not bring throughput benefit to the same scale. The main reason, as explained in [8], is that 5 and 6 layers are very sensitive to channel aging and interference uncertainty. Even if 5 or 6 layer is preferred at the moment when UE estimates the CSI (or estimated by BS via SRS), the latency to apply the new CSI brings the uncertainty of channel and interference changes. Then OLLA algorithm is likely to take over and decreases the MCS of the 5/6-layer PDSCH. Therefore, the benefit of 5/6-layer PDSCH is not as high as expected.

- In above simulation, the DMRS overhead are assumed to be always the same, i.e., 1 symbol. However, with additional double-symbol DMRS for supporting 5 & 6 layers, only 10 symbols remaining for PDSCH. It means extra 9% overhead to be considered.

- In above simulation, BS Tx EVM is assumed to be 0%. However, according to our previous evaluation in [8], assuming Tx EVM 3.5% will bring 4.3% and 2.8% throughput loss in 6L for cell average and 95%-ile cases, respectively.

Observation 11: From system-level simulation, there is no benefit to support up to 6 layer for 6Rx in marco cell (ISD = 200m or 500m) deployment

Observation 12: From system-level simulation, the only scenario with cell average throughput gain by allowing 6 layers for 6Rx is InH with antenna distance 0.5λ, where the gain is around 5%. However, this requires the indoor BS to equipped with at least 8 TxRU

# Summary

In this paper, we provided our view on the feasibility of MIMO layer for 6Rx UE. We have the following observations and proposal.

Observation 1: The values in UE correlation matrix of Option 1 is ECC, not correlation coefficient.

Observation 2: The correlation coefficient for antenna pairs depends on ECC and also antenna efficiency, i.e., ρpq=ηpηqECCp, q, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

Proposal 1: RAN4 to further discuss how to link between the ECC values and the MIMO correlation model in TS 38.101-4

Observation 3: As RAN4 is considering using real antenna performance by adapting the measured ECC values into the link-level simulation, the antenna efficiency needs to be considered as well.

Proposal 4: Use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

Observation 4: In TS38.101-4, the spatial correlation coefficient in correlation matrices, rij, between the ith and the jth antenna element is expressed as rij=ρdijΔ2, where dij=(i-j)Δ(N-1) is the antenna element separation, ρ is the correlation at a reference element separation of Δ and N is the number of antenna.

Observation 5: Following the approach defining MIMO correlation matrix in TS38.101-4, we can assume that size/form factor of 6Rx UE is larger than 4Rx UE and introduce “Medium Correlation C” with β = 0.00718 for 6Rx to attain the same correlation value as 4Rx in Medium Correlation A between two adjacent antennas.

Proposal 3: RAN4 to further discuss whether and how to follow the approach in TS38.101-4 to introduce MIMO correlation for 6Rx.

Observation 6: The additional DMRS can improve the accuracy of channel estimation and results in higher throughput

Observation 7: In TS38.101-4, all PDSCH requirements are configured with additional DMRS except PDSCH mapping type B

Observation 8: In TS38.101-4, antenna ports indexes {1000-1003} are used for Rank 4 requirements

Proposal 4: Use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

Observation 9: Fixed MCS assumptions will restrict the maximum throughput that UE can achieve and difficult to tell whether 6L can provide higher throughput than 4L.

Proposal 5: Instead of using some fixed MCS, we propose to use adaptive MCS with MCS Table 2 for feasibility evaluation.

Proposal 6: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation.

Observation 10: With correlation matrix form HW and “Medium C”, our simulation results show that the throughput of rank 5 and 6 is lower than that of rank 4 considering 3% Tx EVM.

Observation 11: From system-level simulation, there is no benefit to support up to 6 layer for 6Rx in marco cell (ISD = 200m or 500m) deployment

Observation 12: From system-level simulation, the only scenario with cell average throughput gain by allowing 6 layers for 6Rx is InH with antenna distance 0.5λ, where the gain is around 5%. However, this requires the indoor BS to equipped with at least 8 TxRU

# Reference

[1] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon

[2] R4-2414285, WF on 6Rx UE requirements, AT&T

[3] R4-2412352, R19 Simulation results of 6Layer performance, OPPO

[4] R4-2412572, On MIMO layer evaluation for 6Rx UE, Huawei

[5] Y. Kabiri, A. L. Borja, J. R. Kelly, and P. Xiao, ‘‘A technique for MIMO antenna design with flexible element number and pattern diversity,’’ IEEE Access, vol. 7, pp. 86157–86167, 2019.

[6] X. Mei and K.-L. Wu, “Capacity-oriented envelope correlation coefficient for multiple antennas of mobile devices,” IEEE Trans. Antennas Propag., vol. 70, no. 10, pp. 9785–9794, Oct. 2022.

[7] R4-1811394, Way forward on NR UE PDSCH demodulation requirements and General aspects, Intel

[8] R4-2411773, Feasibility evaluation of MIMO layer for 6Rx UE, MediaTek inc.

# Appendix A: Feasibility Study Link-Level Simulation Assumptions

Table: Feasibility Study Link-Level Simulation Assumption

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz or 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 1: TDLA30-5 Option 2: TDLA30-10 Option 3: TDLC300-100 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Option 1: Adaptive MCS (target BLER 10%) Option 2: MCS13, MCS17 from Table 1 Other MCS options are not precluded |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 2: [2+2(4L), 2+2(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random (further check) |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: Other options are not precluded. |