# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415447

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			A-IoT BS RF overview

Agenda item:		6.21.3.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements.

# Discussion

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the A-IoT BS. These factors are listed in Table 1 below. As RAN4 agree that there is no frequency shifting for D2R transmission from device side, the DL spectrum will be used for D2R when CW transmission occurs in DL spectrum. The A-IoT BS can transmit CW transmission with the same hardware and in this paper, the shared hardware architecture is used for this option. When another node other than A-IoT BS transmits the CW signal, we say it is separated hardware architecture.

Use the “shared hardware architecture” or “separated hardware architecture” to represent either CW is transmitted by A-IoT BS or another node.

RAN4 study the RF impact for shared and separated hardware architecture.

Table 1: Factors impacting the A-IoT BS operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | CW transmission power at antenna | Shared hardware or different hardware with CW transmission | CW power at antenna of A-IoT BS | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | 33 dBm | Yes | 33 dBm |  |
|  | DL spectrum | DL spectrum | DL spectrum | 33 dBm | No | >3 dBm and <33 dBm | Antenna isolation between 10 and 30 dB |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | Yes | 23 dBm |  |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | No | >- 7dBm and <13 dBm | Antenna isolation between 10 and 30 dB |

From the Table 1, it can be observed that for different spectrum usage for A-IoT operation. As CW transmission will interfere the R2D reception, the R2D and D2R should be TDM. If a FDD band would be defined, it should be half duplex mode. As it is not clear how the spectrum will be used and it may depend on coexisting study, we recommend prioritizing the FDD band in half duplex mode in RF requirement discussion.

RAN4 prioritize the FDD band in half duplex mode in RF requirement discussion.

For shared and separated architecture, CW signal needs to be transmitted and the CW waveform and generation has been discussed in RAN1 with below agreements:

RAN1#116:

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

RAN1#116bis:

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

From previous discussion, the FDD band in half duplex mode should be prioritized because otherwise the simultaneous CW and R2D transmission may interfere the R2D signal reception at device 1/2a. Therefore, the R2D and CW transmission is TDM and CW signal transmission requirement should be discussed separately with R2D transmission.

CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

CW and R2D transmission requirement can be discussed separately.

For A-IoT R2D transmitter requirement, as the R2D signal design very similar with LP-WUS signal design, for example, he OOK1 and OOK4 design are following the LP-WUS signal design. therefore, the transmitter requirement can depend on the LP-WUS work item conclusion and there is no need to discuss it separately in A-IoT study.

No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

Then only the CW transmission requirements are relevant in study phase: the CW signal is new compared to legacy NR BS. Therefore, RF requirement relating to the CW signal should be discussed for transmitter requirement. This applies to both shared and separated hardware architecture. For separated architecture, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission are relevant requirement. As there is modulated signal interferer in coexisting study. Also the inhibition of the reverse IM product from co-located/nearby CW transmitter should be applied as multiple tone of CW is allowed in RAN1.

For separated hardware for CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

The new test model of CW transmission needs to be defined. According to RAN1 design, the CW waveform is different with R2D/D2R waveform, and it is unmodulated sinusoid waveform according to RAN1 agreement. However, the bandwidth of the CW signal is not defined by RAN1. The discussion relates to the D2R modulation bandwidth, Figure 1 illustrates the PSD of the D2R modulated signal according to RAN1 discussion. It can be observed that CW signal bandwidth should be narrow as the modulated signal is around +/- 100kHz.  But exactly BW to test could be decided in work item phase.

For shared hardware architecture, discuss the CW signal test model in work item phase.

Figure 1: power spectrum of D2R modulated signal

As the CW transmission power may be defined with certain bandwidth, this may not be the same as the A-IoT R2D signal transmission power. If following the NR BS power dynamic range, the output power may be scaled with the transmission bandwidth, therefore, if CW transmission bandwidth is one tone (e.g 15kHz), the CW transmission power would be maximum carrier power – 10.7 dB (e. g 10log (12/1)). So the CW transmission power would be lower if CW signal occupied bandwidth would be smaller than A-IoT R2D signal bandwidth. However, this is not expected from coexisting simulation. Our understanding is that CW signal power should be 33 dBm or 23 dBm even with undefined bandwidth. It can be done with current BS specification framework if the CW transmission bandwidth would be different with A-IoT R2D bandwidth and two different carrier power would be possible to be configured and tested.  As CW and A-IoT R2D signal is TDM operation, transmitting CW and A-IoT R2D with different power with two different channel bandwidth seems possible. However, there may be some re-configuration time needed and there may be some transient time between R2D and CW signal transmission. RAN4 could discuss whether to define a new timing mask for this case for output power dynamic requirement.

Different channel bandwidth may be defined for CW and A-IoT R2D transmission for shared architecture.

A new timing mask may be needed to test outpower dynamic for shared architecture.

No matter how to define the operating bands, from RF perspective, the A-IoT BS needs to tolerate CW signal while receiving the D2R signal. This is true for either shared hardware or separated hardware architecture. For receiver requirement, as the coexisting scenario have both inside and outside topology, it should discuss whether two different set of the RF requirements should be defined corresponding to the either inside topology or outside topology. For inside topology, the receiver needs to tolerate the high leakage CW power either from the transmitter for shared hardware architecture or co-located CW transmission node. For outside topology, CW transmission is at least the distance of the A-IoT BS for D1T1-B, so the received CW power would be low compared to inside topology case and as such it would be less issue for receiver to decode the backscattered signal and receiver linearity is not demanding. In our companion discussion paper for CW cancellation, the additional CW cancellation circuit would be expected if the linearity of the receiver would be compromised. Such would not be needed for outside topology where the leakage power from CW transmission is in par or smaller than backscattered signal. These considerations motivate different receiver architecture depending whether CW inside topology or outside topology is deployed.

Define two sets of the A-IoT BS receiver requirement corresponding to inside topology and outside topology respectively.

The benefit of the defining two sets of receiver requirement can be the reader range. For the REFSENS designed for inside topology, the SNR degradation would be allowed as the high leakage power desensitize the receiver. For the REFSENS designed for outside topology, the need of further relaxing the REFSENS may not be needed therefore larger Reader to device range could be achieved. There are more factors to consider for REFSENS discussion, for example, the energy harvesting sensitivity of device 1 may limit the R2D distance and the min received power may be decided by R2D distance. Such discussion should be in the context of the link budget. Another example, the coexisting may set the limit on the cell range and therefore, the reader to device distance further be limited. For these factors to impact the REFSENS definition, RAN4 can identity these factors in study phase and further address it in work item phase to define the REFSENS.

Two different sets of receiver requirement may target to different reader to device range.

Both link budget and coexisting impact the REFSENS definition.

REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

For the receiver requirements defined for inside topology, the tolerance of the maximum CW power at antenna can be declared by the manufacture. The REFSENS should be tested with a CW signal coupled to the antenna via OTA if A-IoT BS cannot transit the CW signal, how to test it can be further discussed in work item phase. If the A-IoT BS can transit the CW signal, the REFSENS can be tested with CW transmitting at the same time.

REFSENS for inside topology should be tested with the CW signal either transmitted in BS or coupled with OTA.

- For the receiver requirements defined for outside topology, it can be discussed whether there is no need to test it with a CW signal. In current D1T1-B coexisting simulation, the CW is two-cell radii away and PSD difference between backscattered power and CW signal may depend on the backscatter loss, e.g 6 dB for OOK in RAN1 assumption. The close-in noise from CW signal should not distort the backscattered signal and this should be the same with inside topology. Further investigate the PSD difference between CW signal and D2R signal for outside topology.

Further discuss if there is a need to test the REFSENS with CW signal or not.

- For ACS and inband block requirements, it depends on the coexisting study. For the ICS (in-channel selectivity), currently only single tone and single A-IoT UE D2R reception is agreed in RAN1 and FDMA for multiple UE is under discussion. RAN4 can further wait RAN1 progress to discuss this.

Wait further Ran1 progress to discuss the ICS requirements.

TP is started here:

### 6.7.2	Ambient IoT BS

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the A-IoT BS. These factors are listed in Table 1 below. CW signal can be transmitted by A-IoT BS or by a separate node, therefore, the shared or separated hardware architecture for CW transmission can be discussed with CW related RF requirements.

Table 2: Factors impacting the A-IoT BS operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | CW transmission power at antenna | Shared hardware with A-IoT BS or different hardware for CW transmission |
| --- | --- | --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum | 33 dBm | Shared hardware |
|  | DL spectrum | DL spectrum | DL spectrum | 33 dBm | Separated hardware |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | Shared hardware |
|  | DL spectrum | UL spectrum | UL spectrum | 23 dBm | Separated hardware |

Based on the RAN1 and RAN agreements, the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant to RF requirement definition and it is necessary to specify the RF characteristic of CW signal based on these agreements. As CW transmission may interfere the R2D reception, the R2D and D2R transmission should be TDM, therefore, the R2D transmission requirement can be discussed separately with CW signal related requirements.

For transmitter requirement related to R2D signal, as the R2D signal design very similar with LP-WUS signal design, for example, he OOK1 and OOK4 design are following the LP-WUS signal design. therefore, the transmitter requirement can depend on the LP-WUS signal RF requirement specification in LP-WUS work item.

The A-IoT transmitter requirements related to CW signal can be specified separately. When a separate hardware is used to generate CW signal other than A-IoT BS, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and transmitter intermodulation can be specified. When CW signal is generated by A-IoT BS, a new test model for CW signal needs to be defined. The transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and transmitter intermodulation can be specified for CW signal testing in this case.

For receiver requirements, two sets of the A-IoT BS receiver requirements corresponding to inside topology and outside topology respectively should be defined.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Proposal-1: Use the “shared hardware architecture” or “separated hardware architecture” to represent either CW is transmitted by A-IoT BS or another node.

Proposal-2: RAN4 study the RF impact for shared and separated hardware architecture.

Proposal-3: RAN4 prioritize the FDD band in half duplex mode in RF requirement discussion.

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 3 CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Observation 4 CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

Observation 5 CW and R2D transmission requirement can be discussed separately.

Proposal-4: No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

Proposal-5: For separated hardware for CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

Proposal-6: For shared hardware architecture, discuss the CW signal test model in work item phase.

Observation 6 Different channel bandwidth may be defined for CW and A-IoT R2D transmission for shared architecture.

Observation 7 A new timing mask may be needed to test outpower dynamic for shared architecture.

Proposal-7: Define two sets of the A-IoT BS receiver requirement corresponding to inside topology and outside topology respectively.

Observation 8 Two different sets of receiver requirement may target to different reader to device range.

Observation 9 Both link budget and coexisting impact the REFSENS definition.

Proposal-8: REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

Proposal-9: REFSENS for inside topology should be tested with the CW signal either transmitted in BS or coupled with OTA.

Proposal-10: Further discuss if there is a need to test the REFSENS with CW signal or not.

Observation 10 Wait further Ran1 progress to discuss the ICS requirements

# References

- R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR