# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415448

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			A-IoT deployment scenario and spectrum usage

Agenda item:		6.21.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT deployment scenario and spectrum usage.

# Discussion

In WF [1], the deployment scenario is stated as below:

Issue 2-1-1: deployment scenarios for D1T1

Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB

Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware

Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Option 1-1 and 1-2 is currently in coexisting simulation but option 2-1 is 2-2 is deprioritized.

Agreement in RAN4#111:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

RAN4 has below agreement to further study the feasibility fo the option 2-1 and option 2-2.

Agreement in RAN4#112:

- Feasibility of option 2-1/2-2 for device 1 and 2a can be evaluated with LLS. Parameters for LLS are based on company report.

  - Encourage companies to provide the co-existence simulation results.

To study the feasibility of option 2-1/2-2, the LLS is done with the simulation parameters listed in Table below.

Table 1: R2D simulation parameters

| Carrier frequency |  | 900 MHZ |
| --- | --- | --- |
| SCS |  | 15 kHz as baseline |
| Delay spread |  | 30 ns and [150] ns for TDL-A. 30 ns TDL-D. |
| Device velocity |  | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| BS | Number of antenna elements | 2 |
|  | Number of TXRUs | 2 |
| UE | Number of antenna elements | 1 |
|  | Number of TXRUs | 1 |
| Reference data rate |  | [1] kbps (M), [5~7] kbps (M) |
| Message size |  | 20 bits |
| BLER target |  | 1%, 10% |
| Device 1/2a/2b |  | Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED |
| Transmission bandwidth |  | 180 kHz as baseline |
| ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |
| Waveform |  | OOK waveform generated by OFDM modulator |
| Modulation |  | OOK1 |

Figure 1: PSD difference configuration in LLS

The PSD difference between the NR signal and A-IoT signal is simulated with a range of 0 dB to -12 dB. The PSD difference is illustrated in Figure 1. The BLER of detected message is illustrated in Figure 2. It can be observed that if the NR signal PSD is greater than the -11 dB, R2D reception will not reach even the 10% BLER. If in the option 2-1/2-2 co-located NR BS has the same or higher PSD than A-IoT BS, it will not be possible for A-IoT device 1/2a to be able to receive the A-IoT signal in the presence of the similar PSD NR signal.

Option 2-1/2-2 is not possible for A-IoT R2D reception when co-located NR BS transmit equal or higher PSD than A-IoT BS with certain amount.

Figure 2: The BLER for R2D reception with different NR PSD difference

# Conclusions

In this contribution, we present our overview on the A-IoT deployment scenario and spectrum usage with below proposal and observations:

Observation 1 Option 2-1/2-2 is not possible for A-IoT R2D reception when co-located NR BS transmit equal or higher PSD than A-IoT BS with certain amount.

# References

- R4-2414304, WF on co-existence evaluation for ambient IoT, CMCC

- R4-24xyz, On CW cancellation capability, Ericsson