# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415450

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			RF overview for UE as reader

Agenda item:		6.21.3.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements overview for UE as reader.

# Discussion

For UE as reader RF requirement discussion, there are many aspects similar with the BS side. So many aspects should be similar. These aspects are frequency bands discussion and some of transmitter and receiver requirements.

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the UE as reader in D2T2-A2. These factors are listed in Table 1 below. As RAN4 agree that there is no frequency shifting for D2R transmission from device side, the UL spectrum will be used for D2R when CW transmission occurs in UL spectrum. The UE as reader can transmit CW transmission and in this paper, the shared hardware architecture is used for this option. When another node other than UE as reader transmits the CW signal, we say it is separated hardware architecture. Comparing to the A-IoT BS deployment where the separate CW node may be deployed co-located with A-IoT BS, it may not so easy to deploy one UE and another co-located CW node, in this sense, we suggest focusing on the shared architecture only where the UE as reader can also transmit the CW signal.

Use the “shared hardware architecture” or “separated hardware architecture” to represent either CW is transmitted by A-IoT BS or another node.

RAN4 focus on  the RF impact for shared hardware architecture for UE as reader only.

Table 1: Factors impacting the UE as A-IoT reader operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Shared hardware or different hardware for CW transmission | Comment |
| --- | --- | --- | --- | --- | --- |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | Yes | D2T2-A2 |
|  | UL spectrum | DL spectrum | DL spectrum | No | D2T2-B |

For shared architecture, CW signal needs to be transmitted and the CW waveform and generation has been discussed in RAN1 with below agreements:

RAN1#116:

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

RAN1#116bis:

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Because the simultaneous CW and R2D transmission may interfere the R2D signal reception at device 1/2a. Therefore, the R2D and CW transmission is TDM and CW signal transmission requirement should be discussed separately with R2D transmission.

CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

CW and R2D transmission requirement can be discussed separately.

For A-IoT R2D transmitter requirement, the R2D signal design very similar with LP-WUS signal design, for example, the OOK1 and OOK4 design are following the LP-WUS signal design. However, there is no LP-WUS transmission from UE side, therefore, the transmitter requirement should be discussed for transmitter for UE as reader.

Discuss the A-IoT R2D transmitter requirement for R2D transmitter requirements.

RAN1 has below agreement regarding the R2D signal waveform and it can be observed that the OFDM-based waveform is agreed and from this perspective, the legacy UE transmitter requirement can be reused.

RAN1#116:

Agreement A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective. - Depending on what modulation(s) are decided to be studied: - Study whether/how to handle CP at transmitter/device/design - Study other characteristics of the OFDM waveform, e.g.: - CP-OFDM - DFT-s-OFDM - Etc. - The type of OFDM waveform is transparent to A-IoT device. Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study.

Agreement

A-IoT DL study includes OOK from DL transmitter’s perspective.

- For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869.

  - FFS value(s) of M.

  - FFS: Any changes needed from the definitions in TR 38.869.

  - FFS: Exact definition of chip

- If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

However, RAN1 also agrees the below which both OOK1 and OOK4 with M-chip per OFDM symbol transmission.  To have OOK1 waveform, the QPSK modulation can be mapped to CP-OFDM symbol which is used to generate LP-WUS signal. With OOK-4 (DFT-s or waveform fitting approaches) the UE as reader may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. Additionally, the frequency domain symbols used as inputs of IFFT may not be similar to those used for generating existing NR signals (e.g., M-QAM). Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered for this approach to reduce the impact on UE. Therefore, for OOK4 waveform, the higher modulation than QPSK may be mapped to CP-OFDM symbols as illustrated in Figure 1. As Such, most of the legacy transmitter requirement can be reused for UE as reader so no need to define new test case for OOK signal. Doing so will incur some degradation of the SNR of OOK signal, which can be studied further.

RF impact for OOK4 transmission can be kept minimum if the frequency domain mapping for OOK4 can be remapped to legacy modulation (e.g 64 QAM).

Remapping the OOK4 in frequency domain to legacy modulation may incur SNR loss and this aspect can be studied in future meeting.

Keep the transmitter RF impact of the OOK1 and OOK4 signal minimum but study further the impact of OOK4.

RAN1#116:

Agreement A-IoT DL study includes OOK from DL transmitter’s perspective. - For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869. - FFS value(s) of M. - FFS: Any changes needed from the definitions in TR 38.869. - FFS: Exact definition of chip - If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

Figure 1:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

The RF requirement relating to the CW signal should be discussed separately for transmitter requirement as the TDM operation between R2D and D2R.  In this sense, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission are relevant requirement. As this is a new waveform (unmodulated sinusoid), the modulated signal quality is not applied. However, the MPR/A-MPR needed to comply the different regulator requirement needs to be further studied. The high power CW tone transmission may have multiple IM product with the LO leakage and most likely some power backoff would be needed.

For transmitter requirement relating to CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

More study is needed for MPR/A-MPR for CW transmission.

RAN4 can discuss whether MPR/A-MPR simulation can be done in work item phase instead as only two meetings left for study item.

MPR/A-MPR discussion can be postponed to work item phase.

As the CW transmission power may be defined with certain bandwidth, this may not be the same with the A-IoT R2D signal transmission power. Additionally, there may be time constrains on R2D to D2R turn-around time and RAN4 could discuss whether to define a new timing mask for this case for output power dynamic requirement.

Different channel bandwidth may be defined for CW and A-IoT R2D transmission.

A new timing mask may be needed to test outpower dynamic.

For receiver requirement, as the coexisting scenario have both inside and outside topology, it should discuss whether two different set of the RF requirements should be defined corresponding to the either inside topology or outside topology. For inside topology, the receiver needs to tolerate the high leakage CW power from the transmitter for shared hardware architecture. For outside topology, CW transmission is at least the distance of the A-IoT BS for D1T1-B, so the received CW power would be less comparing to the inside topology and it would be less issue for receiver to decode the backscattered signal and receiver linearity is not demanding. In our companion discussion paper for CW cancellation, the additional CW cancellation circuit would be expected if the linearity of the receiver would be compromised. These considerations motivate different receiver architecture depending whether CW inside topology or outside topology is deployed.

Define two sets of the A-IoT UE as reader receiver requirement corresponding to inside topology and outside topology respectively.

The benefit of the defining two sets of receiver requirement is that a larger reader range could be defined. For the REFSENS designed for inside topology, the SNR degradation would be allowed as the high leakage power desensitize the receiver. For the REFSENS designed for outside topology, the need of further relaxing the REFSENS may not be needed therefore larger Reader to device range could be achieved. There are more factors to consider for REFSENS discussion, for example, the energy harvesting sensitivity of device 1 may limit the R2D distance and the min received power may be decided by R2D distance. Such discussion should be in the context of the link budget. Another example, the coexisting may set the limit on the cell range and therefore, the reader to device distance further be limited. For these factors to impact the REFSENS definition, RAN4 can identity these factors in study phase and further address it in work item phase to define the REFSENS.

Two different sets of receiver requirement may target to different reader to device range.

Both link budget and coexisting impact the REFSENS definition.

REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

The REFSENS defined for inside topology should be tested with a CW signal.

REFSENS for inside topology should be tested with the CW signal.

- For the receiver requirements defined for outside topology, it can be further discussed whether there is no need to test it with a CW signal.

Further discuss if there is a need to test the REFSENS with CW signal or not.

- For ACS and inband block requirements, it depends on the coexisting study. It may be necessary to discuss whether the UE as reader only operate for A-IoT RAT but not the other RAT at the same time. This is because when transmitting the CW signal while receiving the backscattered signal, the maximum input power requirement for legacy RAT cannot be met. We suggest no other RAT would be allowed when UE is operating as a reader, otherwise, the will be more impact for legacy requirement especially the strong leakage of CW may desensitize other RAT receiving.

Simultaneous other RAT and A-IoT operation is not allowed for UE as reader.

TP is start here:

### 6.7.3	Intermediate node (UE)

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the UE as reader. These factors are listed in Table 2 below. The UE as reader can transmit CW signal with the same hardware for scenario D2T2-A2.

Table 2: Factors impacting the UE as A-IoT reader operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | Shared hardware with UE or different hardware for CW transmission | Related coexisting scenario |
| --- | --- | --- | --- | --- | --- |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | Yes | D2T2-A2 |
|  | UL spectrum | DL spectrum | DL spectrum | No | D2T2-B |

Based on the RAN1 and RAN agreements, the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant to RF requirement definition and it is necessary to specify the RF characteristic of CW signal based on these agreements. As CW transmission may interfere the R2D reception, the R2D and D2R transmission should be TDM, therefore, the R2D transmission requirement can be discussed separately with CW signal related requirements.

For UE transmitter requirements related to CW signal, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission can be specified. The MPR/A-MPR needs further study for CW signal transmission.

For receiver requirements, two sets of UE receiver requirements as reader can be defined, corresponding to inside topology and outside topology respectively.

# Conclusions

In this contribution, we present our overview on the RF aspects for UE as reader with below proposal and observations:

Proposal-1: Use the “shared hardware architecture” or “separated hardware architecture” to represent either CW is transmitted by A-IoT BS or another node.

Proposal-2: RAN4 focus on  the RF impact for shared hardware architecture for UE as reader only.

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 3 CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Observation 4 CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

Observation 5 CW and R2D transmission requirement can be discussed separately.

Proposal-3: Discuss the A-IoT R2D transmitter requirement for R2D transmitter requirements.

Observation 6 RF impact for OOK4 transmission can be kept minimum if the frequency domain mapping for OOK4 can be remapped to legacy modulation (e.g 64 QAM).

Observation 7 Remapping the OOK4 in frequency domain to legacy modulation may incur SNR loss and this aspect can be studied in future meeting.

Proposal-4: Keep the transmitter RF impact of the OOK1 and OOK4 signal minimum but study further the impact of OOK4.

Proposal-5: For transmitter requirement relating to CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

Proposal-6: More study is needed for MPR/A-MPR for CW transmission.

Proposal-7: MPR/A-MPR discussion can be postponed to work item phase.

Observation 8 Different channel bandwidth may be defined for CW and A-IoT R2D transmission.

Observation 9 A new timing mask may be needed to test outpower dynamic.

Proposal-8: Define two sets of the A-IoT UE as reader receiver requirement corresponding to inside topology and outside topology respectively.

Observation 10 Two different sets of receiver requirement may target to different reader to device range.

Observation 11 Both link budget and coexisting impact the REFSENS definition.

Proposal-9: REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

Proposal-10: REFSENS for inside topology should be tested with the CW signal.

Proposal-11:Further discuss if there is a need to test the REFSENS with CW signal or not.

Proposal-12: Simultaneous other RAT and A-IoT operation is not allowed for UE as reader

# References

- R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR