# 3GPP TSG-RAN WG4 Meeting #112-bis                                                       R4-2415451

# Hefei, China, 14 - 18 October, 2024

Source:		Ericsson

Title:			A-IoT device OTA requirement and testing aspects

Agenda item:		6.21.3.2

Document for:		Discussion

# Introduction

In RAN4#111, there was an agreement to further study Over-The-Air (OTA) test method and performance metrics for A-IoT devices. In this TP we present general aspects for A-IoT devices OTA requirement and test including the measured parameters and the measurement set-up. This paper focus on device 1/2a.

# OTA requirement for A-IoT UE

Apart from the regulatory requirement, e.g spurious requirement, the specified OTA requirements for device 1/2a could be the ones impacting the reader to device range. For the receiver requirement, as device 1/2a does not have the in-band selectivity, the REFSENS may be only relevant to test. For conducted REFSENS, the antenna gain is assumed to be 0 dBi but for A-IoT UE equipped with an antenna, the antenna gain needs to take account for REFSENS and therefore, the antenna model for the device 1/2a may be needed. Currently in RAN1 link budget analysis, the device 1/2a antenna gain is 0 dBi so there is no antenna gain considered in device 1/2a. However, the antenna gain cannot be ignored when designing the OTA requirements. Alternatively, the vendor may declare the post antenna gain as specified for FRMCS (Future Railway Mobile Communication System) in annex M.1 in TS 38.101-1. The post antenna gain in M.1 is defined as “This effective antenna gain includes the feeding loss of all components after the chipset unit antenna connector and the peak directional gain of the external antenna”” with the conducted REFSENS requirement, the post antenna gain can be used to convert from conducted requirement to the OTA requirements with the peak directional gain.

RAN4 OTA requirement needs to take into account the antenna gain of device 1/2a.

- Proposal 1: RAN4 can study to define a typical antenna model for device 1/2a or allow the vendor to declare the post antenna gain to set the OTA requirement for test.

Though energy harvesting is out of the SI scope, it may be useful to discuss the relation to the OTA requirement when the device 1/2a harvesting RF energy from test equipment. As the energy harvesting sensitivity may be higher than the REFSENS calculated with noise figure and SNR threshold, it is a question that whether to allow the vendor to declare the relaxed sensitivity level. In such case, the Energy Harvesting (EH) sensitivity may take into account the antenna gain.

REFSENS can be declared by vendor if the REFSENS relating to energy harvesting only.

As the RAN4 specify the min requirement and according to above discussion, and the REFSENS relates to the reader to device range, it should discuss whether the REFSENS requirements should be specified in two sets, one is without energy harvesting and the other one with RF energy harvesting.

- Proposal 2: Two different sets of the REFSENS could be specified, one with RF energy harvesting and the other is without.

It is not clear how to test the A-IoT device 1/2a with other energy harvesting than RF and how to create the test condition to satisfy the energy harvesting criteria if the device uses other means than RF to harvest energy. From this perspective, we suggest focusing on the RF energy harvesting device and assuming the RF energy is harvested from either R2D transmission or CW signal.

- Proposal 3: RAN4 focus only on RF energy harvesting device 1/2a for test aspect in study phase.

For transmitter requirement, as the backscatter loss is one critical parameter when RAN1 study the link budget. RAN4 could discuss how this parameter can be modeled and tested. The amount of power backscattered can be described using the concept of a cross-section which is used in radar theory. A scattering cross-section can be defined as the ratio of the power radiated by the device to the power density incident on it [2].

Assume the power density of the incident radio wave on the device antenna is S

S=Pt∙Gt4πr2 					(1)

Where the Pt is transmission power of a CW signal and Gt is the transmitter antenna gain, r is the distance between test equipment to the device.

The received power on device antenna is Pa relating to the effective antenna aperture:

Pa=λ2∙Gd4π∙S 					(2)

Where the λ is the radio wavelength and Gd is the device antenna gain.

The power backscattered back in the direction of the transmitter is Pbackscatter:

Pbackscatter=K∙Pa∙Gd=4∙Ra2Za+Zc1,c22  ∙ λ2∙Gd24π∙ S 		(3)

Where the Za=Ra+jXa  is complex antenna impedance and Zc1,c2=Rc1,c2+jXc1,c2 is complex chip load in different modulation status. The antenna impedance and chip load are based on Thevenin equivalent circuit for antenna.

The backscatter modulation efficiency or cross section of the device 1/2a can be defined as

σ=PbackscatterS=Ra2Za+Zc1,c22  ∙ λ2∙Gd2π 			(4)

The received backscattered signal at the test device:

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙σ 				(5)

With the above measurement procedure, the modulation efficiency can be derived from the received modulation signal power and transmission power of the CW signal.

In the test equipment receiver, the received power for the main sideband of the modulated signal can be measured and therefore, the backscattered modulation efficiency or the differential of RCS of backscatter signal can be derived.

The modulation efficiency or backscatter differential cross section can be derived with proper measurement.

- Proposal 3: Specify the backscatter modulation efficiency transmitter requirement for device 1/2a.

| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | - OOK: 6 dB - PSK: 0 dB - FSK: Y dB It is applicable for device 1 and 2a |
| --- | --- | --- | --- |

Currently the test metric is discussed in LP-WUS work item for wake-up receiver, as the A-IoT R2D signal is the same with LP-WUS, the same metric of the testing WUR can be reused.

- Proposal 4: Reuse the test metric for LP-WUS signal detection for wake-up receiver.

- Tp starts here:

#### 6.7.4.1	Device 1

For transmitter requirement, as the backscatter loss is one critical parameter for the link budget. The backscatter loss due to the modulation can be defined as scattering cross-section or modulation efficiency in equation (4). From equation (4), it can be observed that the modulation efficiency is defined as the ratio of the power radiated by the device to the power density incident on its antenna.

Assume the power density of the incident radio wave on the device antenna is S

S=Pt∙Gt4πr2 					(1)

Where the Pt is transmission power of a CW signal and Gt is the transmitter antenna gain, r is the distance between test equipment to the device.

The received power on device antenna is Pa

Pa=λ2∙Gd4π∙S 					(2)

Where the λ is the radio wavelength and Gd is the device antenna gain.

The power backscattered back in the direction of the transmitter is Pbackscatter:

Pbackscatter=K∙Pa∙Gd=4∙Ra2Za+Zc1,c22  ∙ λ2∙Gd24π∙ S		(3)

Where the Za=Ra+jXa  is complex antenna impedance and Zc1,c2=Rc1,c2+jXc1,c2 is complex chip load in different modulation status.

The backscatter modulation efficiency or cross section of the device 1/2a can be defined as

σ=PbackscatterS=Ra2Za+Zc1,c22  ∙ λ2∙Gd2π 			(4)

The received backscattered signal at the test device:

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙σ 				(5)

With the above measurement procedure, the modulation efficiency can be derived from the received modulation signal power and transmission power of the CW signal.

For the receiver requirement, as device 1 does not have the in-band selectivity, the REFSENS may be only relevant to test. For conducted REFSENS, the antenna gain is assumed to be 0 dBi but for A-IoT UE equipped with an antenna, the antenna gain needs to take account for REFSENS.  The post antenna gain in M.1 can be reused for device 1. The post antenna gain is defined as “This effective antenna gain includes the feeding loss of all components after the chipset unit antenna connector and the peak directional gain of the external antenna”. This post antenna gain can be used to convert from conducted requirement to the OTA requirements with the peak directional gain.

#### 6.7.4.2	Device 2a

For transmitter requirement, the modulation efficiency in 6.7.4.1 should be tested as device 2a modulated and backscatter the signal to the BS or UE as reader.

For the receiver requirement, as device 2a does not have the in-band selectivity, the REFSENS may be only relevant to test. For conducted REFSENS, the antenna gain is assumed to be 0 dBi but for A-IoT UE equipped with an antenna, the antenna gain needs to take account for REFSENS.  The post antenna gain in M.1 can be reused for device 2a which is defined as “This effective antenna gain includes the feeding loss of all components after the chipset unit antenna connector and the peak directional gain of the external antenna”. The post antenna gain can be used to convert from conducted requirement to the OTA requirements with the peak directional gain.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 RAN4 OTA requirement needs to take into account the antenna gain of device 1/2a.

Observation 2 REFSENS can be declared by vendor if the REFSENS relating to energy harvesting only.

Observation 3 The modulation efficiency or backscatter differential cross section can be derived with proper measurement.

Proposal 1: RAN4 can study to define a typical antenna model for device 1/2a or allow the vendor to declare the post antenna gain to set the OTA requirement for test.

Proposal 2: Two different sets of the REFSENS could be specified, one with RF energy harvesting and the other is without.

Proposal 3: Specify the backscatter modulation efficiency transmitter requirement for device 1/2a.

Proposal 4: Reuse the test metric for LP-WUS signal detection for wake-up receiver.

# References

[1] 3GPP TS 34.114, “User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing”

[2] Pavel V. Nikitin and K. V. S. Rao, “Theory and measurement of backscattering from RFID tags” IEEE Antennas and Propagation Magazine (Volume: 48, Issue: 6, December 2006)