# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415454

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			BS RF requirement overview for LP-WUS

Agenda item:		6.23.3

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact.

# Discussion

In WF[1], the different aspects of the RF impact of the LP-WUS signal are listed and we share our view on these issues in below discussion.

Issue 1-1: Manufacture declaration on LP-WUS power boosting

- WF

  - FFS on following options:

    - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

    - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification

Referring to the NB-IoT, the NB-IoT operation/feature support is declarable. As the NB-IoT is a separate RAT, declaring it and testing separately make sense. For LP-WUS design, the OFDM transmitter is assumed. If there is no other RF requirement would be impacted but the power boosting level, option 2 is fine as if manufacture does not declare the support the power boosting, the LP-WUS feature can still enabled in deployment.  If there are more RF impact than power boosting, the additional test may be needed to verify the feature support so it may be preferable to set the whole feature as declarable, this is because that there may be BS cannot pass the LP-WUS RF test so it make sense to declare the feature.

Option 2 means LP-WUS can be enabled even without power boosting of LP-WUS.

Option 1 could be used if more RF impact for LP-WUS, if only boosting level is concern, option 2 can be selected.

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.41 | NB-IoT operation | Manufacturer shall declare the support of NB-IoT operation in NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x |  |
| D.42 | NB-IoT sub-carrier spacing | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x |  |
| D.43 | NB-IoT power dynamic range | If the BS supports NB-IoT operation in NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.104 [2] (Note 5). | x |  |

Issue 1-2: Concept of LP-WUS power boosting

- WF

  - FFS on following options:

    - Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs.

      - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

      - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

    - Adopt following definition rather than introduce EPRE ratio.

      - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

In 38.141-1 and 38.141-2, for FR1 test model TM1.2 including a 3dB boosting at the channel edge with up to 40% of the total PDSCH RB allocated in the channel. If the TM1.2 can be referred when manufacture declare the power boosting level, it increase the chance of the LP-WUS feature to be deployed at network, as there is no need for the additional RF test. Therefore, if the CP-OFDM symbol used for LP-WUS generation is QPSK modulation, the 3dB power boosting is possible for legacy NR Base station.

The TM1.2 can test the power boosting of QPSK PDSCH with 3 dB and with 40% RB allocated RB in the channel.

LP-WUS power boosting test can depend on the TM1.2 test model test if the LP-WUS generation using the QPSK and power boosting limit to 3 dB.

Option 3 should be used for power boosting definition and relates to the test model TM1.2 .

##### 4.9.2.2.2	FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Common physical channel parameters are defined in clause 4.9.2.2. Specific physical channel parameters for NR-FR1-TM1.2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB-3P,12NRB-3+3modP-PP-NRB-3+3modP-PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB-3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs which are boosted | NRB-3+3modPP-1 and if NRBG>1, 1, 3, …, 2NRBG-2+1 |
| Level of boosting (dB) | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-3-PNRBG |
| Locations of PDSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) | 10log10NRB-3-103/10PNRBGNRB-3-PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

Issue 1-3: Whether to preclude small CBW for consideration of LP-WUS power boosting

- WF

  - FFS on following options:

    - Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting.

      - Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting.

      - Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded.

      - Option 1-3: other option does not excluded.

    - Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement.

There is minimum BW for LP-WUS which for smaller BW, it means that the channel power will be boosted, though the eMBB coverage is not impacted, it may not be possible for hardware if the BS configure only one carrier and transmit the maximum declared power. If there will be multiple-carrier configured at the same time, boosting one carrier means the power will be stolen from other carrier. So it is doable, but increase the implementation complexity. From this perspective, option 1 is our preference.

Option 1 for issue 1-3 is preferred. Manufacture declare the min channel bandwidth to support the power boosting of LP-WUS.

Issue 1-4: On applicable BS type for LP-WUS

- WF

  - FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration.

  - FFS whether only conductive based BS RF requirements should be defined for LP-WUS:

TM1.2 testing the ACLR and OBUE and these test are TRP test. This test is specified for both for BS type 1-C, 1-H and 1-O. As this test is most relevant to the test applied for different BS types and therefore, LP-WUS can be supported on BS type 1-C, 1-H and 1-O.

BS type 1-C, 1-H and 1-O are all applicable BS types for LP-WUS.

Issue 1-5: On minimum value for LP-WUS power boosting

- WF

  - FFS on the following options:

    - Option 1: If power boosting will be defined as EPRE ratio, use 3dB as minimum requirement.

    - Option 2: If power boosting will be defined as dynamic range, based on existing specification for OOK-1 waveform, the core requirement for the dynamic range should be limited to 3dB.

    - Option 3: Not to introduce minimum value for LP-WUS power boosting.

Issue 1-6: Whether a cap for LP-WUS power boosting should be considered

- WF

  - FFS on the following options:

    - Option 1: It should be considered as 3dB.

    - Option 2: It should be considered as 6dB

    - Option 3: Not to introduce maximum value for LP-WUS power boosting.

As in study phase, the focus of the evaluation of the RF impact of LP-WUS is on the power boosting capability. The boosting of signal may impact the emission performance especially when the WUS signal is located at the channel edge. In 38.104, the boosting is specified in RE power control dynamic range as illustrated below.

Table 6.3.2.2-1: RE power control dynamic range

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

Limit the power boosting for LP-WUS to 3 dB for BS type 1-C, 1-H and 1-O.

Option 2 for issue 1-5 is preferred and option 1 for issue 1-6 is preferred.

Issue 1-8: Transmitted signal quality

- WF

  - Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal.

  - Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design.

- There are two related RAN1 agreements on the OOK1 and OOK4 signal generation:

Agreement

For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1.

- Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

- For the first agreement, it means that the OOK4 will be specified in time domain before DFT/LS. With OOK-4 (DFT-s or waveform fitting approaches) the gNB may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. Additionally, the frequency domain symbols used as inputs of IFFT may not be similar to those used for generating existing NR signals (e.g., M-QAM). Mapping the generated frequency domain values (before IFFT) to existing sequences/QAM modulations could be considered to reduce the impact on gNB. As one example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 1) to 64QAM constellation points (right constellation in  Figure 1) for CP-OFDM symbols as illustrated in Figure 1. As Such, gNB can not boost the LP-WUS signal according to the current base station RE dynamic range requirement.  Figure 2 and Figure 3 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Figure 1:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 2: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 3: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

RAN4 should only allow dynamic range requirements based on existing specification for OOK-1 waveform.

To generate OOK1 waveform, it is not decided whether to specify the time domain or frequency domain according to the RAN1 agreement below. Figure 2 illustrates the frequency domain constellation for either gold sequence or ZC sequence (generated in the frequency domain). Gold sequence frequency domain is similar with NR SSS signal so there is no RF impact if it will be specified in frequency domain. whether ZC or gold sequence to be used is up to RAN1 further discussion.

RAN4 will further RAN1 progress on OOK1 signal design.

Agreement

For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported)

- Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing

- Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing

- Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary

Figure 4: Constellation of ZC sequence or gold sequency in frequency domain before IFFT

For the other requirement than the dynamic range (e.g EVM for OOK), we donot think it is necessary as the new waveform is synthesized with the legacy OFDM transmitter. The OOK waveform is only observable at the receiver side wide with WUR bandwidth filter. From the base station side, it is OFDM waveform it transmitting and transmitting LP-WUS within 5MHz bandwidth does not change this.

No other RF requirement than the dynamic range should be specified for LP-WUS.

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 Option 2 means LP-WUS can be enabled even without power boosting of LP-WUS.

Proposal-1: Option 1 could be used if more RF impact for LP-WUS, if only boosting level is concern, option 2 can be selected.

Observation 2 The TM1.2 can test the power boosting of QPSK PDSCH with 3 dB and with 40% RB allocated RB in the channel.

Observation 3 LP-WUS power boosting test can depend on the TM1.2 test model test if the LP-WUS generation using the QPSK and power boosting limit to 3 dB.

Proposal-2: Option 3 should be used for power boosting definition and relates to the test model TM1.2 .

Proposal-3: Option 1 for issue 1-3 is preferred. Manufacture declare the min channel bandwidth to support the power boosting of LP-WUS.

Proposal-4: BS type 1-C, 1-H and 1-O are all applicable BS types for LP-WUS.

Proposal-5: Limit the power boosting for LP-WUS to 3 dB for BS type 1-C, 1-H and 1-O.

Proposal-6: Option 2 for issue 1-5 is preferred and option 1 for issue 1-6 is preferred.

Proposal-7: RAN4 should only allow dynamic range requirements based on existing specification for OOK-1 waveform.

Observation 4 The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 5 RAN4 will further RAN1 progress on OOK1 signal design.

Proposal-8: No other RF requirement than the dynamic range should be specified for LP-WUS.

# References

- R4-2413512, Way Forward for [112][309] NR_LPWUS, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing