3GPP TSG-RAN4 Meeting #112bis	R4-2415469

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on existing Tx BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#111, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing BS RF requirements for Rel-19 SBFD.

# Discussion

## ACLR and OBUE

In [2], the following was agreed:

- FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

In Section 11 in [4], RAN4 has concluded that outcome of the SBFD and legacy TDD adjacent channel coexistence. To ensure feasible coexistence, RAN4 has concluded that no coexistence issues are identified when coexisting with legacy TDD systems in the adjacent channel. This under the working condition, considered in Rel-19 normative phase, that the SBFD is allowed to be operated in DL and Flexible symbols. Accordingly, adopting legacy ACLR requirement should ensure feasible adjacent channel coexistence.

Observation 1: Adopting legacy gNB ACLR requirement for the SBFD-capable gNB is sufficient to ensure feasible coexistence with adjacent channel deployed legacy TDD systems.

On the other hand, for the coexistence case with gNB-capable SBFD BS in the adjacent channel, RAN4 has identified coexistence challenges due to inter-gNB CLI on the UL subbands. Within the SI phase, RAN4 has considered only ACIR enhancements as a solution to ensure feasible coexistence.

Observation 2: When coexisting with SBFD-capable gNB, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

As a way forward, it is viable to consider urban macro deployment since it is the most challenging deployment scenario. RAN4 then should consider a case where additional new ACLR requirement is considered to allow feasible coexistence with new SBFD-capable gNB while assuming 1 dB desensitization.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

Regarding the CL for inter-gNB CLI, it is a viable approach that RAN4 would consider typical CL values based on typical implementation, leveraging the work carried out in the study item phase, specifically, the feasibility study and the adjacent channel coexistence work. Due to the different views on the implementation capabilities and coexistence results, RAN4 should try to agree on typical values that serve as a baseline to derive the new ACLR/OBUE requirements when coexisting with new SBFD-capable gNBs.

Proposal 2: Regarding CL for inter-gNB CLI, RAN4 to adopt typical CL values based on typical SBFD-capable gNB implementation.

## Joint measurement for normal DL symbols/slots and SBFD symbols/slots

In [2], the following was agreed:

- FFS on the followings: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

In TS 38.104, the procedure to calculate the averaged EVM is given in Annex B.7 as follows:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×NslotNdlTDD and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2

Observed from the above process, it is apparent that the averaging is carried out over the allocated downlink PRBs (full allocation). As a result, for SBFD-capable gNB, the PRBs belonging to the UL subband should not be considered when we are evaluating the average for th EVM of the DL subband for any given SBFD configuration. This by definition, results in difference procedure for EVM calculation for SBFD slot and non-SBFD slot, thus we prefer to agree with the WF captired in [2]. Considering the averaged EVM for SBFD slots may be different from the averaged EVM for non-SBFD slots, in our opinion, they should be measured separately.

Proposal 3: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 4: Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: Adopting legacy gNB ACLR requirement for the SBFD-capable gNB is sufficient to ensure feasible coexistence with adjacent channel deployed legacy TDD systems.

Observation 2: When coexisting with SBFD-capable gNB, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

Proposal 1: RAN4 to consider additional ACLR requirement assuming 1 dB desensitization to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

Proposal 2: Regarding CL for inter-gNB CLI, RAN4 to adopt typical CL values based on typical SBFD-capable gNB implementation.

Proposal 3: Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 4: Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2413515, “Way Forward for [112][308] NR_duplex_evo_General”, RAN4#112, Huawei, HiSilicon.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.