3GPP TSG-RAN4 Meeting #112	R4-2415470

Maastricht, Netherlands, 21st – 25th August, 2024

Agenda item:	6.20.2.3

Source: 	Qualcomm Incorporated

Title: 	Views on existing Rx BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#111, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing Rx BS RF requirements for Rel-19 SBFD.

# Impact on existing BS RF requirements

## Rx Requirements

### Reference sensitivity level and OTA sensitivity

For the reference sensitivity, it was agreed in Rel-18 SI that the existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols for BS type 1-H. For the OTA sensitivity, RAN4 is expected to further discuss the allowed sensitivity degradation at the SBFD BS due to the self-interference. During the SI, value ranges within [0.5-1] dB were considered. The definition of the OTA sensitivity has not been finalized during the SI, it is our view that OTA sensitivity should be defined considering self-interference, inter-site interference, and inter-sector interference.

Proposal 1: RAN4 to agree on 1 dB degradation due to self-interference, inter-site interference, and inter-sector interference when defining the OTA reference sensitivity.

### Rx dynamic range

Since it was recommended during the Rel-18 SI that the Rx dynamic range requirement is applicable to SBFD BS. The IoT level and wanted signal power level are expected to be derived within the adjacent channel coexistence work.

Proposal 2: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

### In-band selectivity and blocking

As agreed, in [2], the in-band blocking requirements are expected to be derived based on RAN4 adjacent channel coexistence as agreed below:

- FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

Based on the WF; the following scenarios, captured in [4], are to be studied through the normative phase, with the corresponding BS class for each scenario, for the case with TDD DL to SBFD UL subband.

| Scenario | FR | Aggressor | Victim | Corresponding BS class |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | Wide area BS |
| 3 | FR1 | Indoor | Indoor | Local area BS |
| 5 | FR1 | Micro | Micro | Medium range BS |
| 6 | FR2-1 | Urban Macro | Urban Macro | Wide area BS |
| 9 | FR2-1 | Indoor | Indoor | Local area BS |

We consider the same simulation framework that is in [4] Annex E with the following highlights:

- UMa with 50% GS

- Option 1 in the LoS probability

- Baseline AAS configuration considering two panels with 46 dBm conducted power per polarization

The metric considered on the x-axis is the absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation. In Figure 1, a preliminary visualization of the CDF of the received power level in dBm is presented along with the 99% percentile point. In order to derive the in-band requirement for that particular case, it is recommended to consider the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

Proposal 3: To derive the in-band requirement for that particular case, it is recommended to consider 50% GS for the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

Figure 1 Received power level CDF for UMa when TDD DL is aggressor and SBFD UL is victim

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Proposal 1: RAN4 to agree on 1 dB degradation due to self-interference, inter-site interference, and inter-sector interference when defining the OTA reference sensitivity.

Proposal 2: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

Proposal 3: To derive the in-band requirement for that particular case, it is recommended to consider 50% GS for the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.