3GPP TSG-RAN4 Meeting #112bis	R4-2415471

Hefei, Anhui, China, 14th – 18th October 2024

Title:	Views on EIRP mask considerations for upper 6GHz

Source:	Qualcomm Incorporated

Agenda item:	6.11.4

Document for:	Approval

# 1	Introduction

In RAN#103 a new work item on BR BS RF requirement evolution for FR1/FR2 and testing has been approved in [1]. One of the objectives of the core part related to the expected EIRP mask for upper 6GHz is the following:

- Introduce the concept of “limits on expected EIRP for angles above horizon” into the RAN4 BS specifications and specify related BS requirement for 6425-7125 MHz based on the WRC-23 outcome.

In RAN4#112, a WF was agreed in [2]. In this paper we share our view on the expected work within RAN4 to define the EIRP mask to control emissions above the horizon for an NR BS.

# 2	Discussion

## 2.1	     Impact of measurement overhead

Based on the expected EIRP framework, current discussions on the conformance testing need to consider several parameters when designing the conformance tests. As an example, the number of beamforming directions over which the test is done, the weights associated with each beamforming direction in the expected EIRP evaluation, and the sampling grid sizes in both elevation and azimuth. To understand the overall testing/ measurement overhead, RAN4 need to investigate the inter-dependency between the above aspects and overall testing complexity.

Observation 1: A balance between testing/measurement overhead and accuracy needs to be considered when determining the number of beamforming directions, weights associated with each beamforming direction and sampling grid sizes.

## 2.2	Number of beamforming directions

Given a coverage angular region (CAR) in the azimuth and elevation planes for the gNB AAS, the most natural array geometry is expected to be rectangular. As part of the conformance test, test beamforming vectors are expected to steer the energy only within the CAR. To derive  meaningful steering angle, in the conformance specs the following should be considered. A large number of beamforming vectors can be used, with the natural measurement unit is the 3-dB beamwidth of a beam. The number of beamforming directions can be evaluated based on the concept of “over sampling of beams”, similar to channel state feedback codebooks in 38.214. An oversampling factor Oazm and Oelev can be considered for both azimuth and elevation planes, leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams with the total the number of beamforming vectors is K=Kazim Kelev. A visualization of a rectangular CAR with a set number of beams is given in Figure 1 below.

Figure 1 A rectangular CAR with a set of beams

Proposal 3: RAN4 to define a set of oversampling factors Oazm and Oelev associated with the coverage angular region leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams (in each dimension) for testing, where θ3dB and ϕ3dB are th3 3-dB beamwidth of the BS main beam. The beam steering directions are θn,φm where m=1,…, Kazim and n=1,…, Kelev. Thus, the number of beamforming vectors is K=Kazim Kelev.

For the weighting factors associated with each of the beamforming directions, it is viable to consider uniform weighting across all the beams in order to avoid increased testing/ measurement complexity.

Proposal 4: RAN4 to consider in the conformance specs that the weighting factors of the supported beamforming directions follow a uniform distribution.

## 2.3	Measurement grid size

Similar to the number of beamforming directions, we can utilize the same rationale to identify the sufficient grid sizes in the azimuth and elevation planes. Since we need to sample the entire 360o in azimuth, we can use the same step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB. For the elevation domain (region of interest being θL ≤θ≤θH), we can consider an oversampling factor of Lelev for a measurement grid of N beams where N=LelevθH- θL θ3dB

Proposal 5: RAN4 to consider step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB for the azimuth plane and a measurement grid of N beams where N=LelevθH- θL θ3dB for the elevation plane.

## 2.4	Measurement accuracy reporting

Recall that the expected EIRP framework is expressed as below

EEIRPθL , θH=1K(θH-θL)NMsinθH-sinθLk=1.. K n=nL nH m=1MEIRPθn,φm;wk cosθn,

the θnand ϕm implicitly considers the sampling grid size, so a framework to determine the estimation process in the above framework/ methodology is sought. Leveraging the introduced oversampling factors for both azimuth and elevation planes, one can define the estimation error in evaluating the expected  EIRP as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

Proposal 6: RAN5 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

## 2.5 	RF channels for conformance testing

In [2], the following was discussed regarding which RF channels to be considered for the conformance testing:

Issue 4-4: RF channels for the conformance testing - Proposal 1: B, T [Nokia] - Proposal 2: M. [Huawei] - Proposal 3: T [Ericsson] Agreement: Companies are encouraged to provide the inputs for RF channels for conformance testing

In our views, proposal 1 is a viable option for RAN4 to provide sufficient confidence on the testing procedures by considering the two extremes of the RF channels to be tested. The top and bottom RF channels will aid in setting the strictest design constraints and ensuring that the conformance test adequately stretches the test to capture the requirement conformance.

Proposal 6: RAN4 to consider both Bottom and Top RF channels for the expected EIRP conformance testing.

# 3	Summary

In this paper, we have shared our initial views on the expected EIRP mask development for upper 6GHz NR BS where we propose the following:

Observation 1: A balance between testing/measurement overhead and accuracy needs to be considered when determining the number of beamforming directions, weights associated with each beamforming direction and sampling grid sizes.

Proposal 2: RAN4 to define a set of oversampling factors Oazm and Oelev associated with the coverage angular region leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams (in each dimension) for testing, where θ3dB and ϕ3dB are th3 3-dB beamwidth of the BS main beam. The beam steering directions are θn,φm where m=1,…, Kazim and n=1,…, Kelev. Thus, the number of beamforming vectors is K=Kazim Kelev.

Proposal 3: RAN4 to consider in the conformance specs that the weighting factors of the supported beamforming directions follow a uniform distribution.

Proposal 4: RAN4 to consider step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB for the azimuth plane and a measurement grid of N beams where N=LelevθH- θL θ3dB for the elevation plane.

Proposal 5: RAN4 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

Proposal 6: RAN4 to consider both Bottom and Top RF channels for the expected EIRP conformance testing.

# 4	References

- RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, RNA4 chair Huawei, RAN#103.

- R4-2413509, “Way Forward for [112][305] NR_BS_RF, ZTE, RAN4#112, Maastricht.