# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415472

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	6.16.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed where several topics were discussed and agreed regarding general aspects and interference profiling that would be considered during the normative phase. In this contribution, we provide our views on the different assumptions and potential deltas that RAN4 is expected to consider in Release 19 compared to the work that was carried out in LTE for BS with advanced receiver.

# 2 	Discussion

## 2.1	Interference modelling

Regarding the interference model, it was agreed regarding the test scenarios in [2] the following

- Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104 - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

To prioritize the work within RAN4, following the WID objectives, our preference would be proposal 1, where focus should be set primarily on TDD synchronous deployments. Considering different TDD patterns between the BSs might be investigated later, but we believe that synchronous deployments along with possibility to incorporate high power UE (HPUE) impact on top of interference profiles realized from TR 36.884 [3] would be relevant to the Rel-19 WID objectives.

Proposal 1: RAN4 to consider Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104, assuming synchronous deployments as a starting point for the Rel-19 WI phase.

On the interference modelling, the following was captured in [2]

- on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling

The DIP model, explained in Section B.6.1 in TS 36.104, is characterized by its associated dominant interferer proportion (DIP) value, which is defined as the ratio of the power of ith strongest interference to the power of all interfering UEs plus noise. It is typically assumed that the thermal noise power for a given channel bandwidth is the same for all UE and denoted by σ2. Additionally, assuming that there are total inter-cell interfering NUE UEs and the received signal power of the UE of interest is So , while I1, I2, …, INUE-1 are the interfering UEs. The DIP for the i-th UE can be expressed as

DIPi=IiN', where N'=j=1NUE(Ij+σ2), i=1,2,…,NUE

Consequently, the UL SINR is SINR= S0N'. On the other hand, INR model was also adopted in several demodulation requirements definition for UE. Mathematically, the INR model is defined as the following:

INRi=j=1NRxIor(i)(j)j=1NRxNoc(j)

Where, Ior(i)(j) is the average received power spectral density from the i-th strongest interfering UE involved in the requirement scenario on the j-th antenna connector and Noc(j) is the average power spectral density of a white noise source on the j-th antenna connector. As shown in R4-2412143, the DIP and INR model are related and thus, a combined formulation between both can be achieved. It is thus our preference for RAN4 to adopt the INR methodology when defining the demodulation requirements for the BS with MMSE-IRC receiver for two reasons. Firstly, INR has been adopted for defining UE demodulation requirements, thus from a consistency point of view, it is desirable to keep the UE and BS requirements consistent. Secondly, for link level simulations, INR is easier to consider when compared to DIP.

Proposal 2: Although INR and DIP interference modelling can be expressed by one another, it is proposed for RAN4 to adopt the INR interference model when defining the demodulation requirements for Rel-19 BS with MMSE-IRC receiver.

As captured in [1], the motivation behind Rel-19 WI on BS demodulation requirement with enhanced receiver is to consider more realistic scenarios for dense deployments where uplink interference from co-scheduled UEs in the neighbouring cells can be a limiting factor to the uplink performance. High power UEs are being extensively studied and standardized within 3GPP (e.g., high-power UE (PC 1.5) in n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation for FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795)), with the system and link level benefits of high power UEs, the inter- and intra-cell interference dynamics can be different, especially for urban dense deployments. Accordingly, suppression of such interference based on advanced BS receiver can provide an uplink performance boost in terms of coverage.

To account for HPUEs, RAN4 could consider artificial values for DIP/INR, where different interference levels could be simulated to ensure the impact of HPUEs is accounted for in the link level studies. For example, the impact of high UE power classes (PC2 or PC1) could be investigated by adjusting the LTE INR/DIP values with the additional 3 or 7 dB to check the performance impact. In our views, addressing HPUEs should be further investigated within RAN4.

Proposal 3: RAN4 to discuss adjusting INR/DIP values, e.g., scaling INR/DIP to account for possible delta in interference resulting from higher UE maximum output power compared to 23 dBm, from LTE studies (TR 36.884) to account for high UE power classes (e.g., PC2 or PC1) when deriving the BS demodulation requirement.

## 2.2	Test parameters and simulation assumptions

Test scope

From the tentative discussion captured in [2], in our view, it is viable for RAN4 to adopt the below SCS and TDD patterns as a starting point for the normative work.

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Proposal 4: RAN4 to adopt the below SCS and TDD patterns as a starting point for the normative work.

    - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

    - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Channel bandwidth

In TS 36.104, BS with advanced receiver requirements were provided for a number of channel bandwidths (1.4MHz till 20MHz). Thus, for Rel-19, in order to ensure that sufficient cases are covered, it is proposed to consider minimum and maximum channel bandwidth for each SCS.

Proposal 5: RAN4 to consider Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz.

Antenna Configuration

During LTE study, the performance gain between baseline (i.e., MMSE) and advanced (i.e., MMSE IRC) was marginal, thus, it was agreed to consider only one explicit interferer for 2Rx cases. However, two explicit interferers are for 4Rx and 8Rx cases. For the Rel-19 WI, a further distinction can be made between FDD and TDD scenarios:

- For FDD scenarios, it is recommended that first RAN4 identify the gains of MMSE IRC receiver considering two interferences as a starting point for 4Rx and 8Rx cases. The rationale is that 2Rx scenarios for NR BSs might not be foreseeable.

- Similarly, for TDD, given that UEs are required to have 4RX capability is several deployment bands, it seems unrealistic use case to consider 2RX BSs, focus should be on 4RX and 8 RX cases

Proposal 6: RAN4 to consider only 4RX and 8RX BS for TDD and FDD scenarios as 2Rx for NR BSs might not be foreseeable.

In addition to considering 8Rx as maximum number of receivers for TDD and FDD BS, it is viable also to consider higher than 8Rx to address massive MIMO deployments, which are considered for 5G deployments. As BS demodulation requirements are typically capped at 8Rx, it would be desirable to understand the feasibility from RAN4 point of view to study more than 8Rx demod requirements.

Proposal 7: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Propagation condition

In terms of multi-path fading propagation conditions, LTE study [2] considered ETU channel with high delay spread for the interferers and EPA5 and EVA70 were chosen for the serving channel while, ETU5 and ETU70 were considered for the interfering channel. Since different channel model for serving and interfering UEs was assumed and based on initial discussions within RAN4 in [2], we consider the same assumption in this work item (i.e., different channel model for serving and interfering UEs). Additionally, in [3], different channel models for homogenous and heterogeneous deployments were assumed, such that low Doppler models were adopted for heterogenous (EPA5 and ETU5) and high Doppler models for homogenous (EVA70 and ETU70) due to anticipated different cell ranges and typical UE speed.

Observation 1: In LTE work, different channel model was assumed for serving and interfering links as well as between homogenous and heterogeneous deployments.

For Rel-19, it is expected that RAN4 would choose an appropriate propagation conditions that is realistic compared to the envisioned use cases and scenarios. Accordingly, it is proposed to select among legacy TDLA30, TDLB100, and TDLC300 conditions. Specifically, we propose to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

Proposal 8: RAN4 to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

# Conclusion

In this contribution we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to consider Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104, assuming synchronous deployments as a starting point for the Rel-19 WI phase.

Proposal 2: Although INR and DIP interference modelling can be expressed by one another, it is proposed for RAN4 to adopt the INR interference model when defining the demodulation requirements for Rel-19 BS with MMSE-IRC receiver.

Proposal 3: RAN4 to adjust INR/DIP values (e.g., scaling INR/DIP by additional 3 or 7 dB) from LTE studies (TR 36.884) to account for high UE power classes (e.g., PC2 or PC1) when deriving the BS demodulation requirement.

Proposal 4: RAN4 to adopt the below SCS and TDD patterns as a starting point for the normative work.

    - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

    - 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Proposal 5: RAN4 to consider Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz.

Proposal 6: RAN4 to consider only 4RX and 8RX BS for TDD and FDD scenarios as 2Rx for NR BSs might not be foreseeable.

Proposal 7: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Observation 1: In LTE work, different channel model was assumed for serving and interfering links as well as between homogenous and heterogeneous deployments.

Proposal 8: RAN4 to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2413570, “Way Forward for [112][328] NR_demod_Ph5”

[3]	TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS” 3GPP V13.1.0