3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415485

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.8.3.3

Source: 	Nokia

Title: 	SAN RF core requirements for less than 5MHz for NTN

Document for:	Approval

# 1.	Introduction

The revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved at TSG RAN#105 [1]. The objectives of this WI include:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

This topic was discussed at TSG RAN4#112, and the Way Forward was agreed [2].

This contribution provides proposals to progress the issue that was FFS in the agreed WF.

# 2.	Discussion

One issue was FFS in the agreed WF as follows:

# FFS on the impact for radiated requirements for SAN type 1-H and 1-O: NR SAN RF radiated Tx/Rx requirement; Proposal for 3MHz channel bandwidth in FR1-NTN bands. 9.2 Radiated transmit power; No specification impact.. 9.3 OTA SAN Output power dynamics; No specification impact.. 9.4 OTA output power dynamic range; 1-O follow same requirement as 1-H, no update on specification.. 9.5 OTA transmit ON/OFF Power; No specification impact.. 9.6 OTA transmitted signal quality; No specification impact.. 9.7 OTA unwanted emissions; 1-O follow same requirement as 1-H, no update on specification.. 9.8 OTA Transmitter intermodulation; No specification impact.. 10.2 OTA sensitivity; No specification impact.. 10.3 OTA reference sensitivity level; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.4 OTA dynamic range; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.5 OTA In-band selectivity and blocking; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.6 OTA Out-of-band blocking; No specification impact.. 10.7 OTA receiver spurious emissions; No specification impact.. 10.8 OTA receiver intermodulation; No specification impact.. 10.9 OTA In-channel selectivity; Similar update as conductive requirements to add 3MHz channel bandwidth..

Currently, the SAN RF core requirements are only defined in TS 38.108 [3] for SAN type 1-H and SAN type 1-O for the NR-NTN bands n255, n256, and n254 considered in this WI. The conducted Tx/Rx requirement for SAN type 1-H were already agreed in the WF [2]. The FFS is on the radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O.

Currently, the less than 5MHz BS RF requirements are only defined in TS 38.104 [4] for BS type 1-C because the NR bands considered were all below 1 GHz where AAS type of implementation is not common. However, the NR-NTN bands n255, n256, and n254 are all above 1.5 GHz where AAS type of implementation is more common. Therefore, the radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should also be defined for less than 5MHz SAN.

Observation: AAS type implementation is more common for currently defined NTN bands.

Proposal: The radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should also be defined for less than 5MHz.

# 3.	Conclusion

This contribution has provided proposals to progress the issue that was FFS in the agreed WF in last RAN4 meeting.

Observation: AAS type implementation is more common for currently defined NTN bands.

Proposal: The radiated Tx/Rx requirement for SAN type 1-H and SAN type 1-O should also be defined for less than 5MHz.

# References

[1]	RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung.

[2]	R4-2413518, “Way Forward for [112][313] NR_IoT_NTN_less_than_5MHz_BSRF”, Nokia.

[3]	3GPP TS 38.108 v18.4.0: “NR; Satellite Access Node radio transmission and reception”.

[4]	3GPP TS 38.104 v18.7.0: “NR; Base Station (BS) radio transmission and reception”.