3GPP TSG-RAN WG4 Meeting #112bis		R4-2415487

Hefei, China, 14 – 18, 2024

Agenda Item:	6.15.3

Source: 	Apple

Title:	Discussion on fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

# Introduction

Fast SCell activation for UE supporting Rel-18 EMR has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

Issue 1-1-2: Applicability of fast SCell activation delay requirements

Agreement:

- The fast SCell activation delay requirements are defined for the case when

  - the UE supports Rel-18 eEMR:

    - Including both EMR and cell reselection measurement,

    - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured

    - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

  - and the UE has reported valid results on the SCell to be activated before SCell activation command.

The FFS part is whether to the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured. We don’t have strong view but slightly prefer not to include this case. Because 1) network is expected to configure the timer so that UE can know clearly NW would like to enable this feature. 2) if a measurement result was obtained long time ago, it may not reliably provide accurate time and frequency info. As also proposed by some other companies in RAN4#112 that “RAN4 to define a condition/limit that measurement performed X seconds before SCell activation is considered as valid for known SCell activation conditions.”

Proposal 1: it is not preferred to include the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreement:

- Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR:

  - Direct SCell Activation at SCell addition

  - FFS: Single MAC CE based DL SCell Activation

  - FFS: Single MAC CE based PUCCH SCell Activation

- RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

On top of direct SCell activation, we support to cover single MAC CE based DL SCell activation as well. Direct SCell activation is an optional feature. If we only focus this case, that would make direct SCell activation as a pre-requisite for R19 fast SCell activation for UE supporting R18 EMR.

Proposal 2: to increase applicability of fast SCell activation for UE supporting R18 EMR, single MAC CE based DL SCell Activation shall be also in the scope.

Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting

Agreement:

- RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command.

  - FFS how to update the known condition (issue 1-2-1a).

  - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting

FFS on proposals listed in [1] for issue 1-2-1a.

To align with proposal 2, we propose to update known conditions in requirements for both direct SCell activation and legacy MAC-CE based SCell activation.

Proposal 3: update known definition for both direct SCell activation and legacy MAC-CE based SCell activation as follows:

- Direct SCell activation

The following update is proposed (section 8.3.4, red part is new):

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last 5 seconds before the reception of the direct SCell configuration command:

-	the UE has sent a valid measurement report for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise, the SCell is unknown.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, the SCell in FR1 is also known provided the following conditions are met for the SCell:

-	During the last [5s] before the reception of the direct SCell configuration command:

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

- Legacy MAC-CE based SCell activation

The following update is proposed (section 8.3.2, red part is new):

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, SCell in FR1 is also known if it has been meeting the following conditions:

-	During the period equal to [5s] for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being activated and

-	the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

-	the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

…

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, for the first SCell activation in FR2 bands, the SCell is also known if it has been meeting the following conditions:

-	During the period equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3, and

-	SCell activation command is received after valid EMR reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from valid EMR reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

Proposals

- Option 1:

  - RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

We would like to propose for fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases.

Proposal 4: for fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases.

Issue 1-2-4: Whether the indication to network is needed

Proposals

- Option 1:

  - The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

We don’t quite understand why additional indication is necessary on top of valid EMR report in R18. Upon receiving valid report from UE, it is up to NW whether to configure and/or activate corresponding carrier(s) for CA/DC.

Proposal 5: valid eEMR report in R18 is enough. More justification is needed for additional network indication.

# Conclusion

In this contribution, we provide discussion on fast SCell activation for UE supporting Rel-18 EMR. After discussion, the following conclusions are provided:

Proposal 1: it is not preferred to include the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured.

Proposal 2: to increase applicability of fast SCell activation for UE supporting R18 EMR, single MAC CE based DL SCell Activation shall be also in the scope.

Proposal 3: update known definition for both direct SCell activation and legacy MAC-CE based SCell activation as follows:

- Direct SCell activation

The following update is proposed (section 8.3.4, red part is new):

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last 5 seconds before the reception of the direct SCell configuration command:

-	the UE has sent a valid measurement report for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise, the SCell is unknown.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, the SCell in FR1 is also known provided the following conditions are met for the SCell:

-	During the last [5s] before the reception of the direct SCell configuration command:

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

- Legacy MAC-CE based SCell activation

The following update is proposed (section 8.3.2, red part is new):

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, SCell in FR1 is also known if it has been meeting the following conditions:

-	During the period equal to [5s] for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3 for the SCell being activated and

-	the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

-	the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

…

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

For a UE supporting measValidationReportEMR and measIdleValidityDuration-r18 is configured with smaller than [X] or a UE supporting measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured with smaller than [X], besides the conditions above, for the first SCell activation in FR2 bands, the SCell is also known if it has been meeting the following conditions:

-	During the period equal to [4s] for UE supporting power class 1/5 and [3s] for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid EMR report according to the reporting requirements in 4.7.3 or 5.8.3, and

-	SCell activation command is received after valid EMR reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from valid EMR reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

Proposal 4: for fast SCell activation based on eEMR, time for AGC is always needed since measurement period in IDLE/INACTIVE mode is longer than 2400ms in most cases.

Proposal 5: valid eEMR report in R18 is enough. More justification is needed for additional network indication.

# Reference

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT