3GPP TSG-RAN WG4 Meeting #112bis		R4-2415488

Hefei, China, 14 – 18, 2024

Agenda Item:	6.22.2.1

Source: 	Apple

Title:	Discussion on on-demand SSB SCell operation

Document for:	Approval

# Introduction

RAN4 requirements for OD-SSB was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1]. Besides, RAN4 also received an LS [2] from RAN1, in which RAN4 action is included.

In this contribution, we will discuss the RAN1 LS and all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

The first issue we would like to discuss is about deactivated SCell L3 measurement for case 1:

Deactivated SCell L3 measurement part

Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement

- Proposals

  - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle).

  - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell.

  - Other option is not precluded

Option 1 is not supported since it would increase UE power significantly. Note that OD-SSB is still a cell level reference signal. It is possible that network activates OD-SSB for some other UE which is going to use CA soon. Network can by the way inform this UE to perform L3 measurement since OD-SSB is already being transmitted. This is a valid scenario for case 1. Because if network really want to use CA for this UE immediately, network shall directly activate both OD-SSB and target SCell (it becomes scenario 2A).

Therefore, option 2 is still preferred. If it is not agreeable. One compromised solution is to apply fast measurement (i.e. following OD-SSB periodicity) within a time window after OD-SSB is activated. If target SCell is not activated during the time window, UE fallbacks to legacy measurement (i.e. follow measCycleSCell).

Proposal 1: Case 1- OD-SSB based deactivated SCell measurement:

- Option 1: follow the measCycleSCell

- Option 2: apply fast measurement (i.e. following OD-SSB periodicity) within a time window after OD-SSB is activated. If target SCell is not activated during the time window, UE fallbacks to legacy measurement (i.e. follow measCycleSCell).

The second issue we would like to discuss is about relationship between always-on SSB and OD-SSB in case 2. Based on existing specification, UE is allowed to cause only one interruption during SCell activation procedure (due to RF activation). Currently, scenario wherein always-on SSB and OD-SSB are on different frequencies is not precluded. We would like to point out that if this scenario is included, UE may cause additional interruption during SCell activation. Some examples:

Proposal 2: do not consider different frequencies between always-on SSB and OD-SSB in based SCell activation requirements for case 2.

The most typical use case in our mind is that network uses long periodicity for always-on SSB to save power. Whenever there is need for CA, network activates OD-SSB with short periodicity to facilitate SCell activation. Therefore, it is proposed to prioritize same time offset, different periodicities between always-on SSB and OD-SSB.

Proposal 3: prioritize same time offset, different periodicities between always-on SSB and OD-SSB for case 2.

Another thing we would like to discuss is about neighbor cell measurement. OD-SSB is operated per cell. When OD-SSB is not transmitted on serving SCell, neighbor cell on the same carrier may still transmit SSB (always-on SSB and/or OD-SSB). From UE power saving point of view, if UE still needs to measure neighbor cell on the same carrier, then there is almost no power saving gain due to OD-SSB deactivation from serving SCell. However, such measurement on neighbor cell could still be useful, from mobility perspective. So far it is unclear whether UE needs to perform such measurement or not. Since network has better understanding on necessity of such measurement, e.g. whether neighbor cell is transmitting OD-SSB or not, it is better to let it to network to decide whether UE shall perform RRM measurement on neighbor cells on deactivated SCC. It could be implicitly controlled by OD-SSB activation/deactivation, e.g. UE stops RRM measurement on all cells on the carrier once OD-SSB from serving SCell is deactivated (including neighbor cell measurement). Alternatively, network explicitly indicates whether neighbor cells measurement is needed or not, regardless of status of OD-SSB. We are open for further discussion.

Proposal 4: RAN4 shall discuss whether and how to perform RRM measurement on neighbor cells on the same carrier of SCell with OD-SSB operation.

Proposal 5: potential solution:

- Option 1: neighbor cell measurement is implicitly controlled by OD-SSB activation and deactivation. When OD-SSB is activated on the SCell, UE starts RRM measurement on all cells on the same carrier. When OD-SSB is deactivated on the SCell, UE stops RRM measurement on all cells on the same carrier.

- Option 2: neighbor cell measurement is explicitly controlled by network, regardless of status of OD-SSB of serving SCell. Details are up to RAN2.

Next issue we would like to discuss is how to facilitate SCell activation in case 1. Note that typically in case 1 target cell is unknown to UE since no always-on SSB before OD-SSB activation. The whole SCell activation would be quite long for unknown target cell since UE needs to start from scratch. Note that in R18 SSB-less scenario, there is a reference cell from which UE can borrow some information like AGC and timing so that the activation latency could be reduced:

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent a "default cell" is the reference cell as defined in TS 38.133 [14].

However, referenceCell configuration only applies for inter-band SSB-less SCell. We think it is beneficial to extend the concept to cover SCell with OD-SSB operation.

Proposal 6: extend referenceCell to cover SCell with OD-SSB operation.

One last thing is about RAN1 LS [2]:

RAN1 has made the following agreements for MAC-CE based indication of on-demand SSB (OD-SSB) transmission on SCell. Only the relevant agreements related to RAN1’s question are excerpted. In highlights in the agreements, RAN1 agreed working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement. <RAN1 #116> Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for <RAN1 #116 bis> Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. <RAN1 #117> Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable <RAN1 #118> Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min= where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and is as defined in current specification. - RAN4 to confirm that T_min can be equal to - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology..

In summary, RAN1 is asking whether OD-SSB activation can be completed by n+m+3Nslotsubframe+1:

In our understanding MAC-CE processing time shall be same as legacy, e.g. it can be finished before n+m+3Nslotsubframe+1. However, from UE measurement point of view, some additional processing time is expected since UE may need to update measurement scheme upon OD-SSB activation. As we all know, two separate searchers are assumed in RRM measurement requirements. Measurement on OD-SSB occupies one searcher. Typically once OD-SSB is activated, UE may assume network is going to activate target SCell soon (e.g. scenario 2A). UE may choose to prioritize OD-SSB measurement.

Even if UE doesn’t always prioritize OD-SSB, effectively there is one more MO to measure upon OD-SSB activation:

Proposal 7: additional leading time T△ is needed on top of n+m+3Nslotsubframe+1 for UE to update measurement sequence.

Proposal 8: additional leading time T△ is [5ms].

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for OD-SSB. After discussion, the following conclusions are provided:

Proposal 1: Case 1- OD-SSB based deactivated SCell measurement:

- Option 1: follow the measCycleSCell

- Option 2: apply fast measurement (i.e. following OD-SSB periodicity) within a time window after OD-SSB is activated. If target SCell is not activated during the time window, UE fallbacks to legacy measurement (i.e. follow measCycleSCell).

Proposal 2: do not consider different frequencies between always-on SSB and OD-SSB in based SCell activation requirements for case 2.

Proposal 3: prioritize same time offset, different periodicities between always-on SSB and OD-SSB for case 2.

Proposal 4: RAN4 shall discuss whether and how to perform RRM measurement on neighbor cells on the same carrier of SCell with OD-SSB operation.

Proposal 5: potential solution:

- Option 1: neighbor cell measurement is implicitly controlled by OD-SSB activation and deactivation. When OD-SSB is activated on the SCell, UE starts RRM measurement on all cells on the same carrier. When OD-SSB is deactivated on the SCell, UE stops RRM measurement on all cells on the same carrier.

- Option 2: neighbor cell measurement is explicitly controlled by network, regardless of status of OD-SSB of serving SCell. Details are up to RAN2.

Proposal 6: extend referenceCell to cover SCell with OD-SSB operation.

Proposal 7: additional leading time T△ is needed on top of n+m+3Nslotsubframe+1 for UE to update measurement sequence.

Proposal 8: additional leading time T△ is [5ms].

# Reference

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple