3GPP TSG-RAN WG4 Meeting #112bis		R4-2415490

Hefei, China, 14 – 18, 2024

Agenda Item:	6.22.2.1

Source: 	Apple

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Approval

# Introduction

RAN4 requirements for adaptation of common signal/channel transmission was widely discussed in the previous RAN4 meeting. The latest agreements can be found in [1]. There are quite many open issues as captured in [1].

In this contribution, we will continue discussion on all open issues listed in RAN4 WF [1]. After discussion, corresponding proposals are provided.

# Discussion

## SSB Adaptation requirements

Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode

- Agreement

  - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode.

  - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress

According to RAN1 progress in RAN1#118, SSB adaptation is not yet confirmed:

Study further the following for Rel-19 NES-capable UE in idle/inactive mode

- [Option 1: Adaptation of CD-SSB]

- Option 2: Adaptation of CD-SSB at least for a cell not providing initial cell selection

  - FFS: whether R19 NES-capable UE can use the cell for initial cell selection

- Option 3: Adaptation of SSB that is not CD-SSB at least for a cell that has large periodicity CD-SSB (e.g. periodicity greater than 20 msec)

  - No adaptation on CD-SSB

- Option 4: No additional SSB adaptation mechanism is specified

Therefore, RAN4 doesn’t need to discuss SSB adaptation in IDLE mode at current stage.

Proposal 1: RAN4 doesn’t need to discuss SSB adaptation in IDLE mode at current stage.

Issue 3-1-2: SSB adaptation impact in RRM requirement

- Agreement: SSB adaptation impact in RRM requirement

  - RAN4 to discuss the L1/L3 measurement requirement impact as starting point.

- Wayforward:

  - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point.

    - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

It is straightforward to us that if SSB periodicity is updated, corresponding measurement period will be updated, including both L1 and L3 measurement. One thing we would like to point out is that minimum requirements upon SSB periodicity adaptation is needed. Take L3 measurement for example, when SSB periodicity adaptation occurs UE doesn’t have enough samples to complete L3 measurement for target cell, UE needs to follow new SSB periodicity to perform L3 measurement. Note that in practice UE may be configured with CA and multiple carriers to measure, and even with measurement gap. It would be quite challenging to determine how many samples are needed to complete the measurement (different UE may have different measurement order, thus for the specific target cell different UE may have stored different number of samples upon SSB periodicity adaptation)

Proposal 2: minimum requirement upon SSB periodicity adaptation is needed:

- When SSB periodicity changes upon SSB periodicity adaptation, the cell identification, measurement period and L1 requirements apply based on the longer delay before or after the adaptation.

## Impact on Paging Adaptation requirements

Issue 3-2-1: Paging adaptation

Agreement:

- Deprioritize Paging adaptation until the RAN1 concrete conclusion is available.

We haven’t identified any RAN4 impact yet.

## Impact on PRACH Adaptation requirements

Issue 3-3-1: PRACH adaptation

Agreement:

- Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.

We haven’t identified any RAN4 impact yet.

# Conclusion

In this contribution, we provide discussion on RAN4 requirements for adaptation of common signal/channel transmission. After discussion, the following conclusions are provided:

Proposal 1: RAN4 doesn’t need to discuss SSB adaptation in IDLE mode at current stage.

Proposal 2: minimum requirement upon SSB periodicity adaptation is needed:

- When SSB periodicity changes upon SSB periodicity adaptation, the cell identification, measurement period and L1 requirements apply based on the longer delay before or after the adaptation.

# Reference

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson