3GPP TSG-RAN WG4 Meeting #112bis		R4-2415492

Hefei, China, 14 – 18, 2024

Agenda Item:	6.24.2.2

Source: 	Apple

Title:	Discussion on conditional Intra-CU LTM

Document for:	Approval

# Introduction

In RANP#105, an updated WID for NR mobility enhancement phase 4 was approved in [1]. According to the update, conditional intra-CU LTM is to be supported in this release:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Correspondingly, RAN4 needs to specify RRM requirements to support the new procedure. In this contribution, we initially discuss RRM scope.

# Discussion

- Scope of conditional LTM cell switch

We first reviewed all LTM related requirements which were introduced in R18:

- Cell switch delay (section 6.3)

- PSCell switch delay (section 8.20 and 8.2)

- Early TA acquisition (section 6.2 and 8.2)

- Early TCI activation (section 8.25)

- L1-RSRP measurement period (section 9.14 and 9.15)

- L1-RSRP measurement accuracy for LTM (section 10.1)

- Test cases (annex A)

To support conditional LTM, it is straightforward that RAN4 needs to introduce conditional cell switch delay requirements. According to the WID, currently the scope is limited to the scenario where the UE is in non-DC. Therefore, conditional PSCell switch is not in scope. For early TCI activation and early TA acquisition, they can increase cell switch efficiency. However, in conditional cell switch procedure, so far it is unclear how network would trigger early TCI activation and early TA acquisition. One solution is to let network configure multiple events with different threshold, e.g. 1dB (higher than serving) to trigger early TCI activation and early TA acquisition and 3dB to trigger conditional LTM. This solution has no further specification impact (once event triggered reporting and conditional cell switch requirements are finalized) and can be supported by network implementation. Alternatively, event triggered TCI activation is also possible. For instance, network configures event triggered TCI activation as well as corresponding event. Then UE decodes the configuration of event triggered TCI activation and keeps monitoring the conditions according to the configured events. When UE identified that condition of target TCI is met, UE automatically activate the target TCI. Similarly, event triggered early RACH procedure can also benefit the system. One difference is that the most suitable beam (target SSB) cannot be reliably predicted by network. RACH with adaptation may need to be discussed to allow UE to perform RACH based on a different SSB (other than the one indicated by network) since measurement period would be quite long (e.g. in FR2) and UE knows better what target SSB is the most suitable one. Note that this issue also exists in R18 early TA acquisition, i.e. time gap between UE measurement (could much earlier than report) and PDCCH reception for early RACH could be so long that the most suitable SSB might have already changed. However, these issues need more study. In current phase we propose to focus on clear scope.

Proposal 1: define conditional LTM cell switch delay requirements in this work item. Conditional LTM PSCell switch delay is not considered in current stage.

In conditional LTM procedure, UE keeps evaluating conditions based on network configuration. Once conditions are met, UE automatically triggers LTM. In conditions evaluation phase, UE performs measurement same as legacy, e.g. based on R18 L1-RSRP measurement requirements. Thus no new measurement requirement is expected. New requirements for CSI-RS based L1 RSRP measurement are needed. However, they are expected to be discussed in the dedicated agenda. Note that in late stage of R18, L1-RSRP measurement and LTM cell switch were decoupled. To increase applicability of conditional LTM cell switch, it is proposed to consider L3 measurement in event configuration for conditional LTM. Note this has RAN2 impact. RAN4 shall inform RAN2 if this is agreeable.

Proposal 2: no new measurement requirements are needed under this agenda.

Proposal 3: since L1-RSRP and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM.

Proposal 4: inform RAN2 if P3 is agreeable.

- Conditional LTM cell switch delay

Fortunately, conditional handover has been supported in standard long time ago. Corresponding requirements are very stable now. Conditional LTM cell switch is similar with conditional handover. CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Proposal 5: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Preliminarily, the following formular is proposed as conditional LTM cell switch delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

Proposal 6: conditional LTM cell switch delay is defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: define conditional LTM cell switch delay requirements in this work item. Conditional LTM PSCell switch delay is not considered in current stage.

Proposal 2: no new measurement requirements are needed under this agenda.

Proposal 3: since L1-RSRP and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM.

Proposal 4: inform RAN2 if P3 is agreeable.

Proposal 5: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Proposal 6: conditional LTM cell switch delay is defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

# Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom