3GPP TSG-RAN WG4 Meeting #112bis                                                           R4-2415496

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Apple

Title:	Discussion on Beam Management: RRM Core Requirements

Agenda Item:	6.17.4.1

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest WID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

Discussion

## 2.1 Framework for Defining Requirements in Tests for AI/ML

From RAN1 evaluation results there are many implementation factors that could influence the AI/ML model performance. Diverse company-specific assumptions and implementations regarding model structure and parameters often lead to significant performance variations. To ensure consistency across companies' results, incorporating a reference AI/ML decoder model becomes critical in defining performance benchmarks for the UE side encoder.

From a UE processing capability limitation perspective, RAN4 will need to find a measure to access and evaluate complexity. There should be some limitations and complexity considerations when RAN4 decides on the requirements.

As an example, from RAN1 performance evaluations for the single sided BM use case, the complexity of AI/ML models reported across the companies can vary drastically as it is shown in the figure below from TR:

Fig 1: Complexity of AI/ML models from evaluation results 
in terms of FLOPs and number of parameters for BM cases

Accordingly, the performance variations across companies as outlined in Section 6.3.2.1.1 from the TR can vary from 60% accuracy to 90% accuracy where most of the performance variation could be attributed to model complexity and other differences in simulation assumptions (training procedures, training data, etc)

One question that arises is: What should be the granularity for defining the parameters to specify the reference AI/ML model for BM use case? In addition to model parameters, other factors that could affect performance include the model output type and the associated types of training data. For example, significant performance variations across companies are reported for the BM use case in section 6.3.2.3 of the TR. These variations depend on the output type, such as labels (classifier network) or L1-RSRP (CNN or MLP). For training input types for BM we could also differentiate between wide beams (SSB based, wide Tx beams) vs narrow beams (CSI-RS based, narrow Tx beams) types of set B of beams.

Similarly to the list of parameters specified for the two-sided model in CSI use case, RAN4 should investigate the set of parameters and granularity for the other use cases. (BM and positioning)

For example, for the BM case we could start with the following parameters:

- General Complexity

  - Number of parameters

  - Memory Size

  - FLOPS

- Training output type (labels vs L1-RSRP, Classifier vs MLP)

- Training Input type (narrow beams (CSI-RS) vs wide beams (SSB) Tx beams)

We provide a table with the proposed parameters for specifying reference model for BM use case. (table 1)

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parameters | Model type | CNN(case 1), MLP (case 1_, RNN (Case 2), LSTM (Case 2) |
|  | Fixed point representation | Int8, int16, floating point etc |
| Model Training/Inference related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, etc) |
|  | Loss function | NMSE, Cross Entropy etc. |
|  | Training datasets | Channel model (CDL) number of set -B/ set-A beams, specify TX codebooks. Best Rx beam selection (best UE beam) SSB beams (setA and setB) And /or CSI RS beams |
|  | Impairments (can be used for inference too) | RSRP measurements with impairments: Impairment types affecting set B measurements and set A measurements. |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, max epochs,etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Complexity | Model Parameter Size |  |
|  | Model Memory Size |  |
|  | FLOPS |  |
| Scalability | Supported set A | 64 beams |
|  | Supported set B pattern | 1/4, => 16 beams |
|  | Time Window configuration (BM case 2) |  |
|  | Number of future predicted instances (BM case 2) |  |
| Performance Metric | Beam ID, RSRP accuracy | Top-K beams, values of K? |

## Table 1: Parameters for Specifying Reference Model for BM use cases

## 2.3  KPIs/Test Metrics for BM Use case

In TR 38.843 the following agreements are captured  for KPIs/Test metrices for BM use case.

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-	Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition.

For option 1, RSRP prediction accuracy focus is about the RSRP difference for the same beam. RSRP prediction accuracy is more similar as legacy RSRP measurement accuracy definition:

Absolute L1-RSRP accuracy = measured L1-RSRP of beam index n - ideal absolute L1-RSRP of beam index n

RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

From RAN4#110-bis the following agreement has been reached:

Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy. Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity. Following this perspective and in accordance with legacy schemes, a minimum RSRP accuracy test should be applied to all reported/predicted beams. Additionally, in legacy systems, there is no testing to confirm if the reported beam powers correspond to the best genie beam indices. Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams.

Observation 1: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

In line with the legacy approach, we aim to ensure RSRP accuracy for all predicted beams. When using neural networks for regression, we can rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

When considering the necessity of testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Proposal 3: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

2.4  Datasets for training/testing

For training the AI/ML models for beam management we need to agree on what the source of the training data is.  In RAN4#111 the following agreement was reached:

Issue 2-6:	Datasets for training/testing

- Proposals

  - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm )

  - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment)

  - Option 3: others

- Recommended WF

  - To be discussed

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

We need to agree if the goal of the training data is to train the model for testing purposes only or for training the model for real deployment as well.

If the training data will be used to train one model (used in testing and deployment) it would be hard to gurantee similar performance (testing environment and in the field) since Tx codebooks could be different from vendor to vendor.

Proposal 4: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

Observation 4: RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

From RAN4 #112 the following agreements were reached:

Issue 1-2: Channel modeling

Agreement:

- Use synthetic channels as the default assumption for conformance requirements/test.

  - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

    - The field data can be considered for the analysis.

Issue 1-3: Testing goals

Agreement:

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - FFS on whether and how many different test configurations/parameters are used for tests.

  - LCM would be tested

For better clarity and understanding, it is beneficial to distinguish between three specific sets of scenarios/conditions:

- All possible Deployment Scenarios/Conditions: These include the entirety of scenarios in which the AI functionality is intended for deployment. These conditions encapsulate the comprehensive real-world scenarios in which the AI system will operate, with the aim of meeting the specified RAN4 requirements within this context.  The requirements could in general be met with either AI/ML or legacy procedures.

- RAN4 Specified Test Conditions: These conditions encompass the specific scenarios for which RAN4 tests will be defined, ensuring that the requirements for these tests are met. These conditions should include the identified scenario as well as the other scenarios for generalization tests.

- Non-Tested Conditions that are part of possible Deployment scenarios/conditions: This category includes additional conditions that are part of the possible deployment conditions but are not explicitly addressed during RAN4 testing. These additional conditions could be met with either AI/ML or legacy operation through LCM procedures and post deployment  (Set 1 = Set 2 union Set 3)

For RAN4 specified test conditions, the testing data could be generated across a variety of different conditions/congiguations:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/set A

- Tx codebooks, Tx antenna architecture layout , antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

We have the following proposals:

Proposal 5: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/ set A

- Tx codebooks,Tx antenna architecture layout, antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 6: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

2.5 Measurement Acuracy for BM

In RAN4#111 the issue of measurement accuracy was discussed as follows:

Measurement accuracy

Several companies brought up the need to improve measurement accuracy (e.g. RSRP measurement accuracy) to enable better AI/ML performance

Issue 2-8:	Measurement error impact

- Proposals

  - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study

  - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now

  - Option 3: No need to evaluate the measurement error impact

- Recommended WF

  - To be discussed

In RAN4#112 the issue of measurement accuracy was discussed as follows:

Issue 2-6:	Measurement error impact

Agreement:

RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study.

impact on different prediction accuracy metrics should be taken into account

studies already performance by other groups(RAN1) could also be taken into account

Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

In case the training data are based on real measurements there is a tradeoff between data quality and model generalization: In both BM-Case 1 and BM-Case 2 sub-use cases, L1-RSRP measurements serve as inputs and outputs for the AI-ML model. However, these measurements are susceptible to errors due to RF impairments,  thermal noise, quantization noise and other non-ideal components at the UE receiver. The range of measurement errors for FR2 is defined by current L1-RSRP requirements outlined in Clauses 10.1.20 of TS 38.133.

For both the Base Station (BS) and UE side models, these measurement errors impact the input L1-RSRPs for SetB beams. Simultaneously, during the training phase, measurement errors (and quantization errors) also influence the output because the labels/RSRPs are determined based on the non-ideal L1-RSRP of SetA beams. This interplay of errors affects both input and output data in the ML model training and inference processes. We would want to avoid the scenario where the AI/ML model attempts to fit the noise or impairment errors.

- Observation 5: The source of training data for beam management will play a crucial role in AI/ML BM performance and in the generalization performance in real deployment

Moreover, there is variability among the magnitude of errors. Such a concern arises when we consider the trade-off between data quality and model generalization. For instance, consider two UEs: UE 1 positioned at the center of the serving cell and UE 2 at the cell's edge (low SNR). We opt to use ground truth to retain "good data" (from UE 1 due to favorable Signal-to-Noise Ratio or SNR) while discarding "bad data" (from UE 2 due to poor SNR), undoubtedly enhancing the model's training performance but trading off generalization performance for users with low SNR conditions. Deploying this model in real-world scenarios where radio conditions (measurments at low SNR) and obstacles, such as Non-Line-of-Sight (NLOS) elements, significantly differ from the controlled test environment, can result in degraded model performance. This discrepancy jeopardizes the model's ability to generalize well, undermining the efficacy of RAN4 testing goals.

Observation 6: For training data based on real measurements, the quality of  training data depends on RF impairments, and other noise sources. There is tradeoff between training data quality and generalization performance. With training data collection from the field (across varying SNR conditions), both set-B and set-A beams will be affected by impairments

The performance of the AI/ML model will depend upon the following factors:

- Measurement accuracy of RSRP

- Assumption on the side conditions, example SNR levels

- Training data assumptions (e.g. what was the impairment model assumed during training)

In order to investigate the performance degradation that results from measurement accuracy we propose to specify an impairment model with its associated parameters so that companies are providing comparable simulation results. We can consider the parameters in the impairment model specified in Fig 8 as a baseline.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Training Data for BM | Training datasets | System level channel model (SLS,Uma, etc) number of set -B/ set-A beams, specific TX and RX codebooks. Antena port layout Best Rx beam selection SSB Wide Tx beams vs CSI-RS Narrow beams Tx Codebook Tx antenna array dimensions Antenna Element Spacings |
|  | Impairment Model to be added to set B and set A measurmenets | Impairment types affecting set B measurements and set A measurements, quantization, SNR levels, RF impairments, other side conditions , UE measurement accuracy modelling, e.g., correlations between errors for different gNB beams, minimum SNR where RSRP is measured, noise figure, etc |

Fig. 8:  Proposed parameters to generate training data with impairment model for BM

Proposal 7: To investigate performance degradation resulting from measurement accuracy, we propose specifying an impairment model as shown along with its associated parameters. This ensures that companies provide comparable simulation results, which will guide the decision on changing RSRP accuracy requirements.

If the outcome of simulating the performance from the impairment model results in degraded performance, then we can consider the following options:

- Make the AI/ML adaptable to changing SNR conditions, for example to maintain performance the number of set B beams can increase, also the number of K in top-K could increase to compensate for degraded performance.

- Change the side conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

- If the side conditions significantly change to support AI/ML functionality, then we could consider tighter accuracy requirements

- L1-RSRP measurement is usually a 1 shot measurement, introduce some avegaging over samples to improve the measurement accuracy for AI/ML operation

Proposal 8: If the outcome of simulating the performance with the impairment model results in degraded performance, then we can consider the following options:

- Adjusting RSRP Requirements and Enhance Measurement Accuracy: Reevaluate and potentially modify the current RSRP accuracy requirements to better align with the observed performance under the impairment model.

- Adopting Compensation Mechanisms: Make the AI/ML adaptable to changing SNR conditions,

  - To maintain performance the number of setp B beams can increase

  - The number of K in top-K could increase to compensate for degraded performance. (change model, address scalability of AI/ML model).

  - Switch from RSRP prediction to beam prediction with an increased number of beams (K). UE signals the predicted beams to the network (NW) and relies on measurements, rather than predictions, of the NW-transmitted beams identified by the UE.

  - Apply AI/ML denoising learning techniques to reject noise and impairments and make the model robust to a range of SNR values

- Change the side conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 2: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

Observation 3: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 4: RAN4's goal of aligning test conditions with real-world deployment scenarios is closely related to ensuring generalization performance. Since field deployment conditions can vary significantly across different environments, the test conditions should account for this variability

- Observation 5: The source of training data for beam management will play a crucial role in AI/ML BM performance and in the generalization performance in real deployment

Observation 6: For training data based on real measurements, the quality of  training data depends on RF impairments, and other noise sources. There is tradeoff between training data quality and generalization performance. With training data collection from the field (across varying SNR conditions), both set-B and set-A beams will be affected by impairments

Proposal 1: RAN4 should investigate the specification of reference AI/ML models for BM, the associated training procedures, and the training datasets to ensure alignment of simulation results. This will help pave the way for defining performance requirements for the BM use case, with careful consideration of constraints on model complexity. To initiate the discussion, we have provided a reference table as a starting point.

Proposal 2: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

Proposal 3: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Proposal 4: RAN4 should agree that the training dataset used in testing is strictly for testing purposes. In real-world applications, it will be the responsibility of UE implementations to determine the appropriate training dataset. This could involve using a vendor-specific dataset or creating a mixed dataset sourced from multiple vendors for training their models.

Proposal 5: To define the testing data for BM and provide sufficient test environments/conditions for vendors to generate the necessary training data, RAN4 should select a set of standardized test conditions. UEs will be tested across these identified conditions, and it will be up to the UE implementation to either switch between models or employ a supermodel capable of accommodating all these conditions during. Example of test conditions/configurations are:

- Details of the channel fading characteristics (CDL, LOS/NLOS, etc)

- Different paterns of set B/ set A

- Tx codebooks,Tx antenna architecture layout, antenna spacing

- SNR conditions

- Carrier Frequency

- Doppler conditions, measurement window configurations and window of predicted time instances (BM case 2)

Proposal 6: For all potential non-tested conditions that are part of possible deployment scenarios, including additional conditions not explicitly addressed during RAN4 testing, we propose to employ Post-Deployment Procedures. These procedures will augment RAN4 conformance testing to effectively manage and assess performance across all possible deployment conditions

Proposal 7: To investigate performance degradation resulting from measurement accuracy, we propose specifying an impairment model as shown along with its associated parameters. This ensures that companies provide comparable simulation results, which will guide the decision on changing RSRP accuracy requirements.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Training/Inference Data for BM | Training datasets | System level channel model (CDL,Uma, etc) number of set -B/ set-A beams, specific TX and RX codebooks. Antena port layout Best Rx beam selection SSB Wide Tx beams vs CSI-RS Narrow beams Tx Codebook Tx antenn array dimensions Antenna Element Spacings |
|  | Impairment Model to be added to set B and set A measurmenets | Impairment types affecting set B measurements and set A measurements, quantization, SNR levels, RF impairmenets, other side conditions, correlations between errors for different gNB beams minimum SNR where RSRP is measured, noise figure, etc |

Proposal 8: If the outcome of simulating the performance with the impairment model results in degraded performance, then we can consider the following options:

- Adjusting RSRP Requirements and Enhance Measurement Accuracy: Reevaluate and potentially modify the current RSRP accuracy requirements to better align with the observed performance under the impairment model.

- Adopting Compensation Mechanisms: Make the AI/ML adaptable to changing SNR conditions,

  - To maintain performance the number of setp B beams can increase

  - The number of K in top-K could increase to compensate for degraded performance. (change model, address scalability of AI/ML model).

  - Switch from RSRP prediction to beam prediction with an increased number of beams (K). UE signals the predicted beams to the network (NW) and relies on measurements, rather than predictions, of the NW-transmitted beams identified by the UE.

  - Apply AI/ML denoising learning techniques to reject noise and impairments and make the model robust to a range of SNR values

- Change the side conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

- References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[4] TR 38.810, v16.7.0, “Study on test methods”.