3GPP TSG-RAN WG4 Meeting #112bis                                                     R4-2415497

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Apple

Title:	Discussion on Beam Management: Testability and Interoperability Issues

Agenda Item:	6.17.4.2

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest WID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

Discussion

## 2.1 Beam Prediction Testability Discussion

According to the discussion during RAN4#111 [ R4-2410571] the following agreements have been reached:

Agreement:

Maximum number of set B Tx beams that test system should be able to emulate: [8-16]

Maximum number of set A Tx beams that test system should be able to emulate:  [64-128]

FFS on AoAs

UE rotation during the test: FFS

UE rotation/repositioning between different tests: Yes

Issue 2-3: Channel models

Agreement: Further discuss and down-select the following options considering TE feasibility

- Option 2:  TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Issue 2-4: Test setup needs

Issues listed below to be further investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

      - number of beams transmitted simultaneously

      - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

Other aspects can also be considered.

We will discuss two options for the Beam Prediction Testability:

Option 5: Multipath based Testing Setup

To verify the accuracy of the L1-RSRP prediction for BM case 1 and case 2, we will need to introduce enough randomness in the spatial and temporal domain.  As it is shown in Fig 2, the propagation environment characterized by the different AoAs and AoDs, effectively sample the radiation beam patterns across the mainlobe and sidelobes of each of the beams in set B/set A at Tx and Rx antenna arrays. Fig 2 shows the radiation pattern of one beam inside set B or set A. The spatial mapping from set B to set A is learnt through all possible samplings of set B Tx beam patterns according to the AoDs along with samplings of the Rx beams across AoAs. AI/ML can learn the mapping from set B samples to set A samples by exploiting the correlations of the radiation patterns. For this learning to be complete we should sample the radation patterns at many points in spatial domain. This could be achieved by introducing randomness in AoA/AoD domain since the channel response is determined by the Tx beam response at the direction of the AoD.

In the presence of multiple AoAs and AoDs, the resulting L1-RSRP is the superposition of these spatial beam samples at both Rx and Tx. The spatial mapping of set B L1-RSRP to set A L1-RSRP is learnt by proving enough random data  to capture the spatial correlations across different angles of departures and arrivals as well as the different superpositions of those angles. The correlations inferred will depend on the specific mainlobe as well as the sidelobe shapes of the setA/B patterns which will have to configured in the test and emulated by the test equipment. Therefore, the AI/ML model should be trained and tested with enough randomness.

Observation 1: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 2:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

It is expected that during deployment in the field, all the aforementioned randomness and variations will be inherent in the real propagation environment. Therefore, the AI/ML model should be tested under similar conditions, otherwise, the UE could easily pass the test and fail during real deployment scenarios.

Observation 3:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Fig. 2: Beam pattern spatial domain sampling across AoA/AoD

The baseline measurement setup for RRM characteristics and SNR referenece point for RRM testing procedure based  on TR 38.810 [4] for the study on test methods specifically for FR2 UE beam management is shown in Fig 3.

Fig. 3:  UE RRM test methodology for FR2 Beam Management

For the testing goals of BM AI/ML use case as described in the paragraph above, there are two main limitations with the existing test methods:

- Limitation on the number of simultaneously active AoAs (=2)

- Limitation of the relative angular separation between AoA pairs: 30°, 60°, 90°, 120° and 150°.

There are questions regarding how to meet the testing goals and RAN4 requirements using the FR2 OTA chamber, particularly with the necessity of sweeping beam patterns across Set-A and Set-B, but with the limitations of only two Angle of Arrivals (AoAs) and coarse angular separation. Moreover, for BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation.

Observation 4: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

In [5] it was concluded that we would neeed for the current 3D MPAC to approx. >100 probes i.e. also  >200 RF channels (to count for dual polarized probes) for emulating CDL-C for AI/ML based BM testing. This would be simply unfeasible. Thus, further simplifications of the test scenarios are necessary. For feasible implementation of an OTA test setup for AI/ML based beam management, further simplifications are inevitable.

For beam management, the primary goal is not to develop an exact or near-perfect representation of the resulting CDL channel model in static scenarios (such as FR2 MIMO OTA analyzed in earlier sections). Instead, the focus is on emulating a time-varying power environment with limited resolution, thereby avoiding the need for physical simulation of angular spread at both the cluster and overall levels

Proposal 1: Multipath based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

- How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

- Consider incorporating UE rotation for the test

- How many different CDL channels we need to support (considering generalization purposes as well)

For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field

(2) Reliability of using synthetic channels for test data in evaluating models trained on real data.

Observation 5: For RAN 4 testing we should have the following goals:

(1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data.

Based on the above goals we have the following:

Proposal 2: To establish a criterion for assessing the feasibility of a simplified sparse probe layout (Option 5) for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.

Fig. 4:  Channel Emulator Based Spatial channel for testing BM

Below we provide some details on how to emulate a CDL channel with a time varying power across the AoA probes.

During RAN4#110-bis a spatial model was proposed to be implemented by the channel emulator as shown in Fig 5.

Fig. 5   Multipath based Testing Setup with a simplified CDL channel and probe layout

The following parameters that can be programmed:

Tx codebook and antenna pattern. The Tx gain is known given the codebook and antenna pattern (across 2 polarizations) The AoD angles from the Tx side. We assume that the AoA angles are fixed and they are part of the probes in the MIMO OTA testing setup. The coupling factors that couple the AoDs clusters to the AoAs clusters Gc  AoDs represent the first stage of clusters and AoAs represent the last stage of clusters. Any intermediate clusters and interactions can be abstracted by the coupling factors Gc,ij(AoAi,AoDj). The different AoDs that couple to AoA_i are denoted as AoDij.  A Tx beam (from set A or set B) is denoted as wk. A UE RX beam is denoted as um. The RX codebook is denoted as U.

By taking into consideration the testing set-up shown in Fig 5, the RSRP for a particular Tx beam is derived as shown below:

Fig. 6:  Proposed BM-Case 1 spatial prediction testing setup ( multiple static channels)

For testing the RSRP accuracy for BM-Case1 spatial prediction we can configure multiple static configuration tests, where each static scenario corresponds to a set of parameters for generating the channel realizations. For each test we configure UE to perform measurements on the set A and we treat those measurements as the genie measurements. The whole procedure is shown in Fig 6.

Proposal 3: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 6. (multiple static configurations)

For testing the RSRP accuracy for BM-Case2 temporal prediction we can configure a non-static configuration test, where the set of parameters for generating the channel realizations vary over time.Therefore, the channel realization vary due to inherent propagation conditions that should alter the optimum Tx beam over time and introduce some randomnsee and correlation over time to be captured by the AI/ML. Similar to BM-Case 1 testing procedure, we configure UE to perform measurements on the set A and we treat those measurments as the genie measurements. The whole procedure is shown in Fig 7.

Fig. 7:  Proposed BM-Case 2 temporal prediction testing setup ( non- static channels)

Proposal 4:  For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Fig 7, through a non static configuration of the channel emulator.

Option 6: Single path/cluster (LOS condition) based Testing Setup

To simplify the testing setup, we can design it under the assumption of near-LOS conditions, which can be emulated using a single cluster and a single AoA probe. This is shown in Fig 7. Note that the multiple AoD links are not active simultenously. Only one link is active at a time, the purpose is to emulate the gains across different angles of the Tx antenna pattern to have multiple samples across the AoD domain and help making different Tx beams distinguishable. We also have one AoA/probe.

Fig. 8:  Single path/cluster (LOS condition) based Testing Setup

For each AoA (spatial sample) we collect RSRP inputs across setB beams (x,y) and we predict the RSRP at the same AoD for beams in set A (z). Let’s assume we consider the angle AoDj.  The received power at UE by using beam x is given by:

RSRPx,AoDj=PtGtx(x,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

Similarly the received power at UE by using beam y is given by:

RSRPy,AoDj=PtGtx(y,AoDj)×Gpath(AoDj,AoA)×Grx(AoA)

The AI/ML model will receive as inputs the RSRPs at set B beams and make a prediction as beam z (in set A):

RSRPz,AoDj=fAI/MLRSRPx,AoDj,RSRPx,AoDj

During test, we can generate multiple realizations of gains by varying the AoD as well as the gain Gpath(AoDj,AoA) to collect multiple RSRP samples across the set B beams and test the prediction against the RSRP of set A beams. The gain Gpath(AoDj,AoA) should remain constant during one realization of testing (sweeping of set B, set A beams).

One remaining question is the availability of ground truth data for comparison with the predicted values. We envision the following procedure. We first sweep the set B of beams and have the UE report the predicted RSRP values. Subsquently, we sweep the set A of beams and we treat the measurments as the ground truth.

Overall we have the following proposal:

Proposal 5: For BM spatial prediction RAN4 to consider the following options option 5 and option 6

Option 5 Multipath based Testing Setup with a simplified CDL channel and probe layout. Criterion of evaluating the feasibility of a particular simplified CDL model has has been proposed.

Option 6  Single path/cluster (LOS condition) based Testing Setup

Observation 6:

Advantages (Option 5): The channel model captures simultaneous reception across multiple AoAs/AoDs, providing a more accurate representation of CDL channels and real-world field data.

Disadvantages (Option 5): It may lead to a more complex testing setup.  There could be challenges in selecting the appropriate simplified CDL channel.

Advantages (Option 6): The testing setup is simplified, and existing OTA set up can be reused. For beam prediction only (not RSRP prediction) it may be possible to avoid transmission of set A beams since TE could know the best beam before hand.

Disadvantages (Option 6): It may not accurately represent real field conditions. The reliability of using a simplified channel model for testing data may be questionable when evaluating models trained on real-world data.

2.3 Generalization issues for BM

For AI/ML based BM solution, generalization poses one of the main challenges for RAN4 testing. Generalization issue includes the following main aspects:

Changing radio conditions

If an AI/ML functionality/model has been trained primarily on a dataset representing specific radio conditions, it may encounter diminished performance when faced with varying channel conditions in real-world environments. For example different propagation environments characterized by different angular/delay spread (LOS/NLOS) can alter the spatial mapping from set B to set A of beams learnt by AI/ML. Similarly the spatial mapping between set B and set A can be impacted by the carrier frequency/band of the measurements.

Changing configurations/parameters settings (conditions and additional conditions)

- The performance of different AI/ML applications is greatly influenced by the level of generalization achieved  which depends on the configuration and parameter settings utilized for training dataset creation. For instance, in the context of beam management in AI/ML, configurations (conditions and additional conditions) need to consider various beam sets and codebooks, along with variations in the number of wide and narrow beams (Fig 9), different beam widths, Tx angles, a diverse grid of set B beam configurations,  and measurements across multiple frequencies or bands.

Fig. 9:  Various beam sets and codebooks across different cells

Observation 7: To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

For the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signalling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

The discussion concerning "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" closely aligns with the RAN1 agreement regarding AI/ML functionality and models. These are proposed for the identification of AI/ML functionality/model, as outlined in the following excerpt from the agreed RAN1 section of TR 38.843:

<RAN1 Agreement, captured from TR38.843 v1.3.0> For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. ... For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

A model associated with configuration/condition associated with UE capability and additional conditions is shown in Fig 10.

Fig. 10:  A model associated with configurations/conditions associated with UE capability and additional condition

Our view on the "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" is shown in Fig 11.  For the AI/ML functionality the UE capability report involves grouping of AI/ML models into buckets corresponding to non-overlapping sets of supporting configurations/conditions (conditions associated with Model A, Model B, etc). These conditions are associated with the UE capability for functionality. For each of these generic model there are sub-models that can be associated with additional conditions for model identification. There are two types of additional conditions. The first type is the additional conditions that can be specified (and they can be signaled as assistance information to the UE to select the proper model). These NW side additional conditions should be aligned between training and inference for the UE sided model for performance improvement. According to this information, UE can select the most suitable model to improve performance and pass the test.

Fig. 11:  Capability report and additional conditions

RAN4 requirements should define tests for the AI/ML model such that the testing data align with the training data. That is, some of the conditions and additional conditions for functionalities and model identification, such as the beam pattern in Set B, the number of beams in setA/B and their associated mapping (wide SSB, versus CSI-RS narrow beams), time window configurations for BM case2, etc should align between training and RAN4 test. In order for the UE to select the appropriate model, some indication should also be part of the test definition. This assistance information will be part of the additional conditons that could be shared from the NW (NW side additional condition). Other more proprietary information (3dB beam width, antenna spacing, Tx beam angles) will be part of the additional conditions that cannot be shared (not part of UE capability) and could be considered to be  part of generalization tests

As mentioned, UE sided model would require some information on the configuration of set B codebook pattern for inference. This could be indicated as additional assistance information. Based on RAN1 discussion, there is no consensus to support the assistance information from NW on NW-side beam shape information. Simulation results confirm that the consistency / association of Set B beams and Set A beams across training and inference is “beneficial” from performance perspective from :

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. Note: Whether specification impact is needed is a separate discussion.

Thus, concerning the testing implications, ensuring the consistency or association of Set B beams and Set A beams throughout training and inference should be guranteed in the RAN4 core requirements. This consistency serves as additional assistance information for inference at the UE from the network side.

Proposal 6:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

From the RAN1 agreement captured above, it can be seen that a UE will report the supported configurations as part of capability signaling for a certain model/functionality. The other scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

For defining tests we can identify a set of reference conditions/configurations based on the capability signaling report from the UE. Example of reference radio conditions that UE could support can be an AWGN propagation condition or a particular CDL channel (link level). For example UE capability could indicate that the output type of the AI/ML model is label based (classifier), therefore the model should be tested under option 2 for BM KPI test metric.

For the other scenarions/configurations for defining generalization tests, we capture the following paragraph from the TR: “. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test)

Specified UE configuration includes functionality and/or model ID if defined ”

For functionality based LCM, functionality is enabled by configurations supported by conditions indicated by UE capability. Given that configuration, and as part of generalization test we can vary some condition/additional conditions.  For example, tests can be defined where the patterns of beam set A and set B vary,  or their associated mapping changes (e.g. SSB to CSI-RS wide to narrow beam mapping), or different antenna configirations/layout like 4x16 antenna versous 8x8 antenna layout, different antenna spacings (e.g spacing = 0.8 lamda).   Another example of additional condition could be the  frequency/band of the sweeping beams, number of history measurements time measurements for BM case 2 (Set B pattern in temporal domain), system level propagation channels (Uma, Umi), different UE speeds, different beam shapes, etc Some of these conditions can be signalled explicitly or implicitly to the UE during the test and the UE can select the appropriate model. This is shown in Fig 12 where UE2 switches models to improve generalization performance based on assistance signaling of additional conditions.

Fig. 12:  Generalization performance improvement with model switch based on assistance information or autonomous model switch

For generalization testing, there is a type of addtional conditions which are the sensitive proprietary information that NW vendors may not be willing to share. Examples of these conditions are:

- NW side beam shape information

  - 3dB bandwidth, physical antenna spacing, antenna array size, Tx beam angles, boressight directions

For defining generalization tests, the identified scenarios and configurations shall be associated with the UE capability of an AI/ML-enabled Feature FG. For example, we could define a generalization test associated with model B as it is shown in Fig 13. Model B is associated with some specific conditions/configurations, for functionality identification as indicated by UE capability. Under that configuration, UE could have other models associated with additional conditions (these conditions cannot be specified). These additional conditions will be the different scenations that will be part of the generalization tests.

Some of the additional conditions can be shared during the test in the form of assistance information to improve the generalization ability of the UE.

For the other additional conditions that the NW cannot share due to proprietary information, we can employ a virtual ID in the training dataset that indicates the additional conditions that the model was trained with. By signaling this ID, the UE can select the model that was trained with this dataset.

Observation 8: For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 9: The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Proposal 7: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 8: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 9: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

2.7 Consistency between Training and Inference

In RAN4#110 bis  multiple companies brought up the need to discuss the consistency between set A and set B, otherwise it is not expected that beam prediction would work. In RAN4#111 the following options were provided:

Issue 2-7:	Consistency

- Proposals

  - Option 1: consistency between set A and set B is reflected by below aspects:

    - Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference.

    - Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?)

  - Option 2: Consistency should be defined in a different way

  - Option 3: no need for any consistency definition

- Recommended WF

  - To be discussed

Proposal 10: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

As it is shown in the figure below, in a non-static condition the set B configuration could change (let’s say B’). In this case the consistency is violated since set B’ is a different configuration from set B that the model was trained with. Consistency should be considered only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions.

Table 1. Examples of Static versus Non-static Conditions

Proposal 11: Consistency should be considered in RAN4 only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions (generalization)

The existence of generalization cases highlights the necessity of tackling performance issues that arise when the assumptions made during training, such as using a machine learning model trained with a specific set of network/user equipment (NW/UE)-side additional conditions, do not align with the assumptions made during inference. This misalignment can lead to certain performance degradations.

As agreed in RAN#102, ensuring consistency between training and inference regarding any identified additional conditions on the network side for inference at user equipment (UE) is a key objective for the beam prediction use case within the scope of Release 19 work item.

As highlighted in TR 38.843 (Clause 4.2.3), additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Moreover, additional conditions of a model can be divided into two categories: NW-side additional conditions and UE-side additional conditions

The following options can be taken as potential approaches to improve generalization and maintain performance by ensuring consistency between training and inference, in accordance with clause 4.2.3 of TR 38.843:

·  Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side.

·  Model training at NW and transfer to UE, where the model has been trained under the additional condition.

·  Information and/or indication on NW-side additional conditions is provided to UE.

·  Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

The concept of additional conditions was introduced during the latter stages of the study item when the necessity for these conditions became apparent due to the presence of AI/ML training variables that may lack standardization. For example, in AI/ML-based beam management use cases, elements of network topology such as cell layout or antenna configuration, or internal variables of user equipment (UE) such as UE speed, can influence the selection of the AI/ML model to be utilized during AI/ML operations.

Nonetheless, while these variables may be essential for training AI/ML models and thus required as input for AI/ML model operation, they may not be standardized due to proprietary information concerns. Therefore, categorizing these variables as additional conditions can facilitate the AI/ML operation.

Among the methods explored for ensuring consistency between the network and UE for these additional conditions, the "Consistency assisted by monitoring" methodology appears to be the least optimal solution. It is expected to cause significant delays in selecting the optimal AI/ML model for the UE.

Consider a scenario where multiple sets of beam management models are developed for different user equipment (UE) speeds, and the network intends to utilize the monitoring procedure to select the optimal model. In this case, the network must activate multiple models at the UE one by one, each associated with different UE speeds, until it identifies an AI/ML model that yields the best performance. During the period when the network is activating non-optimal model(s) at the UE, UE performance suffers, resulting in a delay in selecting the most optimal model.

Observation 10:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts.

Observation 11:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings (additional conditions), thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance.  UE complexity increases if the UE needs to store and manage multiple AI/ML models.

Proposal 12:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 13: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

If the UE doesn’t have the ID but the NW has the model ID, then the NW can transfer the model to UE and UE update its list of models:

Proposal 14: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

If the UE has a model that partially supports the NW-sided additional conditions then the NW can trigger data collection for fine tuning of the UE-sided model:

Proposal 15: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

If there is no model at the network or UE, then the network configures the UE to fallback to legacy mode.

In case there is no ID available, then NW can configure a model monitoring procedure with UE-side measurements to check if the consistency of the additional conditions can be achieved. If performance criteria are met the consistency is achieved and the model has been verified. Otherwise, fallback to legacy mode is configured.

Proposal 16: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 17: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

For model identification each model is characterized by conditions and additional conditions. An example is shown in Fig 12. The complexity of LCM model management increases since the number of candidate models is large. Deactivations, latency and delays can occur when model monitoring takes place or when an appropriate model is not found (in this case UE fallback to legacy)

Fig. 13:  Model Identification and NW-side additional conditions

The current schemes proposed to ensure consistency between training and inference for sustaining system performance is through assistance information or model identification which is only used for model selection.

Observation 12: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

We propose defining some form of assistance information signal as input to the AI/ML model, which will be incorporated into the model’s inference engine. We propose the employment of an AI/ML supermodel that supplements its core inputs with auxiliary information signals, which will be integrated into its inference engine. The form of auxiliary information signals should optimize the tradeoff between disclosing proprietary information and enhancing the generalizability of the AI/ML model. As a result, the number of required AI/ML models needed to be stored, managed, and tested will be drastically reduced. An example is shown in Fig 13.

Fig. 14:  AI/ML Super Model with auxiliary information signals

Proposal 18:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 19:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 2:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

Observation 3:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Observation 4: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

Observation 5: For RAN 4 testing we should have the following goals: (1) Provide confidence that if a DUT passes the test it will also perform well in the field (2) Ensure reliability of using synthetic channels for test data in evaluating models trained on real data.

Observation 6: (BM testing set up):

Advantages (Option 5): The channel model captures simultaneous reception across multiple AoAs/AoDs, providing a more accurate representation of CDL channels and real-world field data.

Disadvantages (Option 5): It may lead to a more complex testing setup.  There could be challenges in selecting the appropriate simplified CDL channel.

Advantages (Option 6): The testing setup is simplified, and existing OTA set up can be reused. For beam prediction only (not RSRP prediction) it may be possible to avoid transmission of set A beams since TE could know the best beam before hand.

Disadvantages (Option 6): It may not accurately represent real field conditions. The reliability of using a simplified channel model for testing data may be questionable when evaluating models trained on real-world data.

Observation 7: To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

Observation 8: For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 9: The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Observation 10:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

Observation 11:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

Observation 12: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

## Beam Prediction Testability Discussion (Testing set-up)

Proposal 1: Multipath based Testing Setup for BM: Evaluate the testability requirements for simulating time-varying input power to a sparse probe layout based on a simplified channel CDL model.  The key aspects to be considered are:

- How to determine the minimum number of clusters to be emulated without introducing bias in the results. Investigate a quantitative “goodness” criterion to selecting the number of probes/clusters.

- Consider incorporating UE rotation for the test

- How many different CDL channels we need to support (considering generalization purposes as well)

Proposal 2: To establish a criterion for assessing the feasibility of a simplified sparse probe layout (Option 5 simplified CDL) for beam management (BM) testing, the following procedure can be considered:

- Train a model using the reference CDL channel and test it with the simplified sparse layout-based CDL channel.

- Train a model using the simplified CDL channel and test it with the reference CDL channel model.

- Train and test a model using the reference CDL channel.

A similarity metric should be defined to compare the RSRP accuracies across these test cases. This metric would indicate the effectiveness of testing with the simplified CDL model. The similarity metric can be derived through simulations.  Below figure: CDL_ref is the reference channel to be approximated by the simplified CDL_TE channel during test.

Proposal 3: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Figure below. (multiple static configurations) (Option 5)

Proposal 4:  For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and visualized in Figure below, through a non static configuration of the channel emulator. (dynamic channel)

Proposal 5: For BM spatial prediction RAN4 to consider the following options option 5 and option 6

Option 5 Multipath based Testing Setup with a simplified CDL channel and probe layout. Criterion of evaluating the feasibility of a particular simplified CDL model has been proposed.

Option 6  Single path/cluster (LOS condition) based Testing Setup

Option 6 for BM testing set-up is shown below:

Generalization issues for BM

Proposal 6:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

Proposal 7: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 8: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 9: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

Consistency between Training and Inference

Proposal 10: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

Proposal 11: RAN4 to consider option 1 for achieving consistency between training and inference for only static conditions.

Proposal 12:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 13: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

Proposal 14: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

Proposal 15: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

Proposal 16: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 17: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

Proposal 18:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 19:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

- References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[4] TR 38.810, v16.7.0, “Study on test methods”.

[5]  R4-2411587,   “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz