3GPP TSG-RAN WG4 Meeting #112bis                            R4-2415499

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.3.1

Source:	Apple

Title:	Discussion on CSI Prediction: Testability and Interoperability issues

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Discussion on CSI prediction

In the previous meeting RAN4#112, the following agreements was reached for CSI prediction.

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

using relative throughput as the test metric for inference, the baseline denominator for relative throughput still requires further clarification.

In the current PMI test case, RAN4 uses random PMI as the baseline to evaluate whether the UE-reported PMI is accurate enough. This approach is analogous to a scenario where no UE reporting occurs.

According to the agreement, relative throughput should be tested for prediction accuracy. The γ value comparing predicted PMI versus random PMI can be used in the test. To demonstrate the performance gain of AI-based CSI prediction, the γ value from the AI-based prediction would differ from the γ value observed in previous non-AI tests.

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

For CSI monitoring, since SGCS is the direct KPI of the AI/ML model, SGCS may be considered as a monitoring KPI metric. The relevant KPIs and evaluation metrics for CSI prediction from RAN1, as outlined in TR 38.843 section 7.1.2, are noted below.

7.1.2 CSI feedback enhancement Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: … - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

As outlined above, for NW-side performance monitoring (Type 2) in functionality-based LCM, predicted CSI along with its corresponding ground truth is sent to the network. The same mechanism can be applied for testing, where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Proposal 2: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: In the context of CSI prediction performance monitoring, RAN1 is already exploring a network-side mechanism that relies on UE-reported ground truth. This same approach can be utilized to evaluate the accuracy of CSI prediction as a performance KPI within RAN4

Proposal 1: The γ value comparing predicted PMI to random PMI can be used in the test for CSI prediction. To infer the gains of AI/ML-based CSI prediction, the γ values can be compared between AI/ML and non-AI/ML cases.

Proposal 2: Same mechanism used for NW-side performance monitoring can be employed for testing where the UE provides both the current channel information and the predicted channel information. Subsequently, the TE can calculate the SGCS by comparing the current channel from the latest report with the predicted channel from the previous report.

# Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”