3GPP TSG-RAN WG4 Meeting #112bis                                                        R4-2415502

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.5.2

Source:	Apple

Title:	Discussion on Testability and interoperability issues for positioning accuracy enhancement

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-19 Work Item WI was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this paper, we provide some additional views on the topic of testability and interoperability issues for positioning accuracy enhancement.

Discussion

Positioning Use Cases

In TR 38.843 the following test metrics for positioning enhancement have been captured:

Both direct AI/ML positioning and AI/ML assisted positioning are considered.

For metrics for positioning requirements/tests, the candidate options include

-	Option 1: positioning accuracy: Ground truth vs. reported

-	only option available for direct positioning

-	Option 2: CIR/PDP, channel estimation accuracy

-	Option 3: ToA, RSTD and RSRP, and RSRPP

-	Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

The feasibility and testability of different options should be further justified in WI.

RAN1 evaluation was mainly driven to understand if AI/ML model based positioning can outperform legacy NR positioning method in terms of achievable positioning accuracy in the considered scenario and to also investigate the generalization capability of the AI/ML models for positioning.

In RAN1 #110bis meeting, the following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 	UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For case 1 and case2a, the AI/ML model is deployed at UE, both model input and model output may need data/reference signal transmission between UE and NW. For case 2b, case 3a and case 3b, the AI/ML model is deployed at NW side(e.g., gNB or LMF), only model input may need data transmission from UE to NW.

Testability and Interoperability Issue for Positioning Use Cases

### Case 1: UE-based positioning with UE-side model

By setting up a controlled test area or grid with known reference points, positioning accuracy can be systematically verified.  It is crucial to establish a reference for comparison: a grid or test area consisting of location points with known coordinates. This ground truth could be set up through several methods, including:

- Positioning Reference Units (PRUs): These are physical units deployed at known locations within the network. The position of each PRU is known and communicated to the TE. PRUs provide a highly accurate reference for determining ground truth positioning in a test setup.

- GNSS-Based Positioning: Another method of determining ground truth is by using GNSS (Global Navigation Satellite Systems), like GPS, which can provide location accuracy.

As the UE moves from one predefined point to another within the test area or grid, it periodically reports the inferred coordinates, which are derived from the AI/ML model’s positioning predictions. These predicted coordinates are then sent to the test equipment for comparison against the known ground truth.

Proposal 1: If the report of positioning from UE has been established by other WG, RAN4 to investigate the feasibility and testability for Case 1, where a UE predicts its position relative to ground truth obtained through physically located PRUs with known grid locations or derives the ground truth through GPS.

Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

We can simulate various propagation conditions by emulating a single path channel for Line-of-Sight (LOS) scenarios and multipath channels for Non-Line-of-Sight (NLOS) conditions, such as by placing obstructions between the transmitter and the user equipment (UE). To establish the ground truth, a legacy LOS/NLOS estimator can be employed, or it can be derived from different path loss models and exponents (refer to 38.901). This ground truth can then be compared against the AI/ML predictions of the LOS/NLOS state.

Proposal 2:  RAN4 to investigate emulating various propagation conditions like single path channel for Line-of-Sight (LOS) scenarios and multipath channels for Non-Line-of-Sight (NLOS) conditions in the test equipment. Ground truth can be established by legacy LOS/NLOS estimation.

Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

For Case 2b, the decision on whether to define new measurements is still being discussed in other working groups (WGs). RAN4 can begin investigating the related testability once it has received sufficient input from these groups

Even if CIR/PDP specifications may be determined by other working groups (WGs), the testing methodologies remain an unresolved issue. On one hand, the test equipment (TE) cannot establish the ground truth for CIR/PDP using current testing methods. Even though a Positioning Reference Unit (PRU) could potentially assist in this process, the limitations of existing techniques still pose significant challenges.

It is challenging to determine the extent to which the degradation of measurement accuracy like CIR/PDP will impact the final positioning accuracy. Specifying the test requirements or measurement tolerances remains a challenging issue.

Proposal 3: RAN4 can begin investigating the related testability of new defined metrics (like CIR/PDP) once it has received sufficient input from other groups

Proposal 4: RAN4 should investigate the relationship between the measurement accuracy provided by the UE and the eventual positioning accuracy at the LMF before establishing any requirements.

# Conclusion

In this section we summarize the list of observations and proposals we discussed in this contribution:

Proposal 1: If the report of positioning from UE has been established by other WG, RAN4 to investigate the feasibility and testability for Case 1, where a UE predicts its position relative to ground truth obtained through physically located PRUs with known grid locations or derives the ground truth through GPS.

Proposal 2:  RAN4 to investigate emulating various propagation conditions like single path channel for Line-of-Sight (LOS) scenarios and multipath channels for Non-Line-of-Sight (NLOS) conditions in the test equipment. Ground truth can be established by legacy LOS/NLOS estimation.

Proposal 3: RAN4 can begin investigating the related testability of new defined metrics (like CIR/PDP) once it has received sufficient input from other groups

Proposal 4: RAN4 should investigate the relationship between the measurement accuracy provided by the UE and the eventual positioning accuracy at the LMF before establishing any requirements.

- References

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”