# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415508

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.19.3.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112 the RRM requirements impact was discussed for MIMO phase 5 and WF [1] was approved. In this contribution we discuss RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] it was agreed that there are impacts to RRM core requirements with UE initiated beam management. In addition the following agreements were also made:

Issue 2-1-2: Scope of the scenario of event-triggered L1 beam reporting Agreement: - Intra-cell and inter-cell for FR2. - Note: If specific requirements cannot be applicable, discuss it case by case. Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement Issue 2-1-5: measurement metrics for beam report? Agreement: - Based on L1-RSRP - Note: others can be revisited if RAN1 have further agreement Issue 2-1-6: L1-RSRP for CSI-RS measurement? Enhancement for UE-initiated/event-driven beam management Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

During RAN1#118 the following working assumption was made:

- RAN1 would possibly define additional events other than Event 2 to trigger beam reporting.

- The same design as Event 2 will be reused for other events introduced.

In RAN4 we will define reporting delay and measurement delay requirements. These should be agnostic to the event that triggers report. Hence RAN4 defined requirements for UE initiated beam management should be applicable to all events defined by RAN1.

- RAN4 requirements should be agnostic to the event that triggers the UE measurement report.

RAN4 defined requirements for UE initiated beam management are applicable to all events covered in RAN1.

One open issue is how to define requirements for Mode A and Mode B

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period?

Agreement:

- RAN4 will define Event triggered measurement reporting delay requirement

- Further discuss:

  - Whether to support both mode A and mode B

  - Whether to differentiate mode A and mode B

  - Other related aspects

For beam report transmission procedure for UE-initiated beam reporting, following modes are supported:

- Mode A  - dynamically scheduling UCI by gNB

Figure 1: Mode-A timeline

- Mode B - UCI in pre-configured resource(s) for second UL channel

Figure 2: Mode-B timeline

For both Mode-A and Mode-B the UE first sends PUCCH either to request resources or notify about beam report. The time to PUCCH would be the same for Mode-A and Mode-B and this needn’t be distinguished between the two modes.

- UE sends PUCCH for both Mode-A and Mode-B and the time to transmit PUCCH is the same for both modes.

- Time to PUCCH transmission need not be distinguished for the two modes.

The UE sends beam measurement report on the configured 2nd UL channel for Mode-B. In case of Mode-A it first receives a UL grant before it can transmit beam report on the 2nd UL channel. Since there is an extra step to receive DCI before transmitting on the 2nd UL channel, the time to beam report is longer for Mode-A compared to Mode-B.

- The time for beam report on 2nd UL channel is different for Mode-A and Mode-B.

In general, both modes should be covered in RAN4 requirements. There are few options that can be discussed:

Option 1: Define event triggered reporting delay requirements for PUCCH transmission

Option 2: Define event triggered reporting delay requirements for beam measurement report

PUCCH transmission is an intermediate stage before the actual beam measurement report. It would be incomplete to define UE triggered measurement reporting delay up to only PUCCH transmission.

- It would be incomplete to define reporting delay requirements up to PUCCH transmission only.

RAN4 should define event triggered measurement reporting delay for beam report on the 2nd UL channel for both Mode-A and Mode-B. The starting point is when the event is triggered at the UE. The ending point is when the UE transmits the beam report on the 2nd UL channel.

RAN4 should cover event triggered measurement reporting delay for both Mode-A and Mode-B – Starting point is when event is triggered at the UE; Ending point is when the UE transmits the measurement report on the 2nd UL channel.

The components for the reporting delay are:

Mode A:

Time to PUCCH : time to first PUCCH after trigger( Tfirst-PUCCH ) or PUCCH periodicity (TPUCCH)

Time to receive DCI with UL grant: TDCI

Time to transmit on the 2nd UL channel: K2

Mode B:

Time to PUCCH : time to first PUCCH after trigger( Tfirst-PUCCH ) or PUCCH periodicity (TPUCCH)

Time to PUSCH transmission: Time to first PUSCH for beam report after PUCCH transmission ( Tfirst-PUSCH ) or PUSCH periodicity (TPUSCH)

The delay requirements can be defined as follows:

Tfirst-PUCCH or TPUCCH + TDCI + T1

Where:

- TDCI = 0 for Mode B; Time to receive DCI with UL grant in Mode A

- T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

In legacy requirements the measurement delay and reporting delay are combined for periodic reporting since measurement is based on configured report. For UE initiated beam report, the UE performs measurements on configured periodic resources and reports it based on the event.  Defining only reporting delay requirements is not sufficient, and we need to also define measurement period or delay for L1-RSRP measurement.

- It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report.

The measurement delay can be defined for L1-RSRP based new beam measurements for the following:

- Intra-cell SSB

- Inter-cell SSB

- CSI-RS (inter-cell or intra-cell is not distinguishable)

The delay requirements can be based on existing requirements for L1-RSRP measurement as a baseline and modified for new beam measurement.

Define measurement period for new beam measurement for event triggered reporting with L1-RSRP measurement period as baseline for intra-cell SSB, inter-cell SSB, CSI-RS.

Taking the L1-RSRP measurement delay for SSB as an example, the following need to be removed or further discussed

Treport: removed since there is no reporting period configured for UE triggered report

M: Number of samples – FFS if this should be included as configuration for new beams for measurement is not known yet.

P: Measurement sharing factor – FFS if existing can be used or modified for simplification

Considerations for HST scenarios/ enhancements: removed

- It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report.

From legacy measurement period, for new beam report remove – (1) Treport (2) considerations for HST enhancements/ scenarios

From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

Some of the open issues from [1] are addressed below -

Issue 2-1-8: Whether to filter L1-RSRP?

<Way forward>

- FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

The L1-RSRP measurements are single shot measurements without any filtering. We don’t see the need to introduce time-window based filtering or any other filtering as this is not part of the measurement report in legacy or for event triggered report.

Do not introduce any filtering for L1-RSRP – time window based or any other filtering for new beam measurement for event triggered reporting.

Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states?

<Way forward>

- FFS. Depends on RAN1 further progress.

The TCI state known/unknown condition is relevant only when TCI state switch is activated or indicated for PDCCH/ PDSCH ; PUSCH/PUCCH/SRS in case of joint TCI state. When the UE receives TCI state switch (MAC-CE or DCI based) to a TCI state, the known/unknown condition would be determined by the same criteria as legacy. The L1-RSRP report should have been received within 1280 ms of the TCI state switch command, and it doesn’t matter if the L1-RSRP report was based on UE triggered beam report or legacy measurement report.

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward>

- FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Faster Tx and Rx beam refinement is not in the scope of this WI. Keeping in mind the objectives of this WI, RAN4 shall define requirements associated with UE/ event triggered beam reporting with legacy assumptions for beam sweeping.

- Faster RX, TX beam refinement is not in scope of this WI.

Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting.

Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward>

- Option 1: No new accuracy requirements for existing L1 measurements are required.

- Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

The L1-RSRP measurement is the same as legacy whether it is for beam report configured or based on UE/event triggered beam report. With the assumption that same one-shot measurement for RSRP is employed for new beams as legacy L1-RSRP measurement, the accuracy requirements should remain the same. Hence there is no need to define new accuracy requirements for L1-RSRP measurements for UEIBM.

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

- RAN1 would possibly define additional events other than Event 2 to trigger beam reporting.

- The same design as Event 2 will be reused for other events introduced.

- RAN4 requirements should be agnostic to the event that triggers the UE measurement report.

- RAN4 defined requirements for UE initiated beam management are applicable to all events covered in RAN1.

- UE sends PUCCH for both Mode-A and Mode-B and the time to transmit PUCCH is the same for both modes.

- Time to PUCCH transmission need not be distinguished for the two modes.

- The time for beam report on 2nd UL channel is different for Mode-A and Mode-B.

- It would be incomplete to define reporting delay requirements up to PUCCH transmission only.

RAN4 should cover event triggered measurement reporting delay for both Mode-A and Mode-B – Starting point is when event is triggered at the UE; Ending point is when the UE transmits the measurement report on the 2nd UL channel.

Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B

- It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report.

Define measurement period for new beam measurement for event triggered reporting with L1-RSRP measurement period as baseline for intra-cell SSB, inter-cell SSB, CSI-RS.

- It would be incomplete to define only reporting delay requirements without covering measurement period/delay for the new beams for event triggered beam report.

From legacy measurement period, for new beam report remove – (1) Treport (2) considerations for HST enhancements/ scenarios

From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1.

From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification.

Do not introduce any filtering for L1-RSRP – time window based or any other filtering for new beam measurement for event triggered reporting.

matter if the L1-RSRP report was based on UE triggered beam report or legacy measurement report.

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

- Faster RX, TX beam refinement is not in scope of this WI.

Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting.

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.

# Reference

- R4-2414035, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung.