3GPP TSG-RAN4 Meeting  #112bis	R4-2415510

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.6.2

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Scope confirmation

In the WF, the following is put in brackets pending further confirmation. The reason is to allow companies to check the potential impact on UE implementation.

- [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks]

As to the number of fragments and whether the fragments are contained the 100MHz range, there are three different cases. This is shown in Fig. 1. In Case 1, two fragments are fully contained in a 100MHz range, which is the scope of the SI; in Case 2, two fragments are not fully contained in a 100MHz range; in Case 3, while the first two fragments, i.e., Fragments 1 and 2, are fully contained in a 100MHz range, there are additional fragments, say Fragment 3, which falls outside of the 100MHz range.

Case 1

Case 2

Case 3

Figure 1. Different cases in terms of number of fragments and whether the fragments are contained in 100MHz range

In terms of implementation, one RF chain including ADC can handle more than 100MHz. On the other hand, usually BB can handle up to 100MHz per CC, since 100MHz is the max. CBW in FR1. More CCs can always be processed separately. In Case 2, the bandwidth of Fragment 1 or Fragment 2 or both have to be reduced to be able to fit into 100MHz if we follow the scope of the SI.

On the other hand, if we only focus on Case 1 and leave Cases 2 and 3 out in the study, we may run the risk that the final requirements (to be specified in a follow-on WI of this SI) may be different from those for Cases 2 and 3. This is undesirable from RF design standpoint since the RF design usually considers the entire band, not part of the band.

There are two options:

- Option 1: The study is focused on Case 1 only following the scope of the SI.

- Option 2: The study is focused on Case 1 for the time being. Meanwhile, we can request a revision of the scope in Dec. RAN plenary to include Case 2 and Case 3.

To ensure the design of using a common RF chain to support larger than 100MHz, we prefer Option 2. Of course, we understand the decision is one of RAN plenary, not RAN4.

Proposal 1: Regarding the scope, there are two options. Option 2 is preferred pending RAN decision.

- Option 1: The study is focused on Case 1 only following the scope of the SI.

- Option 2: The study is focused on Case 1 for the time being. Meanwhile, we can request a revision of the scope in Dec. RAN plenary to include Case 2 and Case 3.

Furthermore, regarding the example bands, it is better to understand why band n39 is included. Currently, no intra-band non-contiguous CA has been specified for band n39.

Reference architecture

The discussion on reference architecture is captured in the WF as shown below

#### Issue 2-1-2: Reference architecture Recommended WF: - The existing requirements for DL NCCA was specified based on partially-shared Rx architectureaccording to TR36.823. - FFS on following options for reducing number of Rx chains for DL non-contiguous CCs: - Option 1: Only the “fully shared Rx chain architecture” needs to be studied - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz - Other options are not precluded - Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study - FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier.

Regarding the options for reducing number of Rx chains for DL non-contiguous CCs, we agree with Option 1, as the common RF chain (or fully shared RF chain) seems to be the way to reduce the number of RX chains. On Option 2, while we understand 4RX (and support of 4 layer DL MIMO) is only mandated for single carrier mode and UE can report how many MIMO layers in CA, we do not see a strong motivation/relevance for this relaxation under this SI, which is focused on reducing the number of RF chains to support intra-band non-contiguous CA and hence would spare RF chains.

Proposal 2: Regarding the options for reducing number of Rx chains for DL non-contiguous CCs, Option 1 is agreeable and the intention of Option 2 needs further clarification.

Regarding BB processing including FFT operation, in theory either single FFT or common FFT is possible. In real implementation, it is common to process each CC separately, including down-sampling, applying filtering, correcting IQ imbalance, etc. after ADC. In the case of common RF chain architecture, it is critical to rely more on digital filtering/processing to deal with the interference than the case of separate RF chains architecture.

It is worth noting that using single FFT may not be an option for some FDD band, say band 7, since > 60MHz channel cannot be supported with 15kHz SCS.

Proposal 3: It is proposed to leave it to implementation whether a single FFT is expected to cover the fragmented carriers or separate FFTs are expected for each fragmented carrier.

Power spectral density difference between carriers of co-located adjacent channel operators

The discussion on PSD difference is captured in the WF as shown below

#### Issue 2-1-4: Study on power spectral density difference between carriers of co-located adjacent channel operators Recommended WF - FFS on following options in next meeting - Option 1: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure) - For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level - equal PSD between two co-location operators could be assumed from gNB transmisstion point of view for evaluation purpose, as the starting point - The PSD difference from UE reception point of view could vary as per the different antenna type, antenna pointing direction, etc between operators - Option 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered - Other options are not precluded

It is true that for co-located deployment, if BS antennas from different operators points to different direction, there could be significant gain difference. In addition, different operators may apply different TX power. On the other hand, the RAN4 conventional assumption for co-located deployment is 6dB PSD difference for intra-band CA, based on which UE is designed and implemented. Therefore, it is reasonable to reuse the 6dB assumption.

Proposal 4: From UE perspective, to enable the use of common RF chain, it is proposed to reuse the 6dB PSD difference between carriers of co-located adjacent channel operators.

Prerequisite conditions

To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, as analyzed in our companion contribution [3], the blocker power level in the gap needs to be manageable to the UE. In other words, the blocker power level needs to be at an appropriate level that the UE can handle with common RF chain. It can either be a specified value in the spec or based on UE capability reporting. The Rx chain configuration/reconfiguration may be based on some assistant information by UE measurement of the blocker.

Proposal 5: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Others including signaling aspects

#### Issue 2-1-6: Others FFS following proposals in next meeting: - Proposal 1 - The impact of UE switching between common RF chain and separate RF chains, including potential interruption, should be studied. If there is RRM impact, RAN4 should consider it. - Proposal 2 - The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration. - Proposal 3 - gNB trigger UE to measure gap could be considered.

Besides the UE ability to switch, there needs to be signaling support for UE to inform the network of appropriate CA configuration it can support after switching to common RF chains allowed by adjusted RF requirements. Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of RF chain switching, however, the UE may need to update its supported CA configuration after it is configured with some CA combination by the network through RRC. An alternative way is that the UE reports to network that it is capable of Rx chain sharing for the supported band combination, either by additional signaling on top of existing band combination reporting or by defining a dedicated band combination reporting for UE supporting Rx chain sharing.

On the proposal that gNB triggers UE to measure blocker power in the gap in between two fragments, we agree this is an important aspect if the blocker power is not properly restricted. In this case, UE may or may not need measurement gap to measure the blocker, depending on if the network configures such measurement before or after configuring the CA for the two non-contiguous fragments (CCs). And if such a measurement of blocker is considered, there could be a third alternative on the signaling consideration. For example, the UE can first report high UE capabilities, i.e., support of CCs, assuming non-continuous CCs can be handled by common RF chain. If the blocker power is too high, the UE can inform the network that only fewer CCs can be supported. It is important to note that the need of blocker measurement is only needed if the blocker power cannot be properly restricted by deployment.

In addition, there is a proposal from [4] to consider a new syntax/notation for indicating support of fragmented carrier operation in a CA configuration. We agree this can be further explored.

Proposal 6: In terms of signaling, several alternatives can be considered for band combination reporting for UE capable of Rx chain sharing

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

Another point is how to efficiently configure and activate fragmented carriers. In legacy CA, typically secondary carrier is added and activated based on UE measurement. However, in fragmented carriers scenario there are multiple non-contiguous spectrum in one single cell. It is inefficient for UE to measure all spectrums and then derive cell level measurement report. One possible solution is to introduce anchor spectrum/SSB. In the figure below, UE measures the anchor SSB to determine cell level quality. After the SCell is added, network can further configure UE to measure other spectrums. Based on UE report network can add and activate corresponding spectrum in the same SCell. Some signaling update is expected to support the new procedure.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: Regarding the scope, there are two options. Option 2 is preferred pending RAN decision.

- Option 1: The study is focused on Case 1 only following the scope of the SI.

- Option 2: The study is focused on Case 1 for the time being. Meanwhile, we can request a revision of the scope in Dec. RAN plenary to include Case 2 and Case 3.

Proposal 2: Regarding the options for reducing number of Rx chains for DL non-contiguous CCs, Option 1 is agreeable and the intention of Option 2 needs further clarification.

Proposal 3: It is proposed to leave it to implementation whether a single FFT is expected to cover the fragmented carriers or separate FFTs are expected for each fragmented carrier.

Proposal 4: From UE perspective, to enable the use of common RF chain, it is proposed to reuse the 6dB PSD difference between carriers of co-located adjacent channel operators.

Proposal 5: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Proposal 6: In terms of signaling, several alternatives can be considered for band combination reporting for UE capable of Rx chain sharing

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2414311, "WF on fragmented DL carriers study," MediaTek

R4-24, "On UE RF requirements for fragmented carriers," Apple

R4-2413340, " Discussion on impacts on UE RF requirements and DL performance for Fragmented Carriers," Nokia