3GPP TSG-RAN4 Meeting  #112bis	R4-2415511

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.6.3

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Common RF chain vs. separate RF chains

As the goal of the SI is to study under what conditions a UE can reduce its number of RF chains to support two non-contiguous CCs within a CA combination, it is necessary to establish a common understanding of common/shared RF chain vs. separation RF chains. In Fig. 1, both architectures are shown.

- Common RF chain

- Separate RF chains

Figure 1. Two UE RF architectures (a) Common RF chain (b)Separate RF chains

Note that in the separate RF chain architecture, there is usually an eLNA before the splitting. Of course, there could be architectures of separate RF chains where there is no shared eLNA.

Receiver sensitivity

In current CA receiver sensitivity requirement, there is ΔRIBNC to allow relaxation due to TX noise at RX band, i.e., TX noise at SCC, which assumes UE architecture of separate RF chain.

Fig. 2 below (copied from [2]) shows for CA_n25(2A), PCC UL is closer to SCC in the CA receiver sensitivity requirement setting than the usual TX and RX in a single carrier case.

Figure 2. CA receiver sensitivity setting for band n25

With common RF chain architecture as shown in Fig. 1(a), the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, the SCC performance may take some hit. This performance hit is expected to be less severe than the TX noise at SCC. More detailed analysis is needed to quantize the SCC performance degradation.

In addition, we would like to propose to apply some relaxation like ΔRIBNC for PCC receiver sensitivity. This is because after the down-conversion, the mirror frequency of PCC UL would be close to PCC DL and become a blocker. With the wider analog BB filtering covering PCC and SCC, the filter rejection is reduced compared to single-CC narrowband filtering.

Proposal 1: ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC.

ACS

In current ACS requirement, P_interference = P_wanted + ACS – SNR. Depending on the CC bandwidth, the ACS requirement ranges from 26 - 33dB, meaning P_interference is 24.5 – 31.5dB higher than P_wanted, assuming the interference is 5MHz wide.

In real co-located deployment, the power difference between blocker (i.e. CC from operator B) and wanted signal (i.e., CCs from operator A) is expected to be lower. However, with the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the ACS requirement needs to be adjusted.

Observation 1: With the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the ACS requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the requirement needs to be adjusted.

In-band blocking

Similarly to the analysis for ACS, it is believed that the blocker level in the in-band blocking requirement needs to be adjusted.

It is important to point out that in the common RF chain case, the image of the blocker may fall onto one of the two CCs generating severe interference that cannot be rejected by any filtering. Similar interference issue takes place for ACS requirement, but the most severe case is in-band blocking case 2, where the power difference between blocker and wanted signal is the largest.

Observation 2: With the loss of any analog BB filter rejection for in-gap and reduced rejections for out-gap, meeting the in-band blocking requirement is much harder and likely to eat into the UE implementation margins. Especially, the image of the blocker in Case 2 would cause the most severe interference because of the large power difference between the blocker and wanted signal. Therefore, the blocker power level in the requirement needs to be adjusted.

In-gap blocker power level

From the analysis of both ACS and in-band blocking requirement, it is justified to limit the blocker power level. Furthermore, the blocker would impact RX dynamic range. For example, consider the extreme case where the gap is 13 times wide than a 5MHz channel, like CA_n3(2A). If we assume the blocker is 65MHz wide and there is 6dB PSD difference between the blocker and one of the two non-contiguous CCs, the blocker power is 17dB higher than the CC power, which would affect the ADC dynamic range requirement and the receiver performance. Therefore, it is necessary to limit not only the PSD difference between the blocker and CCs, but also the gap width between the two CCs. That said, if we follow the ACS and in-band blocking test configurations, in which the blocker BW is either 5MHz or the same as wanted signal BW, we may not need to limit the gap width. Some UE capability reporting could also be considered for UE to report its capability in handling blocker level and the maximum frequency span per shared RF chain. Note also that it was agreed in the SID that there is up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs.

Proposal 2: It is proposed to set the blocker PSD to the same as that of the high PSD CC, i.e., PSD difference is within 6dB.

Requirements for evaluation

#### Issue 3-1-3: Requirements for evaluation Recommended WF: Companies’ views are diverged but not controversial. Further discuss following bullets see if they can be agreeable as a package - Both TDD and FDD intra-band DL contiguous CA should be included - ΔRIBNC, ACS, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be evaluated - Out-of-band blocking and spurious response do not need requirement adjustment - Adjusted requirements for fragmented carriers shall be separated from the existing FDD/TDD intra-band carrier non-contiguous aggregation ΔRIBNC, ACS, IBB, NBB - Discussion on if a rejection level of the image caused by the in-gap interferer is needed - Discussion on new requirement(s) if identified is not precluded

We would like to share views on two points. First, on the minimum guard band sizes, it is not clear what it means. Does it mean we assume the current values specified in the specification should be kept or they are subject to adjustment, like other RF requirements such as sensitivity or ACS? Clarification is needed.

Second, currently there is no requirement on image rejection for NR. On the need of a rejection level of the image caused by the in-gap interferer, our current view is if PSD of blocker is limited, the image rejection requirement can be indirectly ensured. Since any new requirement would incur testing overhead, it is not preferred to specify a new image rejection requirement.

Proposal 3: It is not preferred to specify a new image rejection requirement. Instead, the blocker PSD relative to PSD of CCs should be limited, as in Proposal 2.

Other constraints

Besides the necessary adjustments to requirements, we propose to clearly specify the constraints:

- To ease the receive design when common RF chain is used, it is preferred to have the same SCS for both CCs. We think this can be reasonable since both CCs are in the same band.

- The two CCs are co-located which ensures good time alignment of the two CCs at the UE receiver. This is already assumed in the SID.

Proposal 4: It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC.

Observation 1: With the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the ACS requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the requirement needs to be adjusted.

Observation 2: With the loss of any analog BB filter rejection for in-gap and reduced rejections for out-gap, meeting the in-band blocking requirement is much harder and likely to eat into the UE implementation margins. Especially, the image of the blocker in Case 2 would cause the most severe interference because of the large power difference between the blocker and wanted signal. Therefore, the blocker power level in the requirement needs to be adjusted.

Proposal 2: It is proposed to set the blocker PSD to the same as that of the high PSD CC, i.e., PSD difference is within 6dB.

Proposal 3: It is not preferred to specify a new image rejection requirement. Instead, the blocker PSD relative to PSD of CCs should be limited, as in Proposal 2.

Proposal 4: It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2414311, "WF on fragmented DL carriers study," MediaTek