3GPP TSG-RAN WG4 Meeting #112bis		R4-2415512

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.3

Source:	Apple

Title:   	On UE RRM requirement for SBFD

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

At the last RAN4 meeting, a WF was agreed [2]. In this contribution, we continue to share our views on the open issues.

# 2	Discussion

In the WF, it was agreed to further discuss the side conditions of L1 based UE-to-UE CLI measurement and reporting, as copied below.

#### Issue 2-1-7: Side condition Options for further discussion and down-selection (for information purpose): - Time offset between DL timing and SRS arrival timing - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress. - SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP - Option 1: Rel-16 CLI SRS RSRP assumption - Other options are not precluded.

Time offset between DL timing and SRS arrival timing

In R16 L3 SRS-RSRP measurement, RAN4 agreed to consider time offset between DL reference timing in the serving cell and SRS arrival time to be no larger than Terror_SRS_RSRP, where Terror_SRS_RSRP is TC × NTA_offset + 4.67us for FR1 or TC × NTA_offset + 3.67us for FR2, assuming cell phase synchronization error of 3us and propagation delay of 1.67us and 0.67us based on ISD of 500m and 200m for FR1 and FR2, respectively.

To our knowledge, there was not much discussion in RAN1 on the SRS transmit timing at the aggressor UE or SRS receive timing at the victim UE. At this stage, it is uncertain if RAN1 will ever decide on this issue. Therefore, it seems reasonable to adopt the R16 CLI SRS RSRP assumption as the starting point, namely Option 1 above.

Regarding Option 2, remove the cell phase error from Option 1, if the SRS RSRP measurement is limited to intra-cell case, where both aggressor UE and victim UEs belong to the same serving cell, it is reasonable. However, the UE CLI measurement and reporting in RAN1 evaluation is not limited to intra-cell case only, it also includes the inter-cell case. To decide if RAN4 should consider Option 2, it is better to clarify the applicable scenarios from RAN1 standpoint.

Proposal 1: As there are no further conclusions from RAN1 on the time offset between DL timing and SRS arrival timing, it is reasonable to adopt the R16 CLI SRS RSRP assumption as the starting point.

Proposal 2: Regarding Option 2, remove the cell phase error from Option 1, it is better to clarify with RAN1 first if the L1 based UE-to-UE CLI measurement and reporting is limited to intra-cell case only. RAN4 can send an LS to clarify the applicable scenarios.

SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP

On SRS Es/Iot, the R16 L3 CLI measurement accuracy requirement assumes >= 1dB, considering the required measurement accuracy and the likely separation between the aggressor UE and the victim UE. To L1 CLI measurement, which is targeted for network scheduling to avoid the interference at a small or medium timescale, it can be assumed that there is a need for the same or better accuracy. Since the number of measurement samples is likely to be reduced from 3 samples used in L3 measurement, we propose to a higher SRS Es/Iot, pending the agreement on the number of measurement samples for L1 measurement.

Proposal 3: To ensure same or better measurement accuracy for L1 measurement compared to L3 measurement, RAN4 can consider a higher SRS Es/Iot, pending the agreement on the number of measurement samples for L1 measurement.

On the SRS-RSRP range for reporting, there is not expected to be significant difference between L1 and L3 measurements. Therefore, it seems reasonable to continue using the R16 L3 range.

Proposal 4: From RSRP perspective, there is not expected to be significant difference in RSRP values from L1 measurements or L3 measurements. Therefore, on maximum/minimum SRS-RSRP, it is reasonable to adopt R16 L3 range.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: As there are no further conclusions from RAN1 on the time offset between DL timing and SRS arrival timing, it is reasonable to adopt the R16 CLI SRS RSRP assumption as the starting point.

Proposal 2: Regarding Option 2, remove the cell phase error from Option 1, it is better to clarify with RAN1 first if the L1 based UE-to-UE CLI measurement and reporting is limited to intra-cell case only. RAN4 can send an LS to clarify the applicable scenarios.

Proposal 3: To ensure same or better measurement accuracy for L1 measurement compared to L3 measurement, RAN4 can consider a higher SRS Es/Iot, pending the agreement on the number of measurement samples for L1 measurement.

Proposal 4: From RSRP perspective, there is not expected to be significant difference in RSRP values from L1 measurements or L3 measurements. Therefore, on maximum/minimum SRS-RSRP, it is reasonable to adopt R16 L3 range.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

RP-2413900, "WF on SBFD RRM," Huawei, HiSilicon