3GPP TSG-RAN WG4 Meeting #112bis                               						     R4-2415514

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Apple

Title: 	On reference architecture and RF capability for type 4 UE

Agenda Item:	6.5.2.1

Document for:	Discussion

# Introduction

Reference [1] was presented for RAN4#112 meeting, while some identified issues were not discussed online. One of them is the in-GAP interference which may be essential for type 4 UE reconfiguration to type 1 capability (8layer/CC in collocated condition) based on the agreed reference architecture for type 4 UE. Even though some companies don’t prefer such study for the current stage, there is indeed a need to clarify the implication to the reference architecture and agreed work scope for type 4 UE.

This contribution aims at some clarification on either the reference architecture or the work scope for type 4 UE which is being discussed under the open WI on support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s) [2].

# Discussion

The following reference architecture has been used as the baseline for type 4 UE capability and requirement development. From the table, it indicates that type 4 UE has 6/8 independent Rx chain from LNA to analog BB without further splitting after LNA for each Rx chain.

Table 2.1-1 reference architecture for type 4 UE

| UE type | CC# | Antenna/LNA |  | mixer | Analog BB | Frequency separation | NR CA/EN-DC | Power imbalance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4a | 1 | 4 | 6 total | 4 | 4 | No limitation or <X MHz | EN-DC | 25dB full range |
|  | 2 | 2 |  | 2 | 2 |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | No limitation or <X MHz | NR CA, EN-DC | 25dB full range |
|  | 2 | 4 |  | 4 | 4 |  |  |  |

In RAN4#110bis meeting, the following capability set was agreed [3] to be supported by type 4 UE based on the agreed reference architecture. It means type 4 UE needs to do hardware reconfiguration to support different capability set in collocated condition and non-collocated condition respectively. Given the agreed reference architecture, when collocated condition is met, the UE will fully share 6/8 Rx chains for both CCs to support non-contiguous CA between these 2 CCs.

Observation 1: Type 4 UE assumes fully shared Rx chain RF architecture between non-contiguous CCs when collocated condition is met.

While this seems not compliant with what is agreed for intra-band non-contiguous CA as indicated in TR 36.823. As shown in Section 6.2.3.1 (reproduced below), to support non-contiguous intra-band CA, separate Rx chain is needed for each CC. Only the first LNA is shared between CCs and the signal will be divided and handled by different Rx chain after the signal division component. The motivation on this reference architecture is that both non-contiguous carriers should satisfy the Rel-8 ACS requirements for the in-GAP frequency range.

Observation 2: Partially shared Rx RF architecture was assumed for non-contiguous intra-band CA as shown in Figure 6.2.3.1-1 in TR 36.823.

Back to type 4 UE discussion, fully shared architecture as indicated by observation 1 can only be workable when it is interference free at in-GAP frequency range. Otherwise, the Rx chain sharing will result performance degradation or even not workable depending on the interference level observed by the UE. Since the in-GAP interference is caused by another operator and not controllable, it is suggested RAN4 to study the following options a remedy solution for type 4 UE.

Option 1: Rediscuss the reference architecture for type 4 supporting both capability sets.

Option 2: Type 4 UE does not support reconfiguration to 8Rx type 1 capability.

Option 3:  Conduct in-GAP interference assessment and introduce assistant information to support type 4 UE reconfiguration.

Proposal: RAN4 discuss the identified issue for type 4 UE based on the following 3 options.

Option 1: Rediscuss and update the reference architecture for type 4 UE.

Option 2: Type 4 UE does not support reconfiguration to 8Rx type 1 capability.

Option 3:  Discuss necessary assistant information to support UE reconfiguration.

Observation 3: Same problem exists for type 2 UE given the assumption is the same between type 2 and type 4.

# Summary

This contribution presented our view on new identified issue related to type 4 UE reference architecture/capability. The following observation and proposal are concluded.

Observation 1: Type 4 UE assumes fully shared Rx chain RF architecture between non-contiguous CCs when collocated condition is met.

Observation 2: Partially shared Rx RF architecture was assumed for non-contiguous intra-band CA as shown in Figure 6.2.3.1-1 in TR 36.823.

Proposal: RAN4 discuss the identified issue for type 4 UE based on the following 3 options.

Option 1: Rediscuss and update the reference architecture for type 4 UE.

Option 2: Type 4 UE does not support reconfiguration to 8Rx type 1 capability.

Option 3:  Discuss necessary assistant information to support UE reconfiguration.

Observation 3: Same problem exists for type 2 UE given the assumption is the same between type 2 and type 4.

# References

[1] R4-2411415, On type 4 UE capability, RAN4#112, Apple

[2] RP-240838, New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI

[3] R4-2406627, WF on NonCol_intraB_ENDC_NR_CA, RAN4#110bis, KDDI