3GPP TSG-RAN WG4 Meeting #112bis                               						     R4-2415517

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Apple

Title: 	On type 4 UE capability

Agenda Item:	6.5.2.2

Document for:	Discussion

# Introduction

Type 4 UE capability and new BS signaling have been discussed in the past RAN4 meetings and the latest agreements were captured in the WF [1]. The following issues are remaining open for further study.

- Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA

- Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)

- Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)

- Issue 2-3-7:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b

- Issue 2-3-9: When to inform RAN2 the demand on new UE capability(s) and new BS signaling

This contribution will give further analysis on these open issues and presents our views.

# Discussion

## New UE capabilities for Type 4 UE

### Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

Way forward:

- Define one capability for NR CA and one UE capability for EN-DC

  - FFS on how to differentiate type 4a and 4b.

In the last meeting, the following agreement was agreed.

Type4a/4b differentiation is motivated by inter-band EN-DC band combination with overlapping DL frequency. 6Rx chain or 8Rx chain might be needed depending on whether 2Rx or 4Rx is supported in E-UTRA mode since 4Rx is not mandatory for E-UTRA in the concerning bands. So, for EN-DC type 4 UE, there are the following 2 options in terms of MIMO layer capabilities when non-collocated condition is met.

- max 2 MIMO layer in E-UTRA + max 4MIMO layer in NR (corresponding to type 4a UE)

- max 4 MIMO layer in E-UTRA + max 4MIMO layer in NR (corresponding to type 4b UE)

As long as the MIMO layer reporting for both RATs are on per CC basis, the differentiation of type 4a and type 4b can be realized by MIMO layer reporting. For NR, MIMO layer reporting is per CC based as specified in 38.306. For EN-DC the MIMO layer reporting scheme introduced in R15 is based on per CC reporting as shown below. While the MIMO layer is reported based on enumerated values and only twolayers, fourlayers and eightlayers are specified. Sixlayers needs to be included to support differentiation of type type 4a and type 4b.

With the above-mentioned signaling structure, we think it is sufficient to differentiate type 4a and type 4b for EN-DC UE. E.g.

- Type 4a EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 2 MIMO layer on E-UTRA carrier.

- Type 4b EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 4 MIMO layer on E-UTRA carrier..

Proposal 1: Differentiation of type 4a and type 4b is based on MIMO layer reporting for NR and E-UTRA.

  - Type 4a EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 2 MIMO layer on E-UTRA carrier.

  - Type 4b EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 4 MIMO layer on E-UTRA carrier.

Proposal 2: sixlayers needs to be included in E-UTRA MIMO layer reporting IE.

## New BS signalling supporting type 4 UE switching

In Rel-18, RAN2 introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Though not mentioned in the above Rel-18 BS signalling nonCollocatedTypeNR-CA for CA and nonCollocatedTypeMRDC for EN-DC, it is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4 for type 2 UE.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

When it comes to type 4 UE, we think a similar approach could be considered for network indication supporting UE switching between 6/8Rx type 1 and type 4a/4b.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 3: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

## UE behaviour between different capability set

For type 4 UE, there could be different working mode depending on different operating condition. The switch behaviour between non-collocated type 4 and other main modes is illustrated in figure 2.4-1.

Figure 2.3-1 illustration of UE behaviour and means to switch between different UE capabilities

It should be noted that,

- For non-collocated type 2, there could be further switching between collocated type 2 and collocate type 1 (4Rx). While these are already supported in Rel-18 and is omitted in the figure.

- For collocated type 1 with 4-layers/CC and 8-layers/CC, there will be further switching between them and other x-layers/CC. while it is just done by MIMO layer adaption as usual and is omitted in the figure.

Proposal 4: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.3-1.

## LS to RAN2

As for when to inform RAN2 the demand on new UE capability(s) and new BS signalling(s), our proposal is that LS shall not be sent to RAN2 until all the above issues are clear in RAN4.

Proposal 5: LS to RAN2 shall not be sent until all issues are settled in RAN4.

# Summary

This contribution presented our consideration on the open issues for type 4 UE capability issue. The following observations and proposals are concluded.

Proposal 1: Differentiation of type 4a and type 4b is based on MIMO layer reporting for NR and E-UTRA.

  - Type 4a EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 2 MIMO layer on E-UTRA carrier.

  - Type 4b EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 4 MIMO layer on E-UTRA carrier.

Proposal 2: sixlayers needs to be included in E-UTRA MIMO layer reporting IE.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

Proposal 3: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 4: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.3-1.

NOTE: some proposals may need to be revisited depending on the discussion in [2]

# References

[1] R4-2414453, WF on Rel-19 non-collocated scenario, KDDI, RAN4#112

[2] R4-2415514, On reference architecture and RF capability for type 4 UE, Apple, RAN4#112bis