3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415523

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.2.5

Source: 	Nokia

Title: 	Study of AAS performance in adjacent bands with multiple UE specific beams

Document for:	Information

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. One of the requests from ITU-R WP5D in [2] is:

d) IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations. WP 5D also seeks information about how the AAS performs in adjacent bands i.e. does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation?

The text proposal on revised simulation assumptions for the coexistence study was agreed in RAN4#112 [3]. The urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the agreed assumptions with 1 active UE per slot were provided in [4]. This contribution provides the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the agreed assumptions with 3 active UE per slot to study the AAS performance in adjacent bands.

Here, 3 UE specific beams are formed by the AAS as described in clause 5.3.2 of TR 37.842 [5], where the ACLR performance of each transmitter is used together with the composite radiation pattern of the 3 beams and the 3rd order intermodulation productions generated by the 3 beams (due to the non-linearization characteristics of the transmitters) to obtain the ACLR pattern (extracted from TR 37.842):

+

where is the ACLR performance of each transmitter in the AAS, and  is the weighting vector for the IMD3 products generated by the 3 beams:

=

The weighting factor above is for the IMD3 products  which is generated by+-, where , , and  represent any frequency component in beam p, q, l correspondingly.

Therefore, the ACLR(φ,θ) of the AAS depends on the ACLR performance of each transmitter, the composite radiation pattern of the 3 beams, as well as the composite radiation pattern of the IMD3 products generated by the 3 beams, and the ACLR(φ,θ) varies with the angles (φ,θ) within the coverage of the AAS.

It is assumed that the horizontal/vertical radiating sub-array spacing (0.5/2.8 of wavelength) and vertical element separation in sub-array (0.7 of wavelength) of 64x24=1536 (Sub Array size 4) AAS is designed for the 15 GHz carrier frequency, and the adjacent band is 400 MHz away at either side of the carrier. Hence the adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array of AAS that are used to form the composite array radiation pattern at the adjacent bands can be obtained as shown in Table 1 below.

| Adjacent Band | Center Frequency | Horizontal radiating sub-array spacing | Vertical radiating sub-array spacing | Vertical element separation in sub-array |
| --- | --- | --- | --- | --- |
| Lower | 14.6 GHz | 0.5*14.6/15=0.487 of wavelength | 2.8*14.6/15=2.725 of wavelength | 0.7*14.6/15=0.681 of wavelength |
| Upper | 15.4 GHz | 0.5*15.4/15=0.513 of wavelength | 2.8*15.4/15=2.875 of wavelength | 0.7*15.4/15=0.719 of wavelength |

Table 1: Adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array of 64x24=1536 (Sub Array size 4 elements) AAS

# 2.	Discussion

2.1	Downlink Results

The urban macro simulation results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45 dB BS ACLR and 33 dB UE ACS) using coordinated operation with FR2 like UE and un-coordinated operation with FR1 like UE are provided, respectively, in Tables 2 and 3 below. The simulation results are obtained at 15 GHz carrier frequency with 450 m inter-site distance, 20% indoor UE ratio, 400 MHz channel bandwidth and 3 active UE per slot. Case (a) are the results with the designed horizontal/vertical radiating sub-array spacing (0.5/2.8 of wavelength) and the vertical element separation in sub-array (0.7 of wavelength), while cases (b) and (c) are the results with the adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array in the lower and upper adjacent bands, respectively. Note with a 4 element sub-array the effective interelement vertical separation for the main array is 2.8λ.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss % (a) | 0.25 | 0.44 | 0.64 | 1.00 | 1.53 | 2.24 |
| 5%-tile throughput loss % (a) | 1.0954e-04 | 2.1856e-04 | 0.61 | 2.23 | 6.03 | 7.82 |
| Average throughput loss % (b) | 0.26 | 0.42 | 0.68 | 1.04 | 1.59 | 2.35 |
| 5%-tile throughput loss % (b) | 9.5585e-05 | 1.9072e-04 | 0.61 | 2.81 | 6.04 | 9.10 |
| Average throughput loss % (c) | 0.23 | 0.38 | 0.61 | 0.95 | 1.47 | 2.17 |
| 5%-tile throughput loss % (c) | 7.6069e-05 | 1.5178e-04 | 0.61 | 2.13 | 5.91 | 7.39 |

Table 2: Urban Macro DL throughput loss of victim FR2 like UE (coordinated operation)

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss % (a) | 0.31 | 0.50 | 0.80 | 1.26 | 1.96 | 2.96 |
| 5%-tile throughput loss % (a) | 1.1647e-02 | 2.3235e-02 | 1.16 | 1.16 | 1.17 | 3.42 |
| Average throughput loss % (b) | 0.30 | 0.49 | 0.79 | 1.26 | 1.96 | 2.96 |
| 5%-tile throughput loss % (b) | 9.9569e-03 | 1.9865e-02 | 1.16 | 1.16 | 1.20 | 3.42 |
| Average throughput loss % (c) | 0.30 | 0.49 | 0.79 | 1.25 | 1.93 | 2.91 |
| 5%-tile throughput loss % (c) | 1.2933e-02 | 2.5802e-02 | 0.51 | 1.16 | 1.17 | 3.36 |

Table 3: Urban Macro DL throughput loss of victim FR1 like UE (un-coordinated operation)

It can be seen from the tables that the average and 5%-tile throughput losses are similar for the three simulated cases. This can be explained by the similar composite array radiation patterns for the three simulated cases despite the adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array in the lower and upper adjacent bands. Figures 1 and 2 below show the elevation composite array pattern at the azimuth angle of 0° and the azimuth composite array pattern at the elevation angle of 96° (including a 6° mechanical downtilt), where the 3 users are at the azimuth angles of (-10°, 0°, 20°) and the elevation angle of 96°.

(a) 15 GHz					(b) 14.6 GHz					(c) 15.4 GHz

Figure 1: Elevation composite array pattern (0° azimuth angle)

(a) 15 GHz					(b) 14.6 GHz					(c) 15.4 GHz

Figure 2: Azimuth composite array pattern (96° elevation angle)

Figures 3 and 4 below show, respectively, the elevation ACLR pattern at the azimuth angle of 0° and the azimuth ACLR pattern at the elevation angle of 96° (including a 6° mechanical downtilt), where the 3 users are at the azimuth angles of (-10°, 0°, 20°) and the elevation angle of 96°. Note that the ACLR patterns only notably vary in the azimuth plane because the 3 beams are formed at the same elevation angle. The minor differences in a few of the 5%-tile throughput losses with larger ACIR offsets can be explained by the differences in the ACLR patterns for the three simulated cases, because the 5%-tile victim UE at the cell edge have lower SNR and thus are more affected by the ACI from the interfering BS.

(a) 15 GHz					(b) 14.6 GHz					(c) 15.4 GHz

Figure 3: Elevation ACLR pattern (0° azimuth angle)

(a) 15 GHz					(b) 14.6 GHz					(c) 15.4 GHz

Figure 4: Azimuth ACLR pattern (96° elevation angle)

It can be concluded that the composite array radiation patterns experience slight changes while the ACLR patterns experience some changes in the adjacent bands, using the current AAS model with correlation of 2 transceiver paths (ρ) equals to 1 when 3 UE specific beams are formed by the AAS.

2.2	Uplink Results

The urban macro simulation results of the uplink throughput loss of the victim UE with different ACLR offsets (compared to the 30 dB UE ACLR) using coordinated operation with FR2 like UE and un-coordinated operation with FR1 like UE are provided, respectively, in Tables 4 and 5 below. The simulation results are obtained using the same parameters as those used for the downlink throughput loss.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss % (a) | 0.14 | 0.19 | 0.28 | 0.41 | 0.59 | 0.87 |
| 5%-tile throughput loss % (a) | 5.4970e-02 | 8.8484e-02 | 1.08 | 1.08 | 2.02 | 3.47 |
| Average throughput loss % (b) | 0.15 | 0.21 | 0.31 | 0.45 | 0.65 | 0.95 |
| 5%-tile throughput loss % (b) | 6.8095e-02 | 0.11 | 1.09 | 1.09 | 2.99 | 4.83 |
| Average throughput loss % (c) | 0.13 | 0.18 | 0.26 | 0.38 | 0.55 | 0.80 |
| 5%-tile throughput loss % (c) | 4.0831e-02 | 6.5731e-02 | 1.07 | 1.08 | 1.99 | 3.44 |

Table 4: Urban Macro UL throughput loss of victim FR2 like UE (coordinated operation)

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss % (a) | 0.20 | 0.29 | 0.43 | 0.64 | 0.97 | 1.47 |
| 5%-tile throughput loss % (a) | 0.83 | 0.87 | 4.03 | 5.02 | 7.01 | 8.80 |
| Average throughput loss % (b) | 0.19 | 0.28 | 0.42 | 0.63 | 0.97 | 1.47 |
| 5%-tile throughput loss % (b) | 0.84 | 0.85 | 4.57 | 5.49 | 7.61 | 9.57 |
| Average throughput loss % (c) | 0.20 | 0.28 | 0.42 | 0.63 | 0.95 | 1.44 |
| 5%-tile throughput loss % (c) | 0.84 | 0.85 | 3.52 | 3.74 | 5.76 | 9.71 |

Table 5: Urban Macro UL throughput loss of victim FR1 like UE (un-coordinated operation)

Again, it can be seen from the tables that the average and 5%-tile throughput losses are similar for the three simulated cases.

# 3.	Conclusion

This contribution has provided provides the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the agreed assumptions with 3 active UE per slot to study the degradation of the AAS performance in adjacent bands taking into consideration the 3rd order intermodulation productions generated by the 3 beams due to the non-linearization characteristics of the transmitters.

Observations from simulation results:

1) 	With 400 MHz channel bandwidth at the 15 GHz carrier frequency, the composite array radiation patterns formed by the AAS in the adjacent bands are similar to that formed at the designed carrier frequency, while the ACLR patterns experience some changes in the adjacent bands, using the current AAS model with correlation of 2 transceiver paths (ρ) equals to 1 when 3 UE specific beams are formed by the AAS.

2)	The average and 5%-tile downlink and uplink throughput losses are similar for the three simulated cases.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP5D.

[3]	R4-2414303, “TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range”, Nokia, Fujitsu.

[4]	R4-2412592, “Study of AAS performance in adjacent bands”, Nokia.

[5]	3GPP TR 37.842 v13.3.0, “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.