3GPP RAN WG4 Meeting #112bis																R4-2415526

Hefei, China, 14th – 18th October 2024

Agenda item:	6.1.1.2.2

Source:	Apple

Title:	On Rel-19 UL CA power domain enhancement for FR1

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

Power domain enhancement is a re-occurring topic and enters the 4th phase when starting the Rel-19 work. The objectives are covered in the WID [1]. This contribution focuses on the high-level aspects of MPR reduction. The WID lists those goals as:

# 2  Discussion

##### Applicable MPR for intra-band contiguous CA with single activated cell

The handling of intra-band carrier aggregation with single activated cell remains an open item. RAN4 discussed several solutions in last meeting and decided to evaluate further discuss appropriate solution in following meetings. The WF [2] captures the state and proposals as follows:

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 #### Issue 2-2-2: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei) - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks) - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson) - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson)

Comparing the three proposals it seems agreeable that improved requirements shall be defined if only a single cell is active in case of intra-band contiguous CA. The performance considerably depends on the RF hardware implementation choice. The most critical architecture consideration is whether the UE supports a single LO for both CCs or whether two separate LOs are deployed for each CC. In the first case the LO will likely be somewhere in the centre of the aggregated bandwidth from the two CCs. To enable improvements the UE needs to re-configure its LO. Otherwise, strong IMD products would fall into frequency regions with tight emission limits. For this architecture RAN4 might need to discuss impact on time masks and align on the question whether transient periods are needed for potential LO switching occurrences. This fundamental discussion might also be needed for non-contiguous carrier aggregation. The second case might be simpler in this regard. The UE can shut-off or supress the unnecessary LO if transmission is not scheduled. In the end both architectures could support smaller MPR for single CC case.

The captured proposal 2 and proposal 3 seem very similar with their main difference being the applicability of Tx power requirements from clause 6.2.4 of TS 38.101-1. If a UE is configured with intra-band non-contiguous or contiguous carrier aggregation those UL CA requirements need to be applied if bandwidth class B and C are used in the UL. In case of proposal 3, if the UE is only scheduled with RB allocations in a single CC while the second CC is empty, then the UE would be required to apply single Tx carrier requirements. If the network updates the uplink configuration with RB allocations occupying both CCs then the UE would need to discard the single Tx carrier requirements and configure the UL CA requirements. This leads to some significant updates of requirements for a simple RB allocation change. It would be much easier to keep the UL CA requirements and simply apply single CC MPR table. This seems to be a smaller change as only the single CC MPR table needs to be substituted with UL CA table. Additionally, the clause 6.2A.4.1.1 refers to clause 6.2.4 for the individual serving cell c. The difference to purely applying 6.2.4 is that total Pcmax is bounded by PPowerClass,CA and not by PPowerClass. This might make a difference for PHR if the CA is PC3,  but the UE advertises the single band as PC2. Further discussion would be needed to check on all those issues.

Observation:  The performance considerably depends on the RF hardware implementation choice. The most critical aspect is whether the UE supports a single LO for both CCs or whether two separate LOs are deployed for each CC. In the first case the LO will likely be somewhere in the centre of the aggregated bandwidth from the two CCs. To enable improvements the UE needs to re-configure its LO. Otherwise, strong IMD products would fall into frequency regions with tight emission limits. For this architecture RAN4 might need to discuss impact on time masks and align on the question whether transient periods are needed for potential LO switching occurrences.

Proposal 1: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation.

Proposal 2: Consider proposal 2 from Issue 2-1-1 for further discussion as it seems to be the simpler option due to less requirement switching when the network changes CC activation.

# 4	Conclusions

This contribution provides certain considerations to the discussion on the MPR reduction which his part of the power domain enhancement WID. The following observations are made:

Observation:  The performance considerably depends on the RF hardware implementation choice. The most critical aspect is whether the UE supports a single LO for both CCs or whether two separate LOs are deployed for each CC. In the first case the LO will likely be somewhere in the centre of the aggregated bandwidth from the two CCs. To enable improvements the UE needs to re-configure its LO. Otherwise, strong IMD products would fall into frequency regions with tight emission limits. For this architecture RAN4 might need to discuss impact on time masks and align on the question whether transient periods are needed for potential LO switching occurrences.

Proposal 1: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation.

Proposal 2: Consider proposal 2 from Issue 2-1-1 for further discussion as it seems to be the simpler option due to less requirement switching when the network changes CC activation.

# 4	References

- RP-240828, “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, 3GPP TSG RAN Meeting #103

- R4-2410749, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111

- R4-2411153, “On Rel-19 power domain enhancement”, Apple, Maastricht, Netherlands, August 19th – 23rd, 2024