3GPP TSG-RAN4 Meeting #112bis	R4-2415529

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	CW cancellation feasibility

Source:	Qualcomm Incorporated

Agenda item:	6.21.3.4

Release:	Rel-19

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

CW cancellation was discussed in RAN#105 [14] and conclusion was:

Conclusion 6

RAN guidance to RAN4:

  - RAN confirms that RAN4 studies and documents the methods and feasible values of CW interference cancellation according to their agreements.

In this paper we continue our discussion on CW cancellation from [13] with some practical boundaries for Intermediate UE and gNB and discuss what impact the cancellation has for the system.

# 2. 	Discussion

As discussed in [13], there CW can be cancelled in analogue domain and in digital domain. The co-ex study sees CW cancellation as one number but in the following discussion we continue what boundaries analogue domain CW cancellation set and what digital domain defines. We split the discussion in to four parts, Intermediate node and gNB as readers and CW from the reader and CW from separate node.

## 2.1	Intermediate node and CW from the reader

In the first phase, to make the analysis a bit more concrete, we can assume that current handheld UE requirements are taken as baseline. Here we consider the maximum input level of -25 dBm and further assume that digital path dynamic range is defined by SNR for 256 QAM and ACS resulting in to 68 dB (12 bits) but in practice some more dynamic range is needed to accommodate some range for the digital AGC in practice. The output power is the one used in co-ex work, 23 dBm, and antenna coupling is 10 dB thus same what is used in [13] and in all other work such as MSD work in RAN4 for handhelds. Additionally, we can assume that the needed D2R signal SNR for successful decoding is 10 dB.

Table 1. Assumptions for the CW cancellation for intermediate node

| Parameter | Value | Note |
| --- | --- | --- |
| Max input level | -25 dBm |  |
| Output power of CW | 23 dBm |  |
| Antenna coupling | -10 dB | Also reflection loss |
| Digital path dynamic range | 12 bit (14 bit) | 68 dB (74 dB) |
| SN(I)R for D2R signal | 10 dB | At receiver |

With the above numbers, we can calculate that the maximum CW cancellation in digital domain is 68 – 10 = 58 dB. This will depend on number of bits used in the ACD, and for effective 14 bit ADC it would be 74 dB.

Reflected power from the antenna is 10 dB lower than the transmitted power, this means LNA without analogue domain CW cancellation will see 13 dBm CW but since the maximum input level is -25 dBm, the analogue domain cancellation has to be 38 dB for 23 dB output power and to ensure LNA max input level is not exceeded. Even if handheld device uses different antennas, the coupling can be 10 dB and the result is the same conclusion.

Observation 1: Analogue domain CW cancellation must be 38 dB for 23 dBm output power and -25 dBm maximum input level for the receiver.

The analysis in [13] also showed that this is feasible from the control point of view. With 38 dB in analogue domain, the total CW cancellation maximum is 112 dB.

Observation 2: The total feasible CW cancellation capability in Intermediate node UE is no more than 112 dB.

Looking more closely, the range of the reader can be calculated from the above numbers following the logic:

Received D2R power = Max CW power – 2x pathloss – backscattering loss

Minimum received power at lowest SNR = max input level + SNRD2R – DNRADC

When me make both equations equal:

Max CW power – 2x pathloss – backscattering loss >= max input level + SNRD2R – DNRADC

Pathloss =<  (-max input level - SNRD2R + DNRADC + Max CW power - backscattering loss)/2

In numbers, assuming 10 dB backscattering loss:

Pathloss =< (-(-25 dBm) – 10 dB + 84 dB + 23 dBm - 10 dB)/2 = 56 dB, which is about 14 meters at 1 GHz with 14 bit ADC.

And this is assuming the analogue domain CW cancellation is as above.

Observation 3: Intermediate reader range is 15 meters assuming feasible CW cancellation performance.

Proposal 1: RAN4 to consider no more than 15 meters are Intermediate node reader range limited by CW cancellation capability

To understand the CW cancellation role to the range, we can first look at the analogue domain cancellation. If the cancellation is less than 38 dB but the maximum input level can not be exceeded, the output power has to be reduced  and this reduces the range accordingly. If the CW cancellation is better than the 38 dB, the output power can be increased and this increases the range.

Observation 4: Analogue domain CW cancellation defines the range of the reader when the digital domain dynamic range (number of bits) is fixed

Looking at the digital domain cancellation, firstly we state that the digital domain can cancellation can be very large if a long training period is allowed. When intermediate node is the reader, it can choose when to turn CW or and train its own receiver thus time can be long.

For a fixed output power, the digital domain dynamic range defines the range since more bits there are available, more bits can be used for the CW cancellation with 2 effective bits needed to cover 10 dB SNR needed for the successful decoding, only the training time may need to be increased. However, this analysis did not look in to the noise contribution. With the above values, the received D2R signal power is close to -100 dBm. Without knowing what is the bandwidth of the D2R signal, it is difficult to say exactly what is the noise contribution but at -100 dBm level and e.g. 200 kHz BW when kTB = -120 dBm, there will be some noise contribution to the SINR.

Observation 5: For fixed CW output power and receiver maximum input level, increasing digital domain dynamic range increases the reader range until noise contribution limits the sensitivity.

## 2.2	gNB and CW from the reader

As discussed in [13], gNB has additional freedom in the analogue domain at its disposal by means of antenna isolation since it can use spatially separated and directive antennas. gNB also uses higher bit width digital domain that enables larger dynamic range since it can handle better ACS. Receiver is able to handle larger input levels even it is not clear from 3GPP requirements. gNB also needs this additional performance since the output power of the CW is higher.

Observation 6: For feasibility analysis, gNB CW cancellation capability exceeds that of the Intermediate node

## 2.3	Intermediate node and “CW from outside of the topology”

### 2.3.1	CW cancellation

Case is different when CW comes from outside the topology.  The level of the CW at the receiver is lower compared to the D2R signal level but the CW phase noise plays a role in the range since the down conversion will be done with a different signal source with non-correlating phase noise.

Figure 1. CW not in the reader “CW outside topology”

The way the CW nodes are distributed in the aged topologies result almost constant -39 dBm received CW in the reader. The assumption is to always use closest CW source. Looking at the statistical D2R received power distribution in Figure 2, for 90% of the connections see power higher than -101 dBm for D1T1 and -98 dBm for D2T2.

Figure 2. D2R received power distribution in the two topologies. CW level is almost constant at the receiver

Calculating the ratios:

Observation 7: CW to D2R signal levels for 90 % probability are 62 and 62 dBc in D1T1 and D2T2 when CW comes outside the topology.

In this topology, it is difficult to relate the CW cancellation and range since it depends on how the CW source and reader are positioned. From statistical data in Figure we can see that the cancelation needs to be much lower level than when CW source is in the reader and  up to the point that maybe analogue domain cancellation is not needed.

Observation 8: CW outside topology results in to much lower need for CW cancellation in the reader.

### 2.3.2	Phase noise impact

However, since reader will be using a different CW to down convert the D2R signal than the CW used in the backscattering, the phase noise of the incoming CW plays a critical role in system performance.

From the same conclusion, that the CW to D2R power ratio at the reader is approximately 62 dBc, we can conclude that the integrated phase noise over the D2R bandwidth can not be bigger than -72 dBc, to allow for 10 dB SNIR. However, this assumes phase noise is the only noise or interference source and more comfortable specification could be -80 dBc below the CW.

Observation 9: Integrated phase noise for the CW coming outside the topology over the D2R signal BW (Btx,D2R  ) must be better than -80 dBc

Typically the PLLs and oscillators are designed to meet wide BWs and in handheld device also settling time needs to be accounted for and they may not be optimised for close in noise where backscattering signals in A-IoT case are assumed to be positioned with relatively narrow bandwidth.

In practical design, the phase noise is a result of many components, such as VCO, PLL loop and 1/f noise. In Figure 3 from [15] these are summarize and the resulting phase noise is the darker red curve.

Figure 3. Example of a phase noise characterises and contributing sources [15]

Observation 10: 80 dBc integrated phase noise specification may require dedicated high-performance design.

RAN4 then should specify this for the component which provides the phase noise to the system but this issue should be firstly understood and documented in the study item and then what kind of specification will be written then discussed in work item.

Understanding phase noise of the CW source may also be important for the radio system design. RFID uses an offset from CW in the D2R link and this helps with the phase noise since as we can observe from Figure 3, offsetting the placement of Btx,D2R  further away might be useful in relaxing the CW source phase noise requirements.

Observation 11: Placing Btx,D2R  further away from CW in frequency is beneficial for the system throughput and range.

More pressing issue is how co-existence feasibility will address this issue since with wrong assumption, CW source phase noise maybe the largest interfering component.

Proposal 2: RAN4 to account for phase noise of the CW source when it is coming from outside the topology in the feasibility analysis.

## 2.4	gNB and “CW from outside of the topology”

The case is no different when gNB is the reader since phase pollutes the signal BW receiver can not recover the signal. CW cancellation does not cancel the phase noise since it is a result of a random process in the CW source.

Observation 12: gNB as reader has same problem as intermediate node for CW phase noise

# Conclusion

We discussed CW cancellation and made the following observation:

Observation 1: Analogue domain CW cancellation must be 38 dB for 23 dBm output power and -25 dBm maximum input level for the receiver.

Observation 2: The total feasible CW cancellation capability in Intermediate node UE is no more than 112 dB.

Observation 3: Intermediate reader range is 15 meters assuming feasible CW cancellation performance.

Observation 4: Analogue domain CW cancellation defines the range of the reader when the digital domain dynamic range (number of bits) is fixed

Observation 5: For fixed CW output power and receiver maximum input level, increasing digital domain dynamic range increases the reader range until noise contribution limits the sensitivity.

Observation 6: For feasibility analysis, gNB CW cancellation capability exceeds that of the Intermediate node

Observation 7: CW to D2R signal levels for 90 % probability are 62 and 62 dBc in D1T1 and D2T2 when CW comes outside the topology.

Observation 8: CW outside topology results in to much lower need for CW cancellation in the reader.

Observation 9: Integrated phase noise for the CW coming outside the topology over the D2R signal BW (Btx,D2R  ) must be better than -80 dBc

Observation 10: 80 dBc integrated phase noise specification may require dedicated high-performance design.

Observation 11: Placing Btx,D2R  further away from CW in frequency is beneficial for the system throughput and range.

Observation 12: gNB as reader has same problem as intermediate node for CW phase noise

And based on observations, we proposed:

Proposal 1: RAN4 to consider no more than 15 meters are Intermediate node reader range limited by CW cancellation capability

Proposal 2: RAN4 to account for phase noise of the CW source when it is coming from outside the topology in the feasibility analysis.

# Reference

[1]	RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

[2]	R1-2403195, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403195.zip

[3] 	R1-2403194, “Evaluation Assumptions and Results”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403194.zip

[4]	A Low-Power Backscatter Modulation System Communicating Across Tens of Meters With Standards-Compliant Wi-Fi Transceivers

[5]	ETSI EN 302 208, “Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum”, ETSI

[6]	R4-2406714, “WF on Ambient IoT in NR”, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, 15th – 19th April, 2024

[7]	R4-2410597, “WF on impacts of A-IoT on RF requirements​”, Huawei, 3GPP TSG-RAN WG4 Meeting # 111, Fukuoka, Japan, May 20 – May 24, 2024

[8]	R4-2407588, “A-IoT device study and RF requirements aspects”, Qualcomm, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[10]	R4-2410567, “WF on co-existence study for ambient IoT and NR/LTE”, CMCC, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[11]	R4-2410597, “WF on impacts of A-IoT on RF requirements”, Huawei, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

[12]	R4-1706112, “Reply LS regarding 1024QAM”, RAN1(Huawei), 3GPP TSG-RAN WG4 Meeting #83, Hangzhou, China, 15 - 19 May, 2017

[13]	R4-2413322, ”CW cancellation capability”, Qualcomm, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, 19th – 23rd August, 2024

[14]	RP-242360, “Moderator's summary for offline discussion on Ambient IoT”, RAN1 vice-chair (Huawei), 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

[15]	https://www.analog.com/en/resources/analog-dialogue/articles/phase-locked-loop-pll-fundamentals.html