3GPP TSG-RAN4 Meeting #112bis	R4-2415530

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Energy harvesting considerations for A-IoT device

Source:	Qualcomm Incorporated

Agenda item:	6.21.3.2

Release:	Rel-19

SI: 	FS_Ambient_IoT_solutions

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

In RAN4#112 agreement was made in [11] that OTA is baseline testing method for devices 1 and 2a. This paper continues to discuss energy harvesting parameters in an effort to unify RAN4 understanding on feasible conditions where Ambient IoT UE operates to enable fluent WI phase.

# 2. 	Discussion

## 2.1 	Motivation for the studying energy harvesting

In order to test an Ambient IoT device, it will need to be provided with energy. Similarly, in order for device to operate, it needs to harvest energy. RAN1 in their analysis for example for inventory completion time assume that devices wake up after certain period of energy harvesting. If RAN4 does not have good understand how much and what kind of energy signal needs to be provide to the device, then every device manufacturer will implement what they feel sufficient and it can not be guaranteed that different manufacturer devices can operate with different readers. The purpose of a standard work is to ensure that all devices can work in the system and especially RAN4 specifications define what are the minimum requirements that a device needs to fulfil in order to be accepted as part of the system.

Observation 1: Energy harvesting capability of Ambient IoT device is essential part of the system

The agreements made in plenary were that “The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study” and despite RAN4 was not explicitly mentioned but it does not mean RAN4 can not study it. The RAN4 agreement in [11] was that device will be tested over the air as proposed in [10]. RAN4 can ignore this aspect and perhaps even leave all energy transfer aspects undefined but that then results in to a poor standard and it may be difficult to conclude the work, or even start the work. Our view is that without standardizing device’s energy harvesting criteria, the system can not be specified to be functional.

For that reason, RAN4 needs to specify how much energy needs to be provided to the device and how long it is allowed to harvest before it can be assumed to be in on state so that any test is possible. It is early in a study iten to discuss about testing but to ensure that RAN4 has a common understand we states that a device needs to be turned on in order to be able to test it. If testing is not possible, it is pointless to discuss any requirements.

Observation 2: Without studying and specifying energy harvesting boundary conditions in Study Item, it maybe difficult to conclude any testable requirements in WI phase.

The WF [11] list number of specification items that are needed and some that are not needed. This is something that could be determined after all the studies have been concluded. The time and effort could be better used in studying the two truly new RF aspects in A IoT which are CW cancellation and energy harvesting.

Proposal 1: RAN4 to focus on Energy harvesting in the study item and define a list of testable requirements in the WI phase when system design is more mature in RAN1

## 2.2	Principle of device behaviour under Energy harvesting

RAN1 is discussing states for the devices and device is assumed to have at least ON and OFF states and maybe also SLEEP. Device goes from OFF state to ON state when it detects energy and has been able to accumulate sufficient amount of energy for the parts to function. We can use a tentative value of [-35] dBm for the energy harvesting sensitivity. In previous paper [8] we discussed the importance of clear reference plane. It is our understanding that RAN4 task will be to define the energy harvesting sensitivity with proper accuracy and needed reference plane definitions as we discussed in [8]. The value may not be needed during the study item for the detailed value but some aspects how this new type of parameter should be studied.

This Energy Harvesting Sensitivity (EHS) is the value when device starts to harvest energy. As such it may not be useful alone but we will discuss more details on that in the next section. Device behaviour principle with non-constant energy availability is shown in Figure 1.

Figure 1. Energy harvesting principle

Energizing signal is therefore trigger for device to turn on. This is specifically for device type 1. After device turns on, it needs to monitor the WUS and then after successfully receiving signal to transmit, it needs to transmit back what was requested. Device may also decide to sleep or continue monitoring. RAN1 is discussing details the device states behaviour [9] and once there is sufficient information about the expected device behaviour and device states from their studies RAN4 can extract minimum performance requirements for the device. Expectation is that there will be some minimum device operation pattern such as one shown in Figure 2 frow which RAN4 can deduct device ON-OFF dutycycle.

Figure 2. Energy harvesting frame structure

In simple form, this translates to some ON-OFF dutycycle. We will discuss energy harvesting parameters in more detail in next section.

## 2.3	Energy harvesting parameters

Energy will be captured at first hand by the device antenna with the antenna in the device and for study purposes, RAN4 could assume 0 dBi antenna.

Proposal 2: For energy harvesting, assume 0 dBi antenna

When incoming RF signal level exceeds EHS, device starts harvesting energy. Energy harvesting in the device is not perfect but happens with some efficiency < 100% and the efficiency is proportional to the incoming signal level. The accumulated energy can be calculated as a function of incoming signal strength, time and efficiency.

Figure 3. Accumulated energy

Simplifying the frame in Figure 2 so that the ON time is 2 msec and considering that device power consumption is always 1uW when ON the energy consumed is 5 nWs. Time to harvest 5 nWs is 21 msec with -29 dBm received signal strength and 18% harvester efficiency.

Proposal 3: Device wakeup time is 40 msec starting from the beginning of availability of a -25 dBm energizing signal.

# Conclusion

We discussed device energy harvesting and made two observations:

Observation 1: Energy harvesting capability of Ambient IoT device is essential part of the system

Observation 2: Without studying and specifying energy harvesting boundary conditions in Study Item, it maybe difficult to conclude any testable requirements in WI phase.

And made the following proposals:

Proposal 1: RAN4 to focus on Energy harvestign in the study item and define a list of testable requirements in the WI phase when system design is more mature in RAN1

Proposal 2: For energy harvesting, assume 0 dBi antenna

Proposal 3: Device wakeup time is 30 msec starting from the beginning of availability of a -30 dBm energizing signal.

# Reference

[1]	RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

[2]	R1-2403195, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403195.zip

[3] 	R1-2403194, “Evaluation Assumptions and Results”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #116-bis, Changsha, Hunan Province, China, April 15th – 19th, 2024, https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_116b/Docs/R1-2403194.zip

[4]	A Low-Power Backscatter Modulation System Communicating Across Tens of Meters With Standards-Compliant Wi-Fi Transceivers

[5]	ETSI EN 302 208, “Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum”, ETSI

[6]	R4-2406714, “WF on Ambient IoT in NR”, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, 15th – 19th April, 2024

[7]	R4-2410597, “WF on impacts of A-IoT on RF requirements​”, Huawei, 3GPP TSG-RAN WG4 Meeting # 111, Fukuoka, Japan, May 20 – May 24, 2024

[8]	R4-2407588, “A-IoT device study and RF requirements aspects”, Qualcomm, 3GPP TSG-RAN4 Meeting #111, Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

[9]	R1-2407032, “Ambient IoT Device Architecture”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19th – 23rd, 2024

[10]	R4-2411537, “Further considerations on the ambient IoT device implementation and RF aspect”, Sony, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, 19th – 23rd August, 2024

[11]	R4-2414305, “WF on impacts of A-IoT on RF requirements”, Moderator (Huawei), 3GPP TSG-RAN WG4 Meeting # 112, Maastricht, Netherlands,19 – 23 Aug, 2024