3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2415533

Hefei, China, 14 – 18 October 2024

Title: 	On in-gap and adjacent blockers level for fragmented spectrum reception in a single Rx path

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.6.3	Impacts on UE RF requirements and DL performance [FS_NR_FR1_DL_Frag_Carrier]

Document for:	Approval

# 1	Background

In RAN#112, we discussed the potential issues related to receiving non-contiguous DL CCs using a shared Rx path in [1], and way forward [2], captured most of the architecture and performance issues to be discussed. In this contribution, we discuss various architectural aspects, but also address one of the most critical aspects of this approach: the blocking and ACS performance that can be expected in both adjacent and in-gap regions.

# Discussion

## 2.1 Way forward and Revised SID inputs

The way forward [2] captured a number of agreements on the scope shown in italic below.

Online agreements: Issue 2-1-1: Applicability and clarification on the scope

The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

- The two non-contiguous CCs is still under the current DL non-contiguous CA framework

  - Each fragment is treated as an individual CC

- The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

- [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands: Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

    - Provide the details related to gap between fragmented DL blocks]

Observation on candidate bands:

- Since the scope is for bands that have fragments confined within 100MHz, it is useful to consider bands which bandwidth is equal to, or less than 100MHz.

- On the other end, to have at least two non-contiguous CCs, the band must be at least 15MHz large if 5MHz is considered as the lowest channel bandwidth. These bands are summarized in Table 1.

- If there are other bands, such as Bands n41, n77/78, n79 and n104 where a 100MHz could be used for fragmented spectrum, These have large CBW that would not allow placement of two non-contiguous CCs. There may be an exception for n41 in some regions.

- It should be noted that all the bands are below 2.7GHz. Thus the related ACS and blocking requirements can be used as a reference.

- Most of the bands are FDD or SDL bands, except some that are primarily Chinese TDD bands.

- Even if 3MHz is specified in some of these bands, this is an optional CBW that is not used for NC DLCA yet.

- Table 1 clarifies if the band already supports NC DLCA or not. Note that NC ULCA is not specified for n39 thus it does not look like a good example band.

Table 1: Bands with DL CBW ≥ 15MHz and ≤ 100MHz

| NR Band | Mode | UL Low | UL High | DL Low | DL High | ULBW | DLBW | Min CBW | Max CBW | maxCBW NCCA | Intra NC ULCA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | FDD | 1920.0 | 1980.0 | 2110.0 | 2170.0 | 60 | 60 | 5 | 50 | 50 | y |
| n2 | FDD | 1850.0 | 1910.0 | 1930.0 | 1990.0 | 60 | 60 | 5 | 40 | 40 | y |
| n3 | FDD | 1710.0 | 1785.0 | 1805.0 | 1880.0 | 75 | 75 | 5 | 45 | 45 | y |
| n5 | FDD | 824.0 | 849.0 | 869.0 | 894.0 | 25 | 25 | 5 | 25 | 15 | y |
| n7 | FDD | 2500.0 | 2570.0 | 2620.0 | 2690.0 | 70 | 70 | 5 | 50 | 50 | y |
| n8 | FDD | 880.0 | 915.0 | 925.0 | 960.0 | 35 | 35 | 5 | 30 | 25 | n |
| n12 | FDD | 699.0 | 716.0 | 729.0 | 746.0 | 17 | 17 | 5 | 15 | 5 | y |
| n18 | FDD | 815.0 | 830.0 | 860.0 | 875.0 | 15 | 15 | 5 | 15 | 5 | n |
| n20 | FDD | 832.0 | 862.0 | 791.0 | 821.0 | 30 | 30 | 5 | 20 | 20 | n |
| n24 | FDD | 1626.5 | 1660.5 | 1525.0 | 1559.0 | 34 | 34 | 5 | 10 | 10 | n |
| n25 | FDD | 1850.0 | 1915.0 | 1930.0 | 1995.0 | 65 | 65 | 5 | 45 | 45 | y |
| n26 | FDD | 814.0 | 849.0 | 859.0 | 894.0 | 35 | 35 | 5 | 20 | 20 | y |
| n28 | FDD | 703.0 | 748.0 | 758.0 | 803.0 | 45 | 45 | 5 | 40 | 30 | n |
| n34 | TDD | 2010.0 | 2025.0 | 2010.0 | 2025.0 | 15 | 15 | 5 | 15 | 5 | n |
| n38 | TDD | 2570.0 | 2620.0 | 2570.0 | 2620.0 | 50 | 50 | 5 | 40 | 40 | n |
| n39 | TDD | 1880.0 | 1920.0 | 1880.0 | 1920.0 | 40 | 40 | 5 | 40 | 30 | n |
| n40 | TDD | 2300.0 | 2400.0 | 2300.0 | 2400.0 | 100 | 100 | 5 | 100 | 90 | y |
| n50 | TDD | 1432.0 | 1517.0 | 1432.0 | 1517.0 | 85 | 85 | 5 | 80 | 60 | n |
| n65 | FDD | 1920.0 | 2010.0 | 2110.0 | 2200.0 | 90 | 90 | 5 | 50 | 50 | n |
| n66 | FDD | 1710.0 | 1780.0 | 2110.0 | 2200.0 | 70 | 90 | 5 | 45 | 45 | y |
| n67 | SDL |  |  | 738.0 | 758.0 |  | 20 | 5 | 20 | 10 | n |
| n70 | FDD | 1695.0 | 1710.0 | 1995.0 | 2020.0 | 15 | 25 | 5 | 25 | 15 | n |
| n71 | FDD | 663.0 | 698.0 | 617.0 | 652.0 | 35 | 35 | 5 | 35 | 25 | y |
| n74 | FDD | 1427.0 | 1470.0 | 1475.0 | 1518.0 | 43 | 43 | 5 | 20 | 20 | n |
| n75 | SDL |  |  | 1432.0 | 1517.0 | 0 | 85 | 5 | 50 | 50 | n |
| n85 | FDD | 698.0 | 716.0 | 728.0 | 746.0 | 18 | 18 | 5 | 15 | 10 | n |
| n92 | FD+ | 832.0 | 862.0 | 1432.0 | 1517.0 | 30 | 85 | 5 | 20 | 20 | n |
| n94 | FD+ | 880.0 | 915.0 | 1432.0 | 1517.0 | 35 | 85 | 5 | 20 | 20 | n |
| n105 | FDD | 663.0 | 703.0 | 612.0 | 652.0 | 40 | 40 | 5 | 35 | 30 | n |
| n109 | FD+ | 703.0 | 748.0 | 1432.0 | 1517.0 | 45 | 85 | 5 | 50 | 50 | n |
| n254 | FDD | 1610.0 | 1626.5 | 2483.5 | 2500.0 | 16.5 | 16.5 | 5 | 15 | 5 | n |
| n255 | FDD | 1626.5 | 1660.5 | 1525.0 | 1559.0 | 34 | 34 | 5 | 10 | 10 | n |
| n256 | FDD | 1980.0 | 2010.0 | 2170.0 | 2200.0 | 30 | 30 | 5 | 20 | 20 | n |

Proposal on examples for the study:

- Bands that are <2.7GHz are used as examples and ACS and blocking for <2.7GHz is used for reference.

- For non-contiguous CCs, 5MHz is used as minimum CBW and maximum CBW is 90MHz.

- The example band(s) should already have NC ULCA specified.

Recommended WF: Issue 3-1-4: Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements: Proposal 1: put off the signalling related discussions until the UE performance aspects are clear.

The way forward [2] also captured a number of issues to be clarified and tackled during the study.

Issue 2-1-2: Reference architecture, Recommended WF:

•	The existing requirements for DL NCCA was specified based on partially-shared Rx architectureaccording to TR36.823.

•	FFS on following options for reducing number of Rx chains for DL non-contiguous CCs:

-	Option 1: Only the “fully shared Rx chain architecture” needs to be studied

-	Option 2: RAN4 also studies the possibility of supporting  only 2Rx in the mandatory 4Rx band below 2700MHz

-	Other options are not precluded

•	Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study

•	FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier.

Issue 2-1-4: Study on power spectral density difference between carriers of co-located adjacent channel operators, Recommended WF:

•	FFS on following options in next meeting

-	Option 1: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure)

-	For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level

-	equal PSD between two co-location operators could be assumed from gNB transmisstion point of view for evaluation purpose, as the starting point

-	The PSD difference from UE reception point of view could vary as per the different antenna type, antenna pointing direction, etc between operators

-	Option 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered

-	Other options are not precluded

Issue 3-1-1: Assumptions on test configurations for evaluation: Recommended WF: See if RAN4 can agree on the following test configurations:

- The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos

  - UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACPR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

  - Reuse the Rel-15 assumption of Rx chain image rejection, which is 25dB

  - At this stage RAN4 does not take DL MIMO into account

  - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc.

- Other configurations are not precluded if identified as having performance impact

Issue 3-1-2: Prerequisite conditions: FFS on following proposals in next meeting:

- Proposal 1: Maintain the current ACS and IBB requirements to ensure UE selectivity

- Proposal 2: The fourth sub-bullet in the SID should be simplified to “Determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs”

- Proposal 3: RAN4 to confirm it is feasible to receive two non-contiguous CCs in a shared RF chains with the assumption that power spectral density imbalance between any of the CC and also the signal in the gap are within 6dB

  - FFS on assuming higher PSD for the signal in the gap of 2CC

Issue 3-1-3: Requirements for evaluation: Recommended WF:

Companies’ views are diverged but not controversial. Further discuss following bullets see if they can be agreeable as a package

- Both TDD and FDD intra-band DL contiguous CA should be included

- ΔRIBNC, ACS, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be evaluated

  - Out-of-band blocking and spurious response do not need requirement adjustment

- Adjusted requirements for fragmented carriers shall be separated from the existing FDD/TDD intra-band carrier non-contiguous aggregation ΔRIBNC, ACS, IBB, NBB

- Discussion on if a rejection level of the image caused by the in-gap interferer is needed

- Discussion on new requirement(s) if identified is not precluded

Separately, the SID was updated in last plenary in [3] with objectives copied below:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

§   Other requirements for study purpose are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

Observation: if the PSD imbalance between the two DL CCs is understood to be 6dB, such as with current intra-band DLCA cases, it is still unclear what the in-gap, but also adjacent CC power may be:

- For example, are all the in-gap channels and outer adjacent channels co-located?

- Is 6dB PSD imbalance then applicable?

- Is this a scenario that is so frequent that it deserves a specific RAN4 effort and optional UE capability?

In this contribution, we first discuss some of the architecture aspects, scenarios, and then the in-band blocking and ACS performance aspects.

## 2.2 Receiver branch architecture.

The goal is to enable the reception of two (or more?) non-contiguous CCs with a single Rx branch. It is important to compare this with the current architecture for non-contiguous DLCA, where each CC has a dedicated path from the down-conversion stage. This comparison is shown in Table 2. The comparison spans from the antenna to the digital BB and provides insights into impacted receiver performance.

Table 2: Comparison of the antenna to BB architecture for DL NCCA and Single Rx approaches.

| Scenario | Antenna and LNA | Down conversion | Analog BB | ADC | Digital BB | REFSENS, ACS and IBB perf. |
| --- | --- | --- | --- | --- | --- | --- |
| “normal” DL NC CA | Shared | Separate LOs per CC within the CC | Separate with distinct AGC and analog filter BW proportional to each CC | Separate with BW proportional to each CC | Separate with dedicated digital filter and FFT | Same for each CC than in single CC mode |
| Single Rx CCs not FFT aligned | Shared with shared AGC | Shared LO between the CCs | Shared with mutual AGC and analog filter with BW proportional to CC1+gap+CC2 BW | Shared with BW proportional to CC1+gap+CC2 BW => Reduced dynamic range? | Separate with dedicated digital filter and FFT | Worst REFSENS for CC with lower PSD and poor in-gap ACS and IBB |
| Single Rx CCs FFT aligned |  |  |  |  | Shared with Single FFT, potentially digital filtering of each CC from the in-gap interferers |  |

Observation:

- The key architecture difference is obviously in the analog BB section with a wide analog filter that does not reject the in-gap signals, but also offers less rejection on the outside of the wanted CCs. Also, this may result in issues with the UL signal when the duplex distance/gap is minimal.

- The second difference is for the ADC which must also operate on a larger BW, which may also result in some loss in dynamic range, while more dynamic range is required to cope with in-band blockers, the 6dB PSD imbalance, and a shared AGC.

- The other issue is that the LO is placed in the middle of the CCs, and thus a new image signal may need to be considered, together with strong in-gap interference pulling the LO.

- Finally, the digital BB may process the two CCs at once if they are FFT aligned, or separately if they are not FFT aligned.

  - In the FFT aligned case, the maximum total BW is 50MHz for a 15KHz SCS and 100MHz for a 30kHz SCS, to conform with the maximum FFT size.

As an alternative to this single Rx path approach, since the main issue is the lack of receive path for band combinations with a high number of CCs, including intra-band non-contiguous CCs, one option is to borrow receive paths from higher bands with 4Rx capability (mandatory or not) and be able to downgrade them to 2Rx, to cover a higher number of total CCs.

Proposal: In a similar way as specified for overlapping band non-collocated CA scenario, RAN4 studies the possibility of supporting only 2Rx in the 4Rx band in order to free up more receive paths to support more DL bands, and/or CCs:

- This approach may already be supported by current implementations.

- The network may configure, in a semi-static way the number of Rx per band, to trade some of the 4Rx receive path to higher number of DL bands, and/or CCs.

Proposal on architecture options: RAN4 studies different architecture limitations:

- Impact of a wider analog filter on DL reception for:

  - Outer interferers, ACS and IBB scenarios

  - Rejection of the UL signal for small duplex gap/distance FDD bands

- Impact of interferers falling at LO frequency or image frequency.

- Impact of wider BW ADC on its dynamic range and available headroom for in-gap interferers.

- For FFT aligned CCs, the maximum received BW separation for the two CCs/clusters, in one RX path is 50MHz for 15kHz SCS and 100MHz for 30kHz SCS to allow single FFT of maximum FFT size.

## 2.3 ACS and blocking requirements.

Even if there are other new impairments that will need to be reconciled, as we have raised in [1], such as Image rejection, IP2 and LO pulling issues, these may be secondary effects that will affect the performance only for corner cases. Thus, at this point, we should focus on the aspects related to the lack of analog filtering in-gap, and reduced rejection for outer interferers, which is tested with ACS and in-band blocking.

As discussed within the first section, we can focus on the current NR ACS and in-band blocking for bands < 2.7GHz to establish a reference for “normal” DL non-contiguous CA conditions.

These are illustrated in Figure 1, where a 100MHz range is considered, and 4 scenarios of non-contiguous CCs.

- The top four rows illustrate the in-band blocking test case

- The middle four rows illustrate the ACS test case 1 (low power adjacent interferer)

- The bottom four rows illustrate the ACS test case 2 (maximum power interferer)

- The four scenarios are as follows:

  - Two 5MHz CCs with a 60MHz gap

  - Two 10MHz CCs with a 50MHz gap

  - Two 20MHz CCs with a 30MHz gap

  - A 5MHz CC and a 20MHz CC with a 25MHz gap

  - In all cases, the gap is sufficient to illustrate the presence of the highest in-band blocker at -44dBm level.

Terminology:

- In-gap (dark blue) refers to signals that are in the region between the two wanted CCs (cyan)

- Outer (purple) refers to signals that are adjacent to the wanted signals, but outside the region between the two wanted CCs.

- This is represented in Figure 1 for the first row of the in-band blocking test.

If channel bandwidths from 5MHz to 80MHz are considered and assuming a -100dBm REFSENS for a 5MHz channel (band 1 reference):

- For in band blocking: the blocking signal is a 5MHz channel at -56dBm at 7.5MHz offset (second adjacent for a 5MHz wanted channel) from the wanted channel edge and -46dBm at 12.5MHz offset (third adjacent for a 5MHz wanted channel).

  - These powers are to be compared to a wanted signal that is REFSENS+6dB for 5 and 10MHz channels, to REFSENS+9dB for 20MHz, REFSENS+12dB for 40MHz and REFSENS+15dB for 80MHz.

  - While the in-band blockers are at -56dBm and -44dBm in-gap and see no rejection by the analog filter, the wanted signal is at -94dBm for 5MHz CC, -91dBm for 10MHz CC, and -85, -79 and -73dBm for 20, 40 and 80MHz CC respectively.

- For the ACS test case 1 (low power interferer with wanted signal that is close to REFSENS):

  - While the wanted signal is at REFSENS+14dB, the adjacent 5MHz interferer is at REFSENS+45.5dB for 5 and 10MHz channels to REFSENS+39.5dB for 20MHz, REFSENS+36.5dB for 40MHz and REFSENS+33.5dB for 80MHz.

  - Wanted and interfering signal levels are the following for:

    - 5MHz wanted CC: wanted power is -86dBm with adjacent at -54.5dBm

    - 10MHz wanted CC: wanted power is -83dBm with adjacent at -51.5dBm

    - 20MHz wanted CC: wanted power is -80dBm with adjacent at -54.5dBm

    - 40MHz wanted CC: wanted power is -77dBm with adjacent at -54.5dBm

    - 80MHz wanted CC: wanted power is -74dBm with adjacent at -54.5dBm

- For ACS test case 2 (interferer at maximum in band power):

  - While the 5MHz interferer is at -25dBm including in the gap where there is no rejection from the analog filter, the wanted signals are the following for:

    - 5MHz and 10MHz wanted CC: -56.5dBm

    - 20MHz wanted CC: -50.5dBm

    - 40MHz wanted CC: -47.5dBm

    - 80MHz wanted CC: -44.5dBm

Figure 1: In-band blocking and ACS case 1 and 2 scenarios for <2.7GHz bands

Observations:

- When the two wanted CC have a different channel bandwidth, both the in-band blocking and the ACS tests may already induce a PSD imbalance between the wanted CCs.

- In both test cases, the wanted signal is already assumed significantly above REFSENS while in the case of the reception of the two non-contiguous CCs with a shared Rx chain, the in-gap adjacent channels from other operators will be different in level, and BW, and will occupy the entire gap while satisfactory REFSENS is still desired.

- For in-gap, the interfering signals in both the in-band blocking test and the two ACS tests, are at least 30dB above the wanted signals.

  - With no in-gap analog rejection, this will saturate the ADC and will not allow any demodulation even for the ACS test where some digital filtering can be benficial.

- The reception of the two non-contiguous CCs, with a shared Rx chain, is only feasible if the the wanted and the in-gap interfering signal dynamic range are reasonable enough to be handled linearly by the ADC with sufficient SNR. Thus, the in-gap ACS and in-band blocking levels should be significantly reduced.

  - Also note that because the analog filter may be as wide as 70MHz in the example above (but could be even wider) when a 5MHz wanted signal is used, there will be much less rejection of the outer adjacent interferer or outer blocker as compared with the typical case, where the analog filter bandwidth is close to the wanted channel bandwidth.

- The reception of the two non-contiguous CCs, with a shared Rx chain is not feasible for wide received separation bandwidth, if the outer ACS and in-band blocking levels are not reduced.

- Note too that the larger analog filter BW may also cause a lower robustness against out of band blockers but also in the case of FDD bands with a small duplex gap, the own UL signal may not be rejected enough.

  - It should also be noted that the ADC dynamic range may be lower for a 70MHz bandwidth than for a 5MHz bandwidth.

Given the above observations, it is clear that the reception of two non-contiguous CCs, with a shared Rx chain, is not feasible under the current in-band blocking and ACS tests conditions in both the in-gap and outer channels scenario. Thus, this feature can only work if all the channels in the band are collocated and at least the in-gap, but also the outer channels should be at acceptable levels. Given the collocated assumptions, at minimum, the in-gap channels should be within 6dB imbalance as well. Nonetheless,  if there are separate radio units and uncoordinated schedulers, this seems may be far too optimistic.

Proposal on in-band blocking and ACS scenarios:

- In-gap adjacent channel selectivity, and in-band blocking levels, should be derived from a realistic deployment scenario where not only the other wanted CC is collocated, but also where all the in-gap channels are collocated.

  - The wanted signal shall assume a 6dB PSD imbalance based on collocated BS and RU

  - The interfering signals should assume >6dB PSD imbalance as they may derive from a different RU

- Although the signal level may be more relaxed, as they still benefit from some rejection of the wider analog filter, outer adjacent channel selectivity and in-band blocking levels will have to be reduced compared to the current specification, and it may also require a collocated assumption.

  - The wanted signal shall assume a 6dB PSD imbalance based on a collocated BS and RU

  - The interfering signals should assume >6dB PSD imbalance, as these may derive from a different RU or ppossibly from a different BS.

- The impact on receiver blocking, and the performance of a wider analog filter, on the rejection of the UL signal for FDD bands with small duplex gap and distance, should be studied.

# Conclusion

In this contribution, based on our previous work on the topic and further extending our investigations on architecture aspects and ACS and in-band blocking scenarios, we formulate the following proposals.

Proposal on examples for the study:

- Bands that are <2.7GHz are used as examples and ACS and blocking for <2.7GHz is used for reference.

- For non-contiguous CCs, 5MHz is used as minimum CBW and maximum CBW is 90MHz.

- The example band(s) should already have NC ULCA specified.

Proposal: In a similar way as specified for overlapping band non-collocated CA scenario, RAN4 studies the possibility of supporting only 2Rx in the 4Rx band in order to free up more receive paths to support more DL bands, and/or CCs:

- This approach may already be supported by current implementations.

- The network may configure, in a semi-static way the number of Rx per band, to trade some of the 4Rx receive path to higher number of DL bands, and/or CCs.

Proposal on architecture options: RAN4 studies different architecture limitations:

- Impact of a wider analog filter on DL reception for:

  - Outer interferers, ACS and IBB scenarios

  - Rejection of the UL signal for small duplex gap/distance FDD bands

- Impact of interferers falling at LO frequency or image frequency.

- Impact of wider BW ADC on its dynamic range and available headroom for in-gap interferers.

- For FFT aligned CCs, the maximum received BW separation for the two CCs/clusters, in one RX path is 50MHz for 15kHz SCS and 100MHz for 30kHz SCS to allow single FFT of maximum FFT size.

Proposal on in-band blocking and ACS scenarios:

- In-gap adjacent channel selectivity, and in-band blocking levels, should be derived from a realistic deployment scenario where not only the other wanted CC is collocated, but also where all the in-gap channels are collocated.

  - The wanted signal shall assume a 6dB PSD imbalance based on collocated BS and RU

  - The interfering signals should assume >6dB PSD imbalance as they may derive from a different RU

- Although the signal level may be more relaxed, as they still benefit from some rejection of the wider analog filter, outer adjacent channel selectivity and in-band blocking levels will have to be reduced compared to the current specification, and it may also require a collocated assumption.

  - The wanted signal shall assume a 6dB PSD imbalance based on a collocated BS and RU

  - The interfering signals should assume >6dB PSD imbalance, as these may derive from a different RU or ppossibly from a different BS.

- The impact on receiver blocking, and the performance of a wider analog filter, on the rejection of the UL signal for FDD bands with small duplex gap and distance, should be studied.

# References

[1] R4-2413031 On architecture options for fragmented spectrum reception, Skyworks Solutions Inc., RAN4#112

[2] R4-2414457 WF on fragmented DL, MediaTek, RAN4#112

[3] RP-242404	Revised SID: Study on NR FR1 DL fragmented carriers	MediaTek Inc., RAN#105