# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415538

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for common signal or channel adaptation in R19 NES

Agenda Item:	6.22.2.2

Document for:	Discussion

Introduction

In RAN4 #112, the WF for R19 NES [1] is approved. In [1], regarding RS adaptation, the following is agreed.

Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress. Issue 3-2-1: Paging adaptation Agreement: - Deprioritize Paging adaptation until the RAN1 concrete conclusion is available. Issue 3-3-1: PRACH adaptation Agreement: - Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.

Based on latest RAN1/2 conclusions, the RRM requirement impact from paging adaptation or PRACH adaptation is still not clear. Hence, in this contribution, we provide our views on RRM requirements SSB adaptation in connected mode.

Discussion

<On ppp >

In RAN4 #112, the follow issue was discussed.

Issue 4-1-5: The transition period of SSB burst adaptation - Proposals - Option 1: Qualcomm, Nokia, CMCC, vivo - RAN4 to investigate transition period requirements by SSB periodicity adaptation. - Option 1-1: Nokia, vivo - RAN4 to investigate whether legacy SSB burst transition period requirement can be a starting point. - Option 2: MediaTek - RAN4 to discuss the necessity of SSB time domain adaptation requirements on the transitions. - Option 3: Xiaomi - Not to define the transition period requirement. - Only define requirement when no SSB adaptation period changes within one reporting interval

For SSB adaptation, latest RAN1 agreements is as follows

Agreements in RAN1 #117 For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreements in RAN1 #118 For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Firstly, RAN1 may still needs to further discuss whether to support option 2 above, i.e. adaptation based on two SSB configurations. In our view, option-2 potentially means different gNB Tx beam patterns may be assumed between the 2 SSB configurations. However, since RAN1 has not concluded this, we slightly prefer to start RAN4 discussion based on option 1 in RAN1 #117.

Proposal 1  RAN4 may start to discuss RRM requirement impacts based on adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s). RAN4 may further discuss RRM requirement impacts after adaptation based on two SSB configurations is supported in RAN1.

Secondly, so far only NCD-SSB based SSB adaptation for SCell is supported. RAN4 may further discuss RRM requirements on this part. For SCell, at least there is no RLM. Other RRM requirements, including L3 measurements, L1-RSRP/L1-SINR, and BFD/CBD, can be investigated. Note that CBD is only performed after beam failure is detected, and it would be a corner case if SSB periodicity is changed during the CBD period. Hence we prefer the legacy rules, i.e. not to discuss CBD.

Proposal 2  RAN4 prioritize the following RRM requirement impacts due to SSB adaptation on SCell.

- L3 measurements, including cell identification and measurements

- L1-RSRP/L1-SINR measurements

- BFD

So far, the RAN1 conclusion for SSB adaptation is dynamic adjustment of SSB periodicity, as discussed above. Actually, RRM requirements for transition period, when UE switches between DRX and non-DRX, is already clear for all kinds of RRM requirements, e.g. RLM/BFD/CBD, L1-RSRP/L1-SINR measurement and L3 measurement. RAN4 may further check whether no additional impact is necessary.

In existing BFD requirements, transition period requirements are defined for 4 different cases, as cited below. The transition period requirements defined for L3 measurements and L1-RSRP/L1-RSRP measurements are also cited.

TS 38.133 v18.6.0 ### 8.5.10 Minimum requirement at transitions for beam failure detection When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each BFD-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each BFD-RS resource. When the UE transitions from a first configuration of BFD resources to a second configuration of BFD resources that is different from the first configuration, for each BFD resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each BFD resource present in the second configuration. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for BFD present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between BFD CD-SSB resource and BFD NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation period corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each BFD-RS resource. ### 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition. When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

For the first two types of transition requirements of BFD, there is no requirements during transitions. For the third type, there is no transition period, as the requirements switches immediately after TCI is switched. For the 4th type, UE follows the more relaxed requirements between the two different requirements, when possible.

For the L1/L3 part, similar requirements are defined in 9.1.6 of TS 38.133. In that case, only the case with more relaxed requirements is considered.

Based on existing requirements, it is worth to discuss how to capture the SSB adaptation requirements by transition period. Our understanding is that at least RAN4 shall follow the L3 measurement approach, i.e. UE follows the more relaxed requirements between the two different requirements, when possible.

Proposal 3  For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 may start to discuss RRM requirement impacts based on adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s). RAN4 may further discuss RRM requirement impacts after adaptation based on two SSB configurations is supported in RAN1.

Proposal 2  RAN4 prioritize the following RRM requirement impacts due to SSB adaptation on SCell.

- L3 measurements, including cell identification and measurements

- L1-RSRP/L1-SINR measurements

- BFD

Proposal 3  For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements.

- References

[1]  R4-2413885  WF on Netw_Energy_NR_enh, Ericsson. RAN4 #112