# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415540

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.4.3

Source:	Apple

Title:	On RRM core for less than 5MHz Phase 2

Document for:	Discussion

# Introduction

The WF of [112][214] NR_FR1_lessthan_5MHz_BW_Ph2 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2 [2].

In this contribution, we continue the discussion on the RRM requirement for the WI of less than 5MHz Phase 2.

# RRM for CA/DC in less than 5MHz

In CA/DC with less than 5MHz, if the PCell is using 3MHz in band n100, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can be applied without any change, e.g., cell identification/measurement/SSB-index reading, and SSB based RLM/BFD/CBD, handover and etc.

Proposal 1: In CA/DC with less than 5MHz, if PCell is using 3MHz, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can still be applied on this PCell.

Issue 2-1-1: SCell activation and deactivation delay requirements:

- Agreement - Specify new SCell activation delay requirements to support less than 5MHz bandwidth in the target SCell in FR1. - Agreement - Start from single SCell activation delay requirements.

In fast single SCell activation requirement and NES inter-band SSB-less SCell activation, we use A-TRS and TRS for AGC refinement and T/F tracking during SCell activation procedure, like in the following spec text:

Fast SCell activation example:

NES inter-band SSB-less SCell activation example:

TRS and A-TRS are essentially CSI-RS, but the CSI-RS based measurement requirement is not so far specified or verified for less than 5MHz case. In last RAN4 meeting, we had another requirement that,

Agreement:

  - the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement.

CSI-RS based L1 and L3 measurement performance is not guaranteed in current spec for less than 5MHz case, and therefore, UE is allowed to not support this fast SCell activation and inter-band SSB-less SCell activation on less than 5MHz band, i.e., no 3GPP requirement applies for fast SCell activation and inter-band SSB-less SCell activation on less than 5MHz band. Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, UE is not required to support fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation applies to SCell on 5MHz and larger BW. Otherwise, if UE is still required to support such above features, the sample number of A-TRS and TRS shall be increased in the SCell activation requirement, e.g., more than 2 A-TRS occasions or more than 2 TRS for the target SCell activation on less than 5MHz band.

Proposal 2: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW.

One more thing is about L1-RSRP measurement during the SCell activation, RAN4 has following requirement in FR1 SCell activation:

The L1-RSRP measurement in the current SCell activation requirement can be based on either SSB or CSI-RS. However, if the CSI-RS L1 measurement has no requirement on less than 5MHz band, we assume the performance of CSI-RS L1 measurement on less than 5MHz would be not reliable. In that sense, we suggest to only define the requirement of SCell activation with SSB based L1-RSRP measurement on less than 5MHz band, e.g., assume to only use SSB based L1-RSRP measurement/report on being-activated SCell on less than 5MHz band. Otherwise, if CSI-RS is configured for L1-RSRP measurement during SCell activation, more CSI-RS samples shall be used for L1-RSRP measurement, e.g., M=3.

Proposal 3: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell.

Issue 2-1-3: Whether intra-band CA is in the scope:

- Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

As described in WID, we didn’t see any scenario required CA/DC have two or more serving cells with less than 5MHz, that is, RAN4 only needs to consider following scenarios:

- CA with 3MHz PCell + 5MHz or 10MHz SCell(s)

- CA with 5MHz or 10MHz PCell + one 3MHz SCell

- DC with 3MHz PCell + 5MHz or 10MHz PSCell

- DC with 5MHz or 10MHz PCell + 3MHz PSCell

Since the detailed band combination of CA/DC needs to wait RF conclusions, it’s better to decide the corresponding RRM requirement after RF session has concrete conclusions, e.g., whether or not we need to consider intra-band CA case.

Proposal 4: RAN4 to decide the corresponding RRM requirement for intra-band CA after RF session has concrete conclusions.

In R18 less than 5MHz, we had some agreements in RAN4#109 (R4-2321629) on whether network needs to provide the bandwidth information to UE during MO configuration or HO, and the main reason is because CD-SSB PCell has to be on sync raster and R18 only considered single carrier case for less than 5MHz.

## Sub-topic 1-2 Assistance information for PBCH is 12 or 20 PRBs Agreement: There is no need not include information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

However, if now SCell can also use 12 PRB SSB bandwidth, as CD-SSB of SCell can be off sync raster, then it would be necessary to provide assistance information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command (HO with SCell or PSCell), otherwise, UE may assume wrong PBCH DMRS bandwidth and fail the SSB index reading.

Proposal 5: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in MO and/or HO command.

Following table is the summary for CA/DC requirement and potential impacts due to new bandwidth.

Table 1. CA/DC requirements impacts due to less than 5MHz band

| Section ID | CA/DC requirements | Impacts due to less than 5MHz band |
| --- | --- | --- |
| 6.1.5 | HO with PSCell | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 6.1.6 | NR Conditional Handover including target MCG and target SCG | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 7.1 | UE transmit timing | No, the reference cell for timing requirement can reuse the legacy assumption. |
| 7.5, 7.6 | MRTD/MTTD requirement | No, reuse the legacy CA/DC requirement. |
| 8.2.2, 8.2.4 | Interruption requirement related with SCell/PSCell operation | No, reuse the legacy interruption requirement. |
| 8.3 | SCell Activation and Deactivation Delay | Need FFS, if target to-be-activated SCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCell activation requirement. |
| 8.4 | UL carrier RRC reconfiguration delay | No, reuse the legacy requirement. |
| 8.5.9 | Requirements for Beam Failure Recovery in SCell | No, reuse the legacy BFR requirement. |
| 8.9, 8.9A, 8.9C | NR-DC: PSCell Addition and Release Delay; Conditional PSCell Addition Delay; Subsequent Conditional PSCell Addition Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell addition requirement. |
| 8.11, 8.11B, 8.11E | PSCell Change; Conditional PSCell Change; Subsequent Conditional PSCell Change | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement. |
| 8.17 | SCG Activation and Deactivation Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell activation requirement. |
| 9.1.2 | Measurement gap | No, reuse the legacy MG requirement. |
| 9.1.3 | UE Measurement capability | No, reuse the legacy requirement. |
| 9.1.4 | Capabilities for Support of Event Triggering and Reporting Criteria | No, reuse the legacy requirement. |
| 9.1.5 | CSSF | No, reuse the legacy CSSF requirement. |
| 9.1.7~13 | MG enh and MUSIM MG | NA for this WI (MG enh is not considered) |
| 9.2.3 | Number of cells and number of SSB (FR1) | No, reuse the legacy requirement. |
| 9.2.5 | Intra-frequency measurements without measurement gaps | Yes, time period for time index detection for deactivated SCell or deactivated PSCell can be extended of this SCell or PSCell is using 3MHz. |
| 9.2.6 | Intra-frequency measurements with measurement gaps | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |
| 9.2.7 | Intra-frequency measurements with NCSG | NA for this WI (MG enh is not considered) |
| 9.3 | NR inter-frequency measurements | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |

Proposal 6: agree the above summary table.

# Conclusion

In this contribution, we discuss the RRM requirement for co-located and non-co-located inter-band NR CA/DC UE with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band.

Proposal 1: In CA/DC with less than 5MHz, if PCell is using 3MHz, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can still be applied on this PCell.

Proposal 2: Even though UE capability indicates to support fast SCell activation or inter-band SSB-less SCell activation, no requirement shall be applied for fast SCell activation or inter-band SSB-less SCell activation when the target SCell BW is less than 5MHz, i.e., UE capability of fast SCell activation and inter-band SSB-less SCell activation only applies to SCell on 5MHz and larger BW.

Proposal 3: SCell activation requirement shall not be applied when CSI-RS is configured for L1-RSRP measurement/report for this being-activated SCell.

Proposal 4: RAN4 to decide the corresponding RRM requirement for intra-band CA after RF session has concrete conclusions.

Proposal 5: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in MO and/or HO command.

Proposal 6: agree the following summary table.

| Section ID | CA/DC requirements | Impacts due to less than 5MHz band |
| --- | --- | --- |
| 6.1.5 | HO with PSCell | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 6.1.6 | NR Conditional Handover including target MCG and target SCG | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 7.1 | UE transmit timing | No, the reference cell for timing requirement can reuse the legacy assumption. |
| 7.5, 7.6 | MRTD/MTTD requirement | No, reuse the legacy CA/DC requirement. |
| 8.2.2, 8.2.4 | Interruption requirement related with SCell/PSCell operation | No, reuse the legacy interruption requirement. |
| 8.3 | SCell Activation and Deactivation Delay | Need FFS, if target to-be-activated SCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCell activation requirement. |
| 8.4 | UL carrier RRC reconfiguration delay | No, reuse the legacy requirement. |
| 8.5.9 | Requirements for Beam Failure Recovery in SCell | No, reuse the legacy BFR requirement. |
| 8.9, 8.9A, 8.9C | NR-DC: PSCell Addition and Release Delay; Conditional PSCell Addition Delay; Subsequent Conditional PSCell Addition Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell addition requirement. |
| 8.11, 8.11B, 8.11E | PSCell Change; Conditional PSCell Change; Subsequent Conditional PSCell Change | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement. |
| 8.17 | SCG Activation and Deactivation Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell activation requirement. |
| 9.1.2 | Measurement gap | No, reuse the legacy MG requirement. |
| 9.1.3 | UE Measurement capability | No, reuse the legacy requirement. |
| 9.1.4 | Capabilities for Support of Event Triggering and Reporting Criteria | No, reuse the legacy requirement. |
| 9.1.5 | CSSF | No, reuse the legacy CSSF requirement. |
| 9.1.7~13 | MG enh and MUSIM MG | NA for this WI (MG enh is not considered) |
| 9.2.3 | Number of cells and number of SSB (FR1) | No, reuse the legacy requirement. |
| 9.2.5 | Intra-frequency measurements without measurement gaps | Yes, time period for time index detection for deactivated SCell or deactivated PSCell can be extended of this SCell or PSCell is using 3MHz. |
| 9.2.6 | Intra-frequency measurements with measurement gaps | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |
| 9.2.7 | Intra-frequency measurements with NCSG | NA for this WI (MG enh is not considered) |
| 9.3 | NR inter-frequency measurements | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |

# References

[1] R4-2413883, WF for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2, Intel, RAN4 #112

[2] R4-2411809, Topic summary for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2, Intel, RAN4#112