# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415543

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.26.4.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][226] NR_NTN_Ph3 [2].

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# RRM for downlink coverage enhancements

Issue 2-1-1: RRM impact of SSB periodicity enhancement

<Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC): - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a (Samsung): - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

In last RAN1 and RAN2 meetings, some agreements for DL coverage have been made as followings:

Agreement in RAN1 #118 As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement Agreement in RAN2 #127 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

The SSB periodicity extension for DL coverage enhancement can probably impact UE initial access procedure compared with the legacy assumption of 20ms. RAN4 may need to double check if any RRM requirement shall be clarified when the SSB periodicity is changed to a value different from 20ms.

In the current initial access assumption for cell selection, it was defined in RAN1 that,

In the current NTN network (R17, and R18), by default NTN UE will try the initial access to network assuming SSB periodicity as 20ms, however, if the R19 NTN network changes the SSB periodicity to be more than 20ms, it will cause the initial access failure for the R17/R18 NTN UEs. Even for the R19 NTN UE, it has no clear idea if the network behavior is following R17/R18 or R19, and the UE behavior is unclear for implementation during the initial access.

Besides the initial access, there are other UE behaviors requiring assumption of the by-default SSB periodicity in the previous NTN networks (R17/R18). Some examples as followings:

    - For RACH based HO, CHO,  RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

    - For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

    - For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

With the SSB periodicity change(e.g., change to a SSB periodicity > 20ms), the above corresponding requirement needs to be modified.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

If the SSB periodicity is selected to be greater than 160ms, many of the RRM requirements need to be revisited, we identified two key categories: (1)measurement/evaluation requirement with DRX; and (2) UE timing requirement.

(1)identification/measurement/evaluation requirement with DRX

For example, in IDLE mode, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; and the serving cell evaluation period is defined as following:

The M1*N1*DRX cycle can be as low as 320ms, but if the SSB periodicity is chosen as 640ms, the current serving cell measurement interval and evaluation period needs to be re-defined, e.g.,

    - If SSB periodicity >=320ms, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S for the serving cell at least once every M1*N1*max{DRX cycle, SSB periodicity}

    - If SSB periodicity >=320ms and DRX=320ms, The Nserv may be changed to M1*N1*4* max{DRX cycle, SSB periodicity}

(2)UE timing requirement

In current NTN timing requirement, it’s specified that the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. However, if SSB periodicity is chosen to be greater than 160ms, UE may not be able to get an available SSB in the last 160ms before the UL transmission, as shown in following figure.

Figure 1. UE may not get SSB in last 160ms for Tx timing decision

In our view, TRS is always a reliable way for UE to acquire the DL serving cell timing if the SSB is not available. Thus, RAN4 may consider to have the requirement as: The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB or TRS is available at the UE during the last 160 ms.

Proposal 2: If SSB periodicity is greater than 160ms for DL coverage enhancement, RAN4 needs to revisit the identification/measurement/evaluation requirement with DRX in IDLE/Inactive/Connected modes, as well as the timing requirement. However, RAN4 shall wait for decision from RAN1/2 on the SSB periodicity.

In our view, if such SSB periodicity extension is adopted in RAN1, it’s only applied for NTN operation on NTN bands. We think it might be beneficial to have some new assumption at UE to help initial access to R19 NTN, and one possible approach could be to specify band or region specific SSB periodicity. For example, UE will perform initial access by searching the cells with a specific SSB periodicity associated with the specific band. It can be defined in RF that each NTN band is associated with one or more SSB periodicity in RF spec, like the association between band and SCS in existing TS38.101-1/2, e.g., Band X is associated with SSB periodicity M. With this approach, network can follow the RF spec to transmit SSB on each specific band by using the associated periodicity. But details of the solution can be further discussed after RAN1 has more conclusions.

Issue 2-2-1: RRM impact of DTX/DRX

In the note of this SSB periodicity extension, WID has a note that “Rel-18 network energy saving techniques should be considered as baseline in the system level study”. One key technology in Rel-18 NES is the DTX for network energy saving, and RAN4 assumed the SSB and other RS for RRM shall be transmitted even outside DTX active window. However, in R19 NTN enhancement, to support DL coverage enhancement (network may sweep Tx beams among different footprints) SSB may not be transmitted within certain footprint outside certain time window, e.g., outside DTX active window. If that’s the assumption of DTX in Rel-19 NTN, we can foresee that it must have RRM requirement impact compared with Rel-18 NES DTX. Since how to apply the DTX/DRX mechanism of NES to Rel-19 NTN is still under discussing in RAN1, RAN4 shall wait more conclusions from RAN1 before starts RRM work.

Proposal 2: RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

# RRM for uplink Capacity/Cell Throughput Enhancement

Issue 3-1-1: RRM impact for uplink Capacity/Cell Throughput Enhancement

For UL capability/cell throughput enhancement, enhancement may only need RF requirement update instead of any RRM part. Thus. RAN4 doesn’t need to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

Proposal 3: No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

# RRM for support of regenerative payload

Issue 5-2-1: Timing requirements for regenerative payload

<Way Forward> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

RAN4 received one LS (R2-2407623) on common TA in a regenerative payload scenario from RAN2 that,

In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

In early RAN1 discussions, the uplink time synchronization reference point is configured in the feeder link. If the uplink time synchronization reference point is set to satellite, then TAcommon is configured with value 0. For regenerative payload, it is generally assumed that the uplink time synchronization reference point is set to satellite/gNB. Since the regenerative payload scenario is already covered, we do not see the necessity to introduce negative TAcommon values.

Based on our understanding in RAN2 discussions, the main motivation of introducing negative TAcommon values is that network may want to configure a smaller common TA than the actual value in transparent payload to compensate the over-estimated UE specific TA.

Consider the case where UE over-estimates UE specific TA, which results in the TA value used by UE for PRACH transmission is larger than the actual TA value. Then network is unable to fix it in Msg2 since the timing advance command in Msg2 only supports a non-negative value. Specifically, according to the TA formula: TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUE⋅Tc, it is possible that UE estimated NTA,adjUE, estimated may be larger than the actual NTA,adjUE, actual, and hence, UE used TTAused is larger than the actual TTAactual. Note that a negative TA error is unable to be corrected in Msg2. To address this issue, network may artificially set a smaller NTA,adjcommon, configured value than the actual NTA,adjcommon, actual, so that the overall TTAused is always smaller than the actual TTAactual.

However, we do not think the over-estimated UE specific TA leads to a significant issue. First of all, the timing error requirement Te_NTN is upper bounded by 29⋅64⋅Tc for SCS=15 kHz. This implies that the over-estimated UE specific TA should be smaller than 944 ns, and as long as UE can meet the timing requirement the UE specific TA error shall not be beyond the assumption we used for Te calculation in RAN4. Furthermore, once a UE enters CONNECTED mode, the TA command via MAC CE could fix this negative TA value. Overall, we think the system can work without necessity of providing a smaller NTA,adjcommon, configured value than the actual NTA,adjcommon, actual. This applies to both transparent payload and regenerative payload.

With the above arguments, we have the following proposal.

Proposal 4: Regarding the question in RAN2 LS on common TA in a regenerative payload scenario, it is not a problem to stick to 0 as the minimum possible value for TACommon in a regenerative payload scenario, and we should not introduce negative values for ta-Common.

Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload

<Way Forward> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

The satellite switching with sync for regenerative payload has been discussed in last RAN plenary meeting, but no conclusion has been made, as following:

RP-241937 Considerations on the Satellite Switching with re-sync over Regenerative Architecture Thales: could be considered in the future Nokia: we also see a value in the proposal of this Tdoc RAN chair: strong support: 5 companies, strong concerns: 2 companies CMCC: proposal is related to RAN3 and not RAN2 The document was noted.

And therefore, we think it shall be still left to RAN3/2 decision before RAN4 can discuss the corresponding requirement.

Regarding the RRC re-establishment requirement with regenerative payload, it was concluded in RAN3 #125 that,

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

Thus, the legacy NTN RRC re-establishment requirement can be applied to the scenario with regenerative payload.

Regarding the RACH-less handover with regenerative payload, RAN2 concluded in RAN2#127 that, RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed. Thus, the legacy RACH-less HO requirement for NTN can be applied to scenario with regenerative payload.

Proposal 5: Whether to update the specific RRM requirements for satellite switch with regenerative payload should wait for RAN3/2 progress.

Proposal 6: the legacy NTN RRC re-establishment requirement can be applied to the NTN with regenerative payload.

Proposal 7: the legacy NTN RACH-less HO requirement can be applied to the NTN with regenerative payload.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

Proposal 2: If SSB periodicity is greater than 160ms for DL coverage enhancement, RAN4 needs to revisit the identification/measurement/evaluation requirement with DRX in IDLE/Inactive/Connected modes, as well as the timing requirement. However, RAN4 shall wait for decision from RAN1/2 on the SSB periodicity.

Proposal 3: No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

Proposal 4: Regarding the question in RAN2 LS on common TA in a regenerative payload scenario, it is not a problem to stick to 0 as the minimum possible value for TACommon in a regenerative payload scenario, and we should not introduce negative values for ta-Common.

Proposal 5: Whether to update the specific RRM requirements for satellite switch with regenerative payload should wait for RAN3/2 progress.

Proposal 6: the legacy NTN RRC re-establishment requirement can be applied to the NTN with regenerative payload.

Proposal 7: the legacy NTN RACH-less HO requirement can be applied to the NTN with regenerative payload.

# References

[1] R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT, RAN4#112

[2] R4-2411821, Topic summary for [112][226] NR_NTN_Ph3, CATT, RAN4#112