# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415544

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.8.3.4

Source:	Apple

Title:	On less than 5MHz for NTN RRM

Document for:	Discussion

# Introduction

The RAN4 led NTN enhancement WID has been revised in RAN plenary #105 and the latest WID was approved in [1]. The core part of this WI includes high power UE (HPUE) for NTN and less than 5MHz for NTN. We didn’t see impact of HPUE to RRM requirement but the less than 5MHz for NTN will decently need new RRM requirement in NTN.

The WF for RRM of NTN with channel BW less than 5MHz [2] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][216] NR_IoT_NTN_req_test_enh [3].

In this contribution, we continue the discussion the RRM requirement for less than 5MHz in NR NTN.

# Discussion

In current Rel-18 TS38.133, the main change to legacy requirement due to less than 5MHz are followings:

- Applicability requirement in clause 3.6.18

- Handover requirement (T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.)

- RLM requirement (PDCCH transmission parameters)

- BFD requirement (PDCCH transmission parameters)

- time index detection for intra-frequency/inter-frequency cell identification with/without MG

Like the TN less than 5MHz, we also think the above change shall be applied to NTN requirement as well. Thus, the RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement, e.g., T∆ in HO requirement, PDCCH transmission parameters in RLM/BFD requirement, and	time index detection for intra-frequency/inter-frequency cell identification with/without MG.

Proposal 1: the relaxation/extension part of RRM requirement for TN less than 5MHz can also be used for NTN less than 5MHz requirement, e.g.,

- T∆ in HO requirement,

- SSB index acquisition for intra-frequency/inter-frequency cell identification with/without MG.

Issue 1-3-3: whether timing requirements for NTN in current TS38.133 will be impacted becasue of less than 5MHz bandwidth

For TN less than 5MHz case, the UE timing requirement was not changed from the legacy TN band, that means, the Rx timing acquisition will not be impacted by less than 5MHz BW. Furthermore, NTN UE timing requirement additionally considered the UE specific TA and common TA parts, and those are irrelevant to less than 5MHz band. Thus, the timing requirements for NTN in current TS38.133 will not be impacted by using less than 5MHz bandwidth.

Proposal 2: the timing requirements for NTN in current TS38.133 will not be impacted by using less than 5MHz bandwidth.

Issue 1-3-4: RLF/BFD/CBD

WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: - The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario

In TN less than 5MHz, the hypothetical PDCCH transmission parameter for RLM/BFD has been updated as following example:

Since in this WID it stated that “3MHz with the RAN1 design in Rel-18”, both 12PRB/15PRB DL transmission BW shall be considered for NTN as well. Thus, the same hypothetical PDCCH transmission parameter for RLM/BFD in R18 TN less than 5MHz can also be reused to FR1 NTN with less than 5MHz case. However, only SSB based RLM/BFD/CBD shall be considered in this WID (as agreed in last RAN4 meeting), the SSS used for RLM/BFD/CBD evaluation will not be impacted by less than 5MHz BW, and therefore there is no need to modify the evaluation period for legacy RLM OOS/IS, BFD and CBD.

Proposal 3: the same hypothetical PDCCH transmission parameter for RLM/BFD in R18 TN less than 5MHz can also be reused to FR1 NTN with less than 5MHz.

Proposal 4: The legacy RLM OOS/IS, BFD and CBD evaluation period can be reused when defining the related requirements for less than 5MHz in NTN scenario.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: the relaxation/extension part of RRM requirement for TN less than 5MHz can also be used for NTN less than 5MHz requirement, e.g.,

- T∆ in HO requirement,

- SSB index acquisition for intra-frequency/inter-frequency cell identification with/without MG.

Proposal 2: the timing requirements for NTN in current TS38.133 will not be impacted by using less than 5MHz bandwidth.

Proposal 3: the same hypothetical PDCCH transmission parameter for RLM/BFD in R18 TN less than 5MHz can also be reused to FR1 NTN with less than 5MHz.

Proposal 4: The legacy RLM OOS/IS, BFD and CBD evaluation period can be reused when defining the related requirements for less than 5MHz in NTN scenario.

# References

[1] RP-241985, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung, Xiaomi, RANP#105

[2] R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi, RAN4#112

[3] R4-2411811, Topic summary for [112][216] NR_IoT_NTN_req_test_enh, Xiaomi, RAN4#112