3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415555

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.26.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Non-Terrestrial Networks (NTN) for NR Phase 3” described in latest WID [1], and “(e)RedCap UE with NR NTN operating in FR1-NTN bands” are shown as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In last meeting, we have already discussed RRM impacts by above objective and achieved some agreements. In this contribution, we continue to discuss the RRM core requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands

In last RAN4#112 meeting, RAN4 group achieved some agreements in [2]. There are still some open issues in [2] and we will discuss them one by one as below.

#### Issue 6-1-5: The network scenario considered for (e)Redcap UE with FR1-NTN <Way Forward> - For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier. #### Issue 6-1-6: The SCS considered for (e)Redcap UE with FR1-NTN <Way Forward> - For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

For the network scenario, because NTN only support SA operation, for RedCap UE, it should be SA operation.

For the SCS support of RedCap UE in FR1-NTN, we cannot observe the exception or difference for handheld UE and RedCap UE, it should support 15kHz, 30kHz and 60kHz.

Proposal 1: For Issue 6-1-5, scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

For Issue 6-1-6, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

In RAN4#112 meeting, the overall RRM requirements of (e)RedCap UE in FR1-NTN are agreed and listed in WF [2] as below. In addition, there are still some parts are FFS as yellow part:

#### Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN Online agreement <Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps

<Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

From LTE and NR, the driving testing has been introduced. However, it is not the most important use scenarios for (e)RedCap UE in NTN.

Proposal 2: RAN4 NOT to define the requirements for NTN (e)RedCap in Rel-19:

    - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state

For NR Rel-17/18 Conditional Handover, and RACH-less Handover, it is a typical usage in NTN system. For example, the time-based CHO is very useful for UEs in NGSO such as LEO deployments.

For the CG-SDT and RA-SDT, the small data transmission has been introduced for power saving especially for IoT devices such as RedCap UEs. To support such features for NTN (e)RedCap have the same benefit.

Proposal 3: RAN4 to define the requirements for NTN (e)RedCap in Rel-19:

    - NR Rel-17/18 Conditional Handover, and RACH-less Handover

    - Configured Grant based Small Data Transmissions (CG-SDT)

    - Random access based Small Data Transmissions (RA-SDT)

For the RRM requirements for 2Rx/1Rx (e)RedCap UE in FR1-NTN, the agreements and open issues are shown in WF [2]:

#### Issue 6-3-1: The principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN Online agreement <Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. - For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. - Meanwhile, consider the following difference between 2Rx (e)RedCap and non-RedCap UE: - The number of searchers - Others if any #### Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ? <Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

If compare the current TN redcap/eRedCap/FR1-NTN requitements for 2Rx and 1Rx:

Proposal 4:

| RRC_Idle mode mobility and RRC_Inactive mode mobility | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, the ranking should be 1dB relaxed. |
| --- | --- |
| Handover | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse 1 extra Trs as TN 1Rx UE |
| NTN specific: - HO based RACH - RACH-less HO - Time/location-based CHO with/without L3 measurement - Satellite switching with re-sync | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, use 1 extra delay in FR1-NTN requirements |
| RLM/BFD | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse TN 1 Rx UE for hypothetical PDCCH parameters, for the measurement periods, reuse the extended measurement periods as 1Rx in TN |
| Measurement procedure: Intra-frequency measurement w/o MG Inter-frequency measurement w/o MG | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse the additional samples of 1Rx in TN in legacy FR1-NTN requirements. |
| Accuracy | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse 1 more dB relaxation as 1Rx in TN |

For eDRX, it is still open and shown in WF [2]:

#### Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

As we present in last meeting, eDRX has been introduced in Rel-17 and further enhanced in Rel-18. By using eDRX, the devices can achieve better power saving performance especially for IoT devices. In NTN deployments, it can also benefit for power saving for UEs especially for (e)redcap UEs. Therefore, we support to apply the eDRX for (e)RedCap UEs in NTN scenarios.

Proposal 5: To support RedCap/eRedCap in FR1-NTN, RAN4 should to add support eDRX in RRC_Idle/RRC_Inactive mode.

In Rel-17 NTN system, both GSO and NGSO (LEO) are supported.

Observation 1: In eDRX_IDLE cycle, the values are extended to more than 10.24s or even larger one 10485.76s.

Observation 2: In typical LEO scenario such as R=50km and 7.56km/s for satellite speed, not all eDRX can be applicable for NGSO (LEO) scenario.

Proposal 6: RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario.

- Conclusion

In this contribution, we provide our proposals for RRM core requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: For Issue 6-1-5, scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

For Issue 6-1-6, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

Proposal 2: RAN4 NOT to define the requirements for NTN (e)RedCap in Rel-19:

    - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state

Proposal 3: RAN4 to define the requirements for NTN (e)RedCap in Rel-19:

    - NR Rel-17/18 Conditional Handover, and RACH-less Handover

    - Configured Grant based Small Data Transmissions (CG-SDT)

    - Random access based Small Data Transmissions (RA-SDT)

Proposal 4:

| RRC_Idle mode mobility and RRC_Inactive mode mobility | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, the ranking should be 1dB relaxed. |
| --- | --- |
| Handover | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse 1 extra Trs as TN 1Rx UE |
| NTN specific: - HO based RACH - RACH-less HO - Time/location-based CHO with/without L3 measurement - Satellite switching with re-sync | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, use 1 extra delay in FR1-NTN requirements |
| RLM/BFD | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse TN 1 Rx UE for hypothetical PDCCH parameters, for the measurement periods, reuse the extended measurement periods as 1Rx in TN |
| Measurement procedure: Intra-frequency measurement w/o MG Inter-frequency measurement w/o MG | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse the additional samples of 1Rx in TN in legacy FR1-NTN requirements. |
| Accuracy | For 2Rx, reuse the legacy FR1-NTN requirements For 1Rx, reuse 1 more dB relaxation as 1Rx in TN |

Proposal 5: To support RedCap/eRedCap in FR1-NTN, RAN4 should to add support eDRX in RRC_Idle/RRC_Inactive mode.

Observation 1: In eDRX_IDLE cycle, the values are extended to more than 10.24s or even larger one 10485.76s.

Observation 2: In typical LEO scenario such as R=50km and 7.56km/s for satellite speed, not all eDRX can be applicable for NGSO (LEO) scenario.

Proposal 6: RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario.

- Reference

[1] RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

[2] R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT