3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415556

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.26.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Non-Terrestrial Networks (NTN) for NR Phase 3” described in latest WID [1]:

The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In last RAN4 meeting, we analyzed the overall impacts of RRM requirements related to the above objectives and achieved some agreements.

In this contribution, we continue to discuss the RRM requirements for objective#1&objective 4 of DL coverage enhancement and regenerative payload.

- Discussion

2.1 Downlink coverage enhancements

In last RAN4#112 meeting, due to incomplete RAN1/RAN2 conclusion of SSB periodicity enhancement, the RRM impact of SSB periodicity extension is FFS and captured in WF [2] as below:

#### Issue 2-1-1: RRM impact of SSB periodicity enhancement <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC): - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a (Samsung): - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

In RAN1#118 meeting, RAN1 made the follow agreement:

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

In addition, one reply LS from RAN1 has been approved. For the SSB periodicity extension, the answer is shown as below:

According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

From the agreements above and answer in LS, there is no conclusion on final SSB extended periodicity. However, it can be observed that RAN1 is still considering to extend the SSB periodicity to larger than 20ms.

In RAN2#127 discussion,

Agreements:

1.	From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

2.	From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms.

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

4.	If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

As we pointed out in last meeting, many RRM requirements are related to SSB and SSB periodicity. In addition, SMTC is the important configuration for SSB measurement. If SSB periodicity is extended, while if SMTC/MG also have update related to SSB periodicity. Many RRM requirements are related to SSB periodicity and SMTC period/MGRP and their relationship.

Proposal 1: If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. If SMTC/MGRP also have modification due to SSB extended periodicity, RAN4 need to define such RRM requirements for this new scenario. Need further RAN1/RAN2 progress/conclusion.

In last RAN4#112 meeting, due to incomplete RAN1/RAN2 conclusion of DTX/DRX, the RRM impact of DTX/DRX is FFS and captured in WF [2] as below:

#### Issue 2-2-1: RRM impact of DTX/DRX <Way Forward> : Further discuss the following proposal - Proposal 1 (Apple): - RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES. - Proposal 2 (Samsung): - All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs. - Proposal 3 (Ericsson): - RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of DRX periodicity.

As we discussed in last meeting, RAN1 introduced “off” state for some beams. UE cannot receive DL-RS especially SSB both in idle/inactive mode and connected mode. Therefore, the measurements related to DL-RS should be impacts. In the legacy, UE has the DRX. And in Rel-18 NES, cell-level DTX has been introduced. In the NTN hopping beams, if beam level DTX will be introduced, RAN4 to discuss and define new RRM requirements. Need further progress from other WGs.

Proposal 2: All the measurements related to DL-RS should be impacts in “off” state. In addition, if beam level DTX will be introduced, RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs.

#### Issue 2-4-1: Beam switching related issues <Way Forward> : Further discuss the following proposal - Proposal 1 (Ericsson): RAN4 to check if beam steering latency other than 0 may impact RRM requirements. - Proposal 2 (Ericsson): RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements. - Proposal 3 (Ericsson): If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

Proposal 3: For other potential RRM impacts due to beam hopping, RAN4 need further RAN1/RAN2 progress for NTN hopping beams.

2.2 Support of regenerative payload

In last RAN4#112 meeting, the timing requirements for regenerative payload is FFS and captured in WF [2] as below:

#### Issue 5-2-1: Timing requirements for regenerative payload <Way Forward> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

In this meeting, there is also one LS from RAN2 in [4], the contents are shown as:

Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

As we discussed in last RAN4#112 meeting, the component of common TA is for the timing component of feeder link delay. If satellite supports regenerative payload which means the gNB function of TA should be from the satellite but not includes feeder link delay. If it is 0, there is no issue. However, it can also be a fixed value due to different implementation.

In NTN based on regenerative payload, the satellite consists of most of the gNB functions including payload, RF filtering, frequency conversion, amplification, encoding/decoding, modulation/demodulation and so on. The reference point of uplink timing requirements can be different.

In Rel-17 the TA mechanism in NTN is introduced as:

The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

Proposal 4: For regenerative payload, for the timing requirements, NTA,adjcommon can be 0 or a fixed value. For the question from RAN2, Common TA can equal to 0 or a fixed value.

In last RAN4#112 meeting, the other RRM requirements for regenerative payload is FFS and captured in WF [2] as below:

#### Issue 5-3-1: RRM requirements for RRC_INACTIVE state mobility with regenerative payload <Way Forward> - Whether to update the specific RRM requirements for inactive state with regenerative payload should wait for RAN2/RAN3 progress. #### Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload <Way Forward> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

In RAN2#127 meeting, there were some proposals such as for satellite switch, RACH-less HO etc.

In RAN#105 meeting, there is such discussion whether Satellite Switching with re-sync in a regenerative architecture can be supported in Rel-19. There is no consensus to support the satellite switch in regenerative payload in RAN#105 meeting.

Proposal 5: For support of regenerative payload, FFS on other RRM requirements such as re-establishment RACH-less HO etc, further RAN2 progress/conclusion is needed. For satellite switch, there is no consensus to support it in RAN#105.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement and regenerative payload.

Proposal 1: If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. If SMTC/MGRP also have modification due to SSB extended periodicity, RAN4 need to define such RRM requirements for this new scenario. Need further RAN1/RAN2 progress/conclusion.

Proposal 2: All the measurements related to DL-RS should be impacts in “off” state. In addition, if beam level DTX will be introduced, RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs.

Proposal 3: For other potential RRM impacts due to beam hopping, RAN4 need further RAN1/RAN2 progress for NTN hopping beams.

Proposal 4: For regenerative payload, for the timing requirements, NTA,adjcommon can be 0 or a fixed value. For the question from RAN2, Common TA can equal to 0 or a fixed value.

Proposal 5: For support of regenerative payload, FFS on other RRM requirements such as re-establishment RACH-less HO etc, further RAN2 progress/conclusion is needed. For satellite switch, there is no consensus to support it in RAN#105.

- Reference

[1] RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

[2] R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT

[3] R1-2407538, Draft Reply LS on DL coverage enhancements, RAN1

[4] R4-2414914, LS on common TA in a regenerative payload scenario, RAN2