3GPP TSG-RAN WG4 Meeting #112bis	R4-2415560

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	On XR UE OTA test

Source:	Samsung

Agenda item:	6.12.2.1

Document for:	Discussion

1.	Introduction

In previous meetings, test time reduction for XR OTA testing has been discussed. In RAN4#111 [1, R4-2409931], it was agreed that low transmission power configuration can be alternative configuration for TRS, but no discussion on TRP yet.

Issue 2-2-1: alternative test configurations for Testing time reduction solutions Agreements - RAN4 can consider low transmission power configuration (e.g., 13dBm) for XR device TRS testing if a Single Point Offset Test (SPOT) with maximum transmit power is performed and the end of the measurement to scale the TRS result.

In RAN4#112 [2, R4-2413603], more options are provided for further discussion with TRP also involved.

Issue 2-2-1: Coarse measurement grids Agreements: The following Options can be further discussed next meeting: Option 1: Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test. Option 2: Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power. Options 3: - Measure TRP: Averaging the measurement results from 4 of interpolated 62-points measurement grids by TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104) TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410 to achieve the same accuracy level of using 266-points measurement grids during OTA tests. (Define 4 of 62-points interpolated measurement grids based on 266-points measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.) - Measure Peak EIPR: Full sphere coarse girds measurement + partial fine grids measurement (For example, using 62-points measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors (pre-defined 8 or 16 sectors) of 266-points measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP)

In this contribution, we further discuss the test time reduction for XR OTA based on way forward of previous meetings.

2. 	Discussion

The discussion will be focused on the three options in above WF of last meeting.

For Option 1 (Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test), it was actually not reducing test time but increasing test time. Moreover, there would be additional measurement uncertainty due to the re-placement.

Observation 1:	Option 1 (replacement approach) not only increases test time but also brings additional measurement uncertainty due to replacement

For Option 2 (Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power), it is convenient for low-battery UE OTA testing which can avoid suspending test for battery charging. Moreover, scaling approach based on Single Point Offset Test (SPOT) has already agreed for TRS in RAN4#111 meeting.

Observation 2:	Option 2 (scaling approach) is the most convenient way for low-battery UE OTA testing and scaling approach has already been agreed for XR TRS testing.

For Option 3, those are measurement grid proposals for TRP and peak EIRP. The TRP is averaged by 4 of interpolated 62-points measurement grids. It seems that there is no fundamental difference from legacy TRP. Moreover, charging battery is still needed in between the 4 times testing with 62 grid points. So the effect of this method is quite similar as Option 1. In addition, peak EIRP is not performance metric for FR1 UE OTA.

Observation 3:	the TRP of Option 3 is similar as Option 1, and the peak EIRP of Option 3 is out of scope.

Based on above discussion, the scaling approach mentioned in Option 2 seems the best way to go. We can take option 2 as starting point, and further discuss how to do the scaling, e.g. based on the configured power parameter difference, or based on Single Point Offset Test (SPOT)

Proposal 1:	low transmission power (e.g., 13dBm) is configured for XR device TRP testing. Further discuss how to do the scaling.

3. 	Conclusion

Observation 1:	Option 1 (replacement approach) not only increases test time but also brings additional measurement uncertainty due to replacement

Observation 2:	Option 2 (scaling approach) is the most convenient way for low-battery UE OTA testing and scaling approach has already been agreed for XR TRS testing.

Observation 3:	the TRP of Option 3 is similar as Option 1, and the peak EIRP of Option 3 is out of scope.

Proposal 1:	low transmission power (e.g., 13dBm) is configured for XR device TRP testing. Further discuss how to do the scaling.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo

- R4-2413603    “WF for [112][333] TRP_TRS_MIMO_OTA”, vivo