3GPP TSG-RAN WG4 Meeting #112bis	R4-2415562

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	On HARQ parameter for FR1 dynamic MIMO OTA

Source:	Samsung

Agenda item:	6.12.2.3

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, the HARQ parameter “maximum number of HARQ transmissions” was discussed and the agreement is as following [1, R4-2409931]:

Issue 4-2-3: Test parameters for FR1 dynamic MIMO OTA Agreements: - Set the maximum number of HARQ transmissions to [x] for FR1 dynamic MIMO OTA with x>1.

In this contribution, the HARQ parameter “maximum number of HARQ transmissions” was further investigated.

2. 	Discussion

In the static MIMO OTA testing, the baseline figure of merit is the MIMO OTA throughput which is based on the number of ACKs and NACKs in the measurement time.

The MIMO OTA throughput is measured at the top of physical layer of NR system under the use of FRC, the SS transmit fixed-size payload bits to the DUT. The DUT signals back either ACK or NACK to the SS. The SS then records the following: - Number of ACKs, - Number of NACKs, and - Number of DTX slots Hence the MIMO (OTA) throughput can be calculated as Where Transmitted TBS is the Transport Block Size transmitted by the SS, which is fixed for an FRC during the measurement period. MeasurementTime is the total composed of successful slots (ACK), unsuccessful slots (NACK) and DTX-symbols.

Therefore, it is good to set maximum number of HARQ transmission as 1, i.e., no re-transmission, for the sake of conformance.

If HARQ re-transmission is enabled for dynamic OTA, the MIMO OTA throughput definition would need to be revised.

Observation 1:	If HARQ re-transmission is enabled for dynamic OTA, the MIMO OTA throughput definition would need to be revised.

When MIMO OTA comes to dynamic case, it was proposed to allow re-transmission which is similar as real network. However, dynamic MIMO OTA mainly means link adaption based on CQI, RI and PMI etc. Even in the demodulation requirements for CQI reporting, PMI reporting and RI reporting in TS 38.101-4 based on variable measurement channel, the maximum number of HARQ transmission is all set to 1, refer to the tables in clause 6 of TS 38.101-4.

It seems not aligned with what was agreed in last meeting to adopt the mapping table between CQI and MCS used for demodulation in TS 38.101-4 for dynamic MCS adaption.

Observation 2:	in the demodulation requirements for CQI reporting, PMI reporting and RI reporting in TS 38.101-4 based on variable measurement channel, the maximum number of HARQ transmission is all set to 1.

Observation 3:	we have agreed to adopt the mapping table between CQI and MCS used for demodulation in TS 38.101-4 for dynamic MCS adaption, while the maximum number of HARQ transmission is set to 1 for CQI reporting requirements in TS38.101-4.

For test cases with requirements for conformance, HARQ re-transmission will make it difficult to align the simulation/measurement results for requirements derivation. In dynamic MIMO OTA, requirements will be also be derived based on measurement campaign and all UEs need to pass the test, we’d better set the parameter with values which has least impacts to measurement campaign.

In addition, now in CTIA standard, the value of maximum number of HARQ transmission is also 1. It would be also beneficial to avoid standard fragmentation between different SDOs.

Proposal 1:	when specifying the parameter value of maximum number of HARQ transmission, RAN4 to consider the alignment with 3GPP demodulation requirements for CQI/RI/PMI in TS 38.101-4 as well as alignment with CTIA.

3. 	Conclusion

Observation 1:	If HARQ re-transmission is enabled for dynamic OTA, the MIMO OTA throughput definition would need to be revised.

Observation 2:	in the demodulation requirements for CQI reporting, PMI reporting and RI reporting in TS 38.101-4 based on variable measurement channel, the maximum number of HARQ transmission is all set to 1.

Observation 3:	we have agreed to adopt the mapping table between CQI and MCS used for demodulation in TS 38.101-4 for dynamic MCS adaption, while the maximum number of HARQ transmission is set to 1 for CQI reporting requirements in TS38.101-4.

Proposal 1:	when specifying the parameter value of maximum number of HARQ transmission, RAN4 to consider the alignment with 3GPP demodulation requirements for CQI/RI/PMI in TS 38.101-4 as well as alignment with CTIA.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo

- TS 38.151

- TS 38.101-4