3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415566

Hefei, China, 14th – 18th Oct. 2024

Source: 	Nokia

Title: 					LTE 1800 UL emissions on DL

Agenda Item: 			5.14.2

Document for:	Information

# 1	Introduction

We simulate the emissions for LTE 1800 band from UL to DL of the same band without RF filtering.

# 2	Simulation assumptions and scenario

- UL: 1800-1810 MHz

- DL: 1820-1830 MHz

- Power class PC3

- Baseband filtering

- I/Q image -25 dBc

- Carrier leakage -25 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- MPR is applied

- No RF filtering

- QPSK modulation

- DFT-s-OFDM waveform

- We place the allocation at the upper edge of the UL channel, and sweep the allocation size to find the largest emission.

- We take the maximum of the spectrum at 1 MHz MBW between 1820.5 MHz and 1829.5MHz. The outermost MBW/2 wide segments of the DL channel are excluded in order not to include emissions outside the DL channel.

# 3	Simulation results

We simulated all the allocation sizes with 10 MHz CBW, QPSK modulation and DFT-s-OFDM waveform at the upper edge of the UL channel. The worst emissions were obtained with an allocation of 40 RB. The maximum emission with 1 MHz measurement band in the DL band was -31.5 dBm. Figure 1 shows the transmit spectrum with this allocation.

Observation 1: -31.5  dBm at 1 MHz MBW was emitted from UL to DL without RF filtering.

Figure 1: The transmit spectrum. The DL channel is marked with yellow color.

# 4	Conclusions

We have simulated the RF emissions from UL to DL for the LTE 1800 band without RF filtering. Based on our simulations we have made following observation.

Observation 1: -31.5  dBm at 1 MHz MBW was emitted from UL to DL without RF filtering.