3GPP TSG-RAN WG4 Meeting #112bis	R4-2415570

Hefei, China, October 14 – 18, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for NR FR1/FR2 and EN-DC Phase 4

Agenda item:	6.1.2

Document for:	Discussion

Introduction

At RAN4#112 meeting, it was identified the RRM requirements were impacted only by 6Rx UE as captured in the WF. In this contribution, we continue the discussion on the open issues.

Discussion

One of the potential impacts on RRM requirement is the interruptions at SRS antenna switching for 6Rx capable UEs.

<Way Forward> Issue 1-4: Interruption requirements at SRS antenna switching for 6Rx capable UE

- Proposals

  - Option 1(Nokia): To wait for RAN1 conclusion on the 6Rx SRS antenna switching pattern to make sure that the legacy requirements can be reused.

  - Option 2(Huawei, Ericsson): The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

In RAN4#111, an LS R4-2406718 was sent to RAN1 to start the work on enabling 3T6R and 4T6R SRS antenna switching. It is understood 3T6R and 4T6R SRS antenna switching are still under discussion in RAN1.

Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified.

In R17, the RRM requirements for SRS antenna switching were specified in the RRM enhancement WI. During the discussion, the SRS antenna switching before R17 i.e. up to 4Rx was considered hence SRS resource(s) is assumed being configured within the last 6 symbols of a slot as cited below.

8.2.2.2.16	Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx.

Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot.

When 6Rx UEs are considered in R19, it is not clear if SRS resources are always configured within the last 6 symbols of  a slot which depends on the possible SRS switching patterns to be defined in RAN1. If SRS resources are not able to be fit within the last 6 symbols of a slot, we shall define new interruption requirements for SRS antenna switching with 6Rx UEs. As the antenna switching patterns are up to RAN1 discussion, we shall wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements.

Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion.

Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

Conclusion

In this paper we have made the following proposals and observations related to the NR FR1/FR2 EN-DC phase 4:

Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified.

Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot.

Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion.

Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

References

[1] R4-2413972, WF on RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC, Huawei