3GPP TSG-RAN WG4 Meeting #112bis	R4-2415571

Hefei, China, October 14 – 18, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for intra-band non-collocated scenarios Phase2

Agenda item:	6.5.3

Document for:	Discussion

Introduction

At RAN4#112 meeting, it was basically agreed to reuse the RRM requirement defined for Type 2 UEs for Type 4 UE as in the way forward [1]. In this contribution, we will continue discussing the open issues on RRM requirements.

Discussion

In last meeting, it was agreed to reuse the R18 MRTD/MTTD requirements defined for Type 2 UE for Type 4 capable UE in both collocated and non-collocated scenarios. One open issue is the requirement for non-collocated FR1 intra-band asynchronous NR-CA.

<Way Forward> Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 - Proposals - Option 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

We understood synchronization has been always assumed for carrier aggregation hence we didn’t differentiate between synchronous and asynchronous for NR-CA. When non-collocated scenarios are introduced for FR1 intra-band, this should not change the basic assumption for NR-CA. Hence there is no need to discuss the MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA.

Observation 1: Existing MRTD/MTTD requirements are not differentiated between synchronous and asynchronous for NR-CA.

Proposal #1: Do not discuss MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA.

For the other RRM requirements, it was not concluded if the R18 requirements for type 2 UE can be reused in collocated scenario. We understood this is similar as discussion on the UE type transition between Type 1 and Type 2 in R18. If this would be agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling. We need wait for RF conclusion on the UE type transition before changing the RRM requirements accordingly.

<Way Forward> Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied - Interruption at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions during measurements on deactivated SCC Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling. <Way Forward> Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied. Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling. <Way Forward> Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

Proposal #2: If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation.

Proposal #3: To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

Conclusion

In this paper we have made the following proposals and observations related to the intra-band non-collocated scenarios for Type 4 capable UEs:

Observation 1: Existing MRTD/MTTD requirements are not differentiated between synchronous and asynchronous for NR-CA.

Proposal #1: Do not discuss MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA.

Proposal #2: If the UE type transitions between Type 4 and other types are agreed in RF session, the RRM requirements need to be adapted based on the eventual UE type in operation.

Proposal #3: To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

References

[1] R4-2413973, WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple, RAN4#112