3GPP TSG-RAN WG4 Meeting #112bis	R4- 2415572

Hefei, China, October 14 – 18, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	6.22.2.1

Document for:	Discussion

- Introduction

At RAN4#112 meeting, the R19 WI on network energy saving was first discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- Discussion

At RAN4#112 meeting, the following has been agreed on the general aspect of OD-SSB SCell operation. We will elaborate the potential impact on the deactivated SCell measurement and SCell activation requirement in the RAN1 agreed cases and scenarios.

- Sub-topic 1-1: General OD-SSB requirements Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement. Issue 1-1-3: Deactivated SCell measurement requirement - Agreement: - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement. - Includes both FR1 and FR2-1

  - OD-SSB SCell operation in Case 1

In this subclause, we are discussing the RRM requirements for the OD-SSB SCell operation in Case 1. As shown in Fig.1,

there is only OD-SSB transmission if being triggered for the UE i.e. no other regular SSB transmission in the SCell.

Fig. 1 OD-SSB SCell operation in Case 1 and Scenario #2/#2A

OD-SSB configuration

In the latest RAN1 meeting, the following agreements were reached on the OD-SSB configuration. It is understood OD-SSB will be configured as NCD-SSB which may or may not be located on synchronization raster. Referring to Fig.1, Case 1-1 shows the OD-SSB is configured not on sync raster and in Case 1-2 OD-SSB is configured on sync raster but is an NCD-SSB.

RAN1 Agreement​ - For a cell supporting on-demand SSB SCell operation, at least the following is supported​ - On-demand SSB on the cell is not located on synchronization raster.​ - On-demand SSB on the cell is non-cell-defining SSB ​ - FFS: Additional support of OD-SSB for CD-SSB located on sync-raster​

As in Case 1, there is only OD-SSB i.e. no always-on CD-SSB available in the cell for measurement, the UE is required to measure the OD-SSB once OD-SSB transmission is triggered. This requirement is independent of the OD-SSB location on synchronization raster or not. We don’t see any impact on RRM requirements in Case 1 because of OD-SSB being located on sync raster or not.

Observation 1: According to RAN1, OD-SSB will be configured as NCD-SSB which may or may not be located on synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

Deactivated SCell measurement

In Case 1, it was agreed a UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. Meanwhile, we think the UE is required to measure OD-SSB once it is triggered in a deactivated SCell. Otherwise, the network triggered OD-SSBs would be wasted and cause inefficiency from energy saving point of view.

According to TS38.133 clause 9.2.1, a UE is always required to perform intra-frequency measurements on a serving cell including deactivated SCells. In the context of OD-SSB capable SCell in Case 1, this principal requirement would still apply but only if and once OD-SSB transmission is triggered. Hence the existing intra-frequency measurement requirement for deactivated SCell needs to be updated that it applies only if OD-SSB transmission is triggered.

- For Case 1, a UE is required to measure the OD-SSBs when they are transmitted in a deactivated SCell.

- For Case 1, intra-frequency measurement requirement for deactivated SCell needs to be updated to apply only when OD-SSB is triggered.

From last meeting, it is FFS which periodicity the UE shall follow in order to measure the OD-SSB in deactivated SCell.

Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

In this WI, OD-SSB is introduced for network energy saving by enabling reducing the transmissions of SSBs. When UE receives the OD-SSB indication, it is expected to utilize the OD-SSBs for measurement as much as possible hence the measurement based on OD-SSB periodicity i.e. Option 1 above is preferred. This helps UE with a faster cell detection, and network could stop the transmission of OD-SSB at the earliest time for power saving. On the other hand, we understand in Option2 that reusing the legacy measCycleSCell for the deactivated SCell measurement can simplify the UE implementation, but this would cause unnecessary delays (shortest measCycleScell is 160ms) and could lead to unnecessary OD-SSB transmission hence conflict with the objective of this WID. To follow the target and purpose of the WI we expect that the UE shall perform the deactivated SCell measurement based on the OD-SSB periodicity.

Observation 2: Reusing the legacy measCycleSCell for the deactivated SCell measurement would cause unnecessary delays and unnecessary OD-SSB transmission, and hence conflict with the objective of network energy saving.

- For Case 1, the UE shall perform the deactivated SCell measurement based on the OD-SSB periodicity.

SCell activation delay

Another aspect agreed to be discussed is the SCell activation delay requirement. No conclusion was reached in last meeting due to different understanding on Scenario #2A.

Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Way forward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

In our view, no matter how these scenarios are defined in RAN1, the difference from legacy SCell activation delay is that the known/unknown condition is based on the measurement reporting on OD-SSB in the deactivated SCell. OD-SSB may or may not be triggered in different scenarios, but this will determine the known/unknown conditions of the SCell and the SCell activation delay requirement shall be defined for known/unknown conditions respectively based on OD-SSB measurement reporting.

In addition, in the agreed Scenario #2 and #2A, the OD-SSB is triggered no later than the time instance when SCell activation command is received. The UE is expected to monitor OD-SSB immediately after receiving the SCell activation command. This is the same as in legacy SCell activation except OD-SSB is used instead of legacy SSBs. Therefore, exiting SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

- In Case 1, the SCell known/unknown condition shall be defined based on if the UE has sent an OD-SSB based measurement reporting within a certain time period before receiving SCell activation command (in addition to the radio conditions).

- For Case 1, RAN4 shall define the SCell activation delay for both known and unknown SCell conditions based on the OD-SSB based measurement reporting.

- Exiting SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

  - OD-SSB SCell operation in Case 2

In Case 2, it is assumed there is some regular or always-on SSBs in the SCell and OD-SSB may be triggered in addition, as shown in Fig.2.

Fig. 2 Fig. 1 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

OD-SSB/Regular SSB configuration

RAN1 agreed the regular or always-on SSB can be CD-SSB or NCD-SSB up to network implementation. Considering that OD-SSB may or may not in synchronization raster, this may lead to different scenarios as shown in Fig.2 where OD-SSBs and always-on SSBs may be on the same frequency i.e. Case 2-1, or on the different frequency i.e. Case 2-2. These frequencies may or may not be the synchronization raster.

Besides frequency domain, OD-SSB and regular SSBs may also be configured with different parameters on time domain, e.g. different periodicity, different time offset. In our view, it would be more energy efficient to assume the same time offset and different periodicity between OD-SSB and regular SSBs. While the SSB configuration is up to network, RAN4 could start the discussion from the two cases as in Fig.2 in order to identify the impact on RRM requirements.

Observation 3: OD-SSB and regular SSBs may be on the same or different frequencies in a SCell.

- RAN4 to start the Case 2 discussion assuming OD-SSB and regular SSB have the same time offset and different periodicity on the same or different frequencies.

Deactivated SCell measurement

As the regular SSB is available in Case 2, a UE is able to perform the legacy SSB-based measurement based on the regular SSBs. When the SCell is in deactivated state, the UE is required to measure the regular SSBs before OD-SSB is triggered. Hence the existing UE intra-frequency measurement requirements for deactivated SCell shall apply before OD-SSB is triggered on the SCell.

- For Case 2, a UE is required to perform the SSB-based intra-frequency measurements based on regular SSB before OD-SSB is triggered.

- For Case 2, the existing intra-frequency measurement requirement for deactivated SCell shall apply before OD-SSB is triggered on the SCell.

When OD-SSB is triggered on a deactivated SCell, it needs to be discussed how a UE would perform the measurements while both OD-SSB and regular SSBs are present. In our view, the UE shall measure at least OD-SSB if it is triggered on the deactivated SCell. Otherwise, the network would have not triggered the OD-SSBs for network energy saving. It can be further discussed if the measurements on regular SSBs are still required.

If it is agreeable that UE is required to measure OD-SSB once it is triggered on deactivated SCell, we need further clarify how the legacy measurements based on regular SSB is handled. This may depend on the cases in Fig.2:

- If the regular SSBs are on the same frequency with OD-SSB e.g. Case 2-1 in Fig.2, we understood the regular SSBs will fully overlap with some of the OD-SSBs assuming the same time offset. Hence the measurement on OD-SSB will cover the regular SSBs i.e. the regular SSB based measurement is not needed.

- Otherwise, if the regular SSB is not on the same frequency with OD-SSB e.g. Case 2-2 in Fig.2, it needs to be discussed if UE is required to measure both OD-SSB and regular SSBs in the SCell.

- For Case 2, a UE is required to measure at least OD-SSB when OD-SSB transmission is triggered on the deactivated SCell.

- For Case 2, if OD-SSB and regular SSB are not on the same frequency, RAN4 to discuss if UE is required to measure both SSBs in the deactivated SCell.

SCell activation delay

In Case 2, the UE may be able to send the measurement reporting based on either regular SSB or OD-SSB. The known/unknown condition needs to consider the availability of any of the measurement reporting for the deactivated SCell. Hence the SCell shall be considered as known if the UE has sent a measurement reporting for either regular SSB or OD-SSB on the deactivated SCell.

When receiving the SCell activation command, UE will perform the SCell activation procedure based on if OD-SSB is triggered at the time instance T2. In our view, OD-SSB shall be used for SCell activation if available in order to reduce the SCell activation delay e.g. scenario 2A. Otherwise, if OD-SSB is not triggered at the reception of SCell activation command, the regular SSB shall be used for SCell activation.

- For Case 2, the SCell shall be considered as known if the UE has sent a measurement report for either regular SSB or OD-SSB on the deactivated SCell.

- For Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB if OD-SSB is triggered at the time instance when receiving the SCell activation command.

  - Others

In RAN1#118 meeting, it was further concluded to at least support L1 measurement based on OD-SSBs. In our understanding, UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated. This implies the OD-SSB will be triggered after SCell activation procedure is completed hence leads to scenario #3B. However, the scenarios other than scenario #2 and #2A were not concluded explicitly in RAN1.

We would like to wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

Agreements (RAN1#118) [Signalling] Agreement​ Update the previous RAN1 agreement as follows.​ - At least support L1 measurement based on on-demand SSB ​ - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.​ - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB​ - The support of LTM is a separate discussion point​

Observation 4: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated i.e. scenario #3B.

- Wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

Observation 1: According to RAN1, OD-SSB will be configured as NCD-SSB which may or may not be located on synchronization raster.

- For Case 1, there is no impact on RRM requirements because of OD-SSB being located on the sync raster or not.

- For Case 1, a UE is required to measure the OD-SSBs when they are transmitted in a deactivated SCell.

- For Case 1, intra-frequency measurement requirement for deactivated SCell needs to be updated to apply only when OD-SSB is triggered.

Observation 2: Reusing the legacy measCycleSCell for the deactivated SCell measurement would cause unnecessary delays and unnecessary OD-SSB transmission, and hence conflict with the objective of network energy saving.

- For Case 1, the UE shall perform the deactivated SCell measurement based on the OD-SSB periodicity.

- In Case 1, the SCell known/unknown condition shall be defined based on if the UE has sent an OD-SSB based measurement reporting within a certain time period before receiving SCell activation command (in addition to the radio conditions).

- For Case 1, RAN4 shall define the SCell activation delay for both known and unknown SCell conditions based on the OD-SSB based measurement reporting.

- Exiting SCell activation delay requirement shall apply except OD-SSB is used instead of legacy SSBs.

Observation 3: OD-SSB and regular SSBs may be on the same or different frequencies in a SCell.

- RAN4 to start the Case 2 discussion assuming OD-SSB and regular SSB have the same time offset and different periodicity on the same or different frequencies.

- For Case 2, a UE is required to perform the SSB-based intra-frequency measurements based on regular SSB before OD-SSB is triggered.

- For Case 2, the existing intra-frequency measurement requirement for deactivated SCell shall apply before OD-SSB is triggered on the SCell.

- For Case 2, a UE is required to measure at least OD-SSB when OD-SSB transmission is triggered on the deactivated SCell.

- For Case 2, if OD-SSB and regular SSB are not on the same frequency, RAN4 to discuss if UE is required to measure both SSBs in the deactivated SCell.

- For Case 2, the SCell shall be considered as known if the UE has sent a measurement report for either regular SSB or OD-SSB on the deactivated SCell.

- For Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB if OD-SSB is triggered at the time instance when receiving the SCell activation command.

Observation 4: A UE is required to perform only L3 measurements on a deactivated SCell i.e. L1 measurement would happen only after SCell is activated i.e. scenario #3B.

- Wait for further RAN1 clarification on the applicable scenarios for L1 measurement before discussing the L1 measurement requirements based on OD-SSB in RAN4.

- References

[1]	R4-2413885, “WF on Netw_Energy_NR_enh”, Ericsson