3GPP TSG-RAN WG4 Meeting #112bis	R4-2415574

Hefei, China, Oct 14 – 18, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	6.15.2.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. In this contribution we continue the discussion on CSSF optimization based on the agreed way forward [1].

Discussion

WID clarification

At RAN#105 meeting, the RRM Phase 5 WID was revised to clarify the applicability of CSSF optimization [2]. It is now indicated that CSSF optimization is defined assuming a UE is not operating multi-Rx based L3 measurements. While this makes clear the dependency on multi-Rx “L3” measurements, we still see some ambiguity on the expected UE behaviour when CSSF optimization is enabled.

#### 4.1 Objective of SI or Core part WI or Testing part WI - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, iItif feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

In our understanding, UEs are already allowed and may perform L3 measurements using multiple panels in parallel in their implementation. Such implementation is not prohibited but the UE requirements are based on the assumption of only requiring the UE using one panel. As the UE implementation behavior is unknown to network, network does not know if the UE is operating using multi-Rx for performing L3 measurements, and hence is not able to determine if the optimized CSSF would apply. In our view, the UE implementation or support of multi-Rx based L3 measurement (if available) shall not impact the applicability of CSSF optimization. In other words, a UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

Network does not know if the UE is operating multi-Rx based L3 measurement in its implementation, hence network is not able to determine if the optimized CSSF would apply.

A UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

In addition, a UE is understood to be able to operate with the multi-Rx L3 measurement under certain conditions. According to the revised WID, the UE may have to adjust the measurement behavior due to applying CSSF optimization or not based on if the multi-Rx L3 measurement is applicable (triggered) or not. To simplify the discussion, if a UE is configured with CA/DC, it can be assumed not operating with the multi-Rx based L3 measurement. Since the two topics are being discussed separately, the dependency in-between shall not impact the discussion on CSSF optimization solutions.

A UE may have to adjust the measurement behavior due to applicability of CSSF optimization based on if the multi-Rx L3 measurement is applied or not.

However, currently it is not clear when UE is required to apply mRx for L3 measurements and hence it is open when UE shall apply the CSSF optimization. If RAN4 can agree (which we believe was the initial intention) that if the UE is configured with CA or DC, the UE configured with CSSF optimization, applies CSSF optimization. When CSSF optimization is applied, the UE is not required to use mRx L3 measurements.

If a UE is configured with CSSF optimization (CA or DC), the UE is not required to operate with the multi-Rx based L3 measurement.

The independency between CSSF optimization and multi-Rx L3 measurement shall be clear. This would allow the discussion on the CSSF optimization solutions to proceed without impact from the multi-Rx L3 measurement discussion.

Measurement procedures

In last meeting, all the measurement procedures were agreed to be discussed. As the CSSF optimization implies new UE measurement behavior, it would be easier to move forward focusing on very basic measurement procedures. We prefer starting from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the discussion. The applicability to other measurement procedures can be revisited after the baseline solution is concluded.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

Searcher assumption

In the WID, it indicates the baseline assumption on number of searchers is 2 which is aligned with current assumption for deriving CSSF values. We understood the searcher assumption is up to UE capability, but the CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1: Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2: RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

The CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

Solutions for CSSF optimization

Some candidate solutions have been proposed for CSSF optimization. The most popular one is to have UE measure only one serving CC per band if there are multiple serving CCs in the same band i.e. Option 1 as listed below.

Issue 2-2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC [Way forward]: FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

According to the requirements for FR2 measurement in TS 38.133 clause 9.2.3.2, the UE is required to perform a full measurement on a PCC/PSCC or one of the SCCs. While on the other intra-frequency layers in the same band, a relaxed measurement is allowed as the UE is required to measure 2 SSBs on serving cell i.e. the measurement on neighbor cells on the SCC may be skipped.

TS38.133:

#### 9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

In existing specification, UE is allowed to skip the neighbour cell measurements on some SCCs in the same band, but still required to measure all the SCCs configured with serving cell measurements.

- In Option 1, it intends to apply the same measurement relaxation i.e. to allow the UE skip some of the serving cell measurements in order to reduce the measurement delay. We believe there is potential to minimize the measurements on multiple CCs in some scenarios e.g. FR2 intra-band, but the UE measurement behavior needs to be under network control as the measurement results are used for network decision on mobility etc. Network needs to know exactly which CCs the UE has measured and which CCs the UE does not measure.

RAN4 to discuss the solution to further reduce the measurement requirements on serving CCs (on one FR2 band) for CSSF optimization.

The measurement behavior for CSSF optimization needs to be under network control.

As for now, the measurements on serving cells are up to network configuration. According to TS 38.331 clause 5.5, the measurements on serving cells are configured by the network via measConfig and servingCellMO wherein the configuration of servingCellMO is mandatory for PCell/PSCell but optional for SCells. In particular, if a UE is configured with servingCellMO on a serving cell, it is required to measure the associated measObjectID as indicated in measConfig. Otherwise, the UE is not required to measure on the cell. Therefore, the candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

TS38.331 clause 5.5:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

The measurements on serving cells are configured by the network via measConfig and servingCellMO wherein the configuration of servingCellMO is optional for SCells.

The candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

While existing measurement framework can already be used to minimize the UE measurements on multiple CCs, the UE needs to understand this is for CSSF optimization and the measurement result can be reused for other CCs not measured. This is different from the legacy behavior when UE receives servingCellMO on single CC, where the other CCs are assumed not measured hence no measurement result. Therefore, additional signaling is expected from network to enable the measurement behavior due to CSSF optimization.

To enable the reduced measurement behavior, due to CSSF optimization, explicit signaling assistance is needed from network.

When the UE is configured to measure single CC per band, network has assumed that the measurement results can be reused among CCs in the same band. This enables a minimized CSSF value and hence a reduced measurement delay.  But the UE is not able to trigger the measurement reporting for a CC which the UE is not measuring.

Since any network action is likely based on the assistance measurement reporting from the UE including the potential target cell, and the UE requirements are conditioned on UE having sent a measurement report for the target cell, there is benefit in having UE being triggering the measurement reporting for the carriers not (being) measured or not configured with MOs. In order to ensure the mobility performance and minimize the impact to network behaviour, RAN4 shall discuss how to handle the measurement reporting if the UE is not configured with measurements on the CCs.

The UE is not able to trigger a measurement reporting for a CC which the UE is not measuring.

RAN4 to discuss how to handle measurement reporting for a CC which the UE is measuring.

For the measurements on SCCs, UE has been assumed to share one searcher and perform the measurements over multiple SCCs in alternating manner. This leads an increased measurement delay being scaled by the number of CCs to be measured. However, the UE is able to receive simultaneously from the serving CCs e.g. in CA operation, hence it is able to receive SSBs from multiple configured CCs at the same time e.g. within one SMTC window if it is configured with same SMTC on the multiple CCs. This will allow the UE to get the SSB samplings from the multiple CCs in one shot. Afterwards, the UE may process the samples in sequential way offline in order to derive the measurement results for the multiple CCs respectively.

With such measurement behaviour, some additional processing time may be needed but the measurement delay is not scaled by the number of configured CCs hence can be significantly decreased. We would like to define the optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

In addition, the CSSF values are determined considering not only SSB-based measurement but also the CSI-RS based L3 measurements. Although the WID targets SSB based L3 measurement delay reduction, the CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS i.e. twice of the number of SCells configured with CSI-RS based measurement. This is based on the R16 assumption that one CSI-RS based measurement requires an additional measurement on its associated SSB.

The CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS, i.e. twice of the number of SCells configured with CSI-RS based L3 measurement.

However, the CSI-RS resources to be measured are up to network configuration which may not be overlapping with the associated SSB. In some cases, the CSI-RS based measurements may not impact the SSB-based measurements e.g. if they are not overlapping in time, hence the CSSF values can be further optimized. We may study the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

Capability issues

In current specification, CSSF values have been derived to scale the measurement delay requirements when UE is configured to monitor multiple measurement objects. When CSSF optimization is considered in R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution e.g. measuring single CC per band. We believe this shall be applied to the UE starting from release 19.

In R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution.

Meanwhile, network may also use the CSSF values to estimate the UE measurement behaviour and make network decision. When reduced CSSF is applied, the network behaviour may need to be adapted accordingly. It is not possible to apply new network behaviour without knowing about the UE’s support of the reduced CSSF. The UE capability is needed to inform network the support of reduced CSSF.

When reduced CSSF is applied, network behaviour may need to be adapted based on the UE indication of its support of the reduced CSSF.

A UE capability is needed to inform network the support of CSSF optimization.

Conclusion

In this paper the following observations and proposals were made:

- Network does not know if the UE is operating multi-Rx based L3 measurement in its implementation, hence network is not able to determine if the optimized CSSF would apply.

- A UE is assumed not operating with multi-Rx based L3 measurement unless it is known by network.

A UE may have to adjust the measurement behavior due to applicability of CSSF optimization based on if the multi-Rx L3 measurement is applied or not.

If a UE is configured with CSSF optimization (CA or DC), the UE is not required to operate with the multi-Rx based L3 measurement.

The independency between CSSF optimization and multi-Rx L3 measurement shall be clear. This would allow the discussion on the CSSF optimization solutions to proceed without impact from the multi-Rx L3 measurement discussion.

To start from SSB-based intra-frequency and inter-frequency measurements without MG to simplify the CSSF optimization discussion.

The CSSF optimization solution we are defining shall be applicable at least for 2 searchers UEs.

In existing specification, UE is allowed to skip the neighbour cell measurements on some SCCs in the same band, but still required to measure all the SCCs configured with serving cell measurements.

RAN4 to discuss the solution to further reduce the measurement requirements on serving CCs (on one FR2 band) for CSSF optimization.

The measurement behavior for CSSF optimization needs to be under network control.

The measurements on serving cells are configured by the network via measConfig and servingCellMO wherein the configuration of servingCellMO is optional for SCells.

The candidate solution of Option 1 can be implemented via network configuration e.g. to configure the UE with servingCellMO only on one CC per band and no MOs are configured on the other CCs, if there are multiple CCs on the same band.

To enable the reduced measurement behavior, due to CSSF optimization, explicit signaling assistance is needed from network.

The UE is not able to trigger a measurement reporting for a CC which the UE is not measuring.

RAN4 to discuss how to handle measurement reporting for a CC which the UE is measuring.

UE is able to receive SSBs (i.e. get SSB samplings) from multiple configured CCs at the same time e.g. within one SMTC window.

RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

The CSI-RS based L3 measurements contribute to the CSSF values via 2xNSCC_CSIRS, i.e. twice of the number of SCells configured with CSI-RS based L3 measurement.

To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

In R19, a reduced CSSF implies some new UE measurement behaviour pending on the CSSF optimization solution.

When reduced CSSF is applied, network behaviour may need to be adapted based on the UE indication of its support of the reduced CSSF.

A UE capability is needed to inform network the support of CSSF optimization.

- References

- R4-2414026, Way Forward for NR_RRM_Ph5_Part1, Apple

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5