3GPP TSG-RAN WG4 Meeting #112bis                           R4-2415575

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:              Discussion on fast SCell activation for UE supporting eEMR

Agenda item:			6.15.3

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some initial discussions on fast SCell activation based on eEMR. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Issue 1-1-2: Applicability of fast SCell activation delay requirements

In Rel-18, new features in terms of validity check of IDLE/INACTIVE measurement results were introduced, such that NW is capable of utilizing the reported earlier but valid idle mode measurements to enable fast configuration and CA/DC setup reliably. And, the feasibility of using EMR and cell-reselection measurements to validate IDLE/INACTIVE measurement was verified. The measurement report requirements defined for Rel-18 EMR can be found in the current TS 38.133 Section 4.7.3, duplicated as below

### 4.7.3 Measurement Report Requirements …[Omit]… If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

In last meeting, whether the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured shall be included or not is controversial. From our understanding,

- No need to include the case when measIdleValidityDuration-r18 is not configured. If we take a look at the timer T331 behaviour specified in TS 38.331, and the agreements regarding the UE behaviours after T331 expiry reached in RAN2 EMR, duplicated as below

Behavior of timer T331 Timer; Start; Stop; At expiry. T331; Upon receiving RRCRelease message with measIdleDuration; Upon receiving RRCSetup, RRCResume, RRCRelease with idle/inactive measurement configuration, upon cell selection/reselection to a cell that does not belong to the validityArea (if configured), or upon cell re-selection to another RAT.; Perform the actions as specified in 5.7.8.3.. ### 5.7.8.3 T331 expiry or stop The UE shall: 1> if T331 expires or is stopped: 2> release the VarMeasIdleConfig. NOTE: It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped. Agreements: - Upon the expiry of T331 while in IDLE or INACTIVE mode, the UE deletes the dedicated idle mode measurement configuration, if any. - The UE deletes the early measurement results after it has successfully reported them to the network (i.e. in UEInformationResponse or RRCResumeComplete).

We can find that for the legacy EMR 1) Since VarMeasIdleReport is not deleted, if there is no storage problem, the UE may store the measurement results performed during T331 and report the results (e.g., latest measured value) with satisfaction of measurement accuracy requirement at the measurement instance; 2) Since it is UE implementation to continue IDLE/INACTIVE measurements according to measurement configuration provided in SIB after T331 expiry, it is possible for the “clever” UE to perform additional measurement based on SIB information and report the results at the measurement instance, for which the accuracy requirement of the measurement is still be guaranteed. Essentially, the UE behaviors in both bullets align the ones defined for the case if the measIdleValidityDuration-r18 is not configured. Hence, in our opinion, it is inappropriate to classify the case “when measIdleValidityDuration-r18 is not configured” into Rel-18 EMR scope.

- Leave the case when measReselectionValidityDuration-r18 are not configured for further study. Since in Rel-18 eEMR, cell-reselection is taken as a possible measurement in idle mode, the latest result for overlapped carrier/cells configured for cell-reselection is included, and the related new fields are also specified in TS 38.331, the corresponding UE behaviour is new and can belongs to Rel-18 EMR scope.

Proposal 1:

- There is no need to include the case when measIdleValidityDuration-r18 is not configured

- Prefer to include the case when measReselectionValidityDuration-r18 is configured

2.2 Issue 1-1-4: Target scenarios of fast SCell activation

Have checked the scenarios discussed in Rel-18 EMR WI and the corresponding test cases, we found that actually FR1 and FR2 cases are both contained.

#106 - Agreement RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios.

Hence, we’d better to inherit the agreed scenarios including FR1 and FR2-1. However, based on the Tactivation_time defined in RAN4 for FR2 intra-band scenario summarized in our last TDoc, we have the following observation:

Observation 1: For FR2 intra-band scenario, if the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, there is no extra room to reduce the SCell activation delay even the UE supports Rel-18 EMR

Besides, from our understanding, it is reasonable to focus on the following cases in FR1 and FR2:

- SCell being activated belongs to FR2.

  - The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1.

- The SCell being activated belongs to FR1.

Proposal 2: For the target scenarios of fast SCell activation, suggest to focus on the following:

- The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

- The SCell being activated belongs to FR1

2.3 Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting

Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

2.3.1 How to update the known condition

Considering Direct SCell Activation at SCell addition case, the existing known cell condition for FR1&FR2 cells are captured as below, obviously, no matter for FR1 or FR2, three communal aspects are included in the the known codition : 1) Period, during the period equal to [X]; 2) Reporting condition;3) Detected cell condition. And for FR2, additonal TCI activation condition is considered, the corresponding aspects are bold below.

The SCell in FR1 is known provided the following conditions are met for the SCell: - During the period: [last 5 seconds] before the reception of the direct SCell configuration command: - Reporting condition: [the UE has sent a valid measurement report for the SCell being directly activated], and - Detected cell condition: [the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3], - Detected cell condition: [the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3]. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and period: [3s for UE supporting power class 2/3/4] before TCI activation condition: [UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable)]: - Reporting condition: [the UE has sent a valid L3-RSRP measurement report with SSB index], and - TCI & SCell activation condition: SCell activation command is received after L3-RSRP reporting and no later than the time [when UE receives MAC-CE command for TCI activation] - Detected cell condition: [During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes].

In this sense, Issue 1-2-2“Consideration on SINR condition during the whole procedure for known SCell activation”can be brought here to discuss together, since such issue is related to Detected cell condition.

- Reporting condition.

  - The valid measurement report via eEMR for the concern cell should be included, otherwise, it is possible that the UE has detected a certain detecable cell but didnt send any valid measurement results due to none of the eEMR based reporting condition is met during the connected mode

  - For FR2, the SSB index shall be superimposed. Since it can not be ensured that the UE has already obtained the full timing information according to“the UE has sent a valid L3-RSRP measurement report via eEMR”purely.

Observation 2: The valid measurement report via eEMR for the concern cell should be included, and for FR2, the SSB index shall be superimposed.

- Detected cell condition.

  - Obviously, the SINR side condition is inconsistent in connected and IDLE modes. In connected mode, it is provided that Ês/Iot ≥ -2dB, while in IDLE mode is -4 dB. However, from our understanding,

    - Only the SCell candidates with acceptable SINR are expected to be of interest for the early reporting, the single measurement shall be performed on such detected SCells and able to provide satisfied accuracy accordingly (the missed detection probability is low enough above -4 dB).

    - SINR side condition shall to ensure that the SCell to be activated can be detectable over all the activation procedure

  - In order to make the SINR side condition consistent, a simple and reasonable way is to assume that the detected SCells in connected mode would also be assumed to be a detected cell after UE transitioning to IDLE mode

    - Based on the assumption, the UE would only be required to measure SCells in IDLE mode which are already detected cells in connected mode

Proposal 3: In order to make the SINR side condition consistent, it is reasonable to assume that the detected SCells in connected mode would also be assumed to be a detected cell after UE transitioning to IDLE mode

- Period.

  - From our unstanding, the main intention to define the period could be to ensure a SCell which is known in connected mode remains known also in idle mode assuming the SCell has been measured within the X seconds

  - [5] seconds for FR1.

    - The intuition behind this definition is timing jitter within 5s is still tolerable.

    - Based on the NR IDLE mode measurement period requirements defined in Section 4.2.2, the maximum value of Tmeasure,NR is 2.56 x N1 (1 x N1)

    - It can be noted that the maximum interval between the two adjacent measurements for SCell is also less than [5]s, that means based on the new and exact definition of known, the valid timing can be always guaranteed between the two adjacent measurements of SCell.

    - [5] s is also the candidate value for validity check timer X

  - [3] seconds for FR2 for PC3.

    - Assuming the Tx beam is unchanged during X period;The Tx beam remains detectable during X period

    - The measurement is performed within validity check timer []s before reporting, the measurement result is valid

  - In this sense, we have the following concerns on:

    - 1) If we extend the X=[3]/[5] s to longer period, i.e., X>[3]/[5] s, whether the sync information (/cell’s timing information) stored is still valid in connected mode;

    - 2) If we shorten the period for FR1, whether it is difficult to use potential SCell candidates obtained from eEMR efficiently or provide UE implementation limitation

Observation 3: Longer period may cause the cell’s timing information stored is invalid in connected mode; Shorter period may cause difficulty to use potential SCell candidates obtained from eEMR efficiently and limit UE flexibility

Proposal 4: For the known condition on during the period [X] s

- For FR1, suggest to use X=[5] s as baseline

- For FR2 PC3, suggest to use X=[3] s as baseline

- Additional eEMR-specific SCell activation indication.

From our understanding, it is possible and feasible to define an additional eEMR-specific SCell activation indication to clearly represent the case is SCell quick activation based on eEMR, and the MAC-CE command could be provided together with MAC-CE for SCell activation

Proposal 5: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication

- The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation

From our understand, based on the new known conditions, the cell information becomes visible for UE, there is no need to perform cell detection and AGC gain getting. In summary, the possible way to define SCell known conditions (Direct SCell Activation at SCell addition case) can be:

Proposal 6:

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last [5]seconds before the reception of the direct SCell configuration command:

-	the UE has sent a [valid measurement report via Rel-18 EMR] for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

-	[the SSB measured during the period equal to [5] seconds also remains detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

Otherwise, the SCell is unknown.

Note: FFS Whether to modify the existing 4.4.2.1

Proposal 7:

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a [valid L3-RSRP measurement report via Rel-18 EMR with SSB index], and

-	[Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from [L3-RSRP reporting via Rel-18 EMR ]to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

-	[The reported SSBs with indexes remain detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

Note: FFS Whether to modify the existing 4.4.2.1

2.3.2 The applicability of valid EMR reporting for the Rel-19 fast SCell activation

From our understanding, the applicablity of valid EMR reporting for the Rel-19 fast SCell activation requirement is for known cell (e.g., measured within the last [5]s for FR1) in connected mode. That is to say, a cell which is known in connected mode would remain also known in idle mode assuming the same conditions—the cell has been measured within the [X] seconds. Based on the assumption, UE would need to measure the known cell in idle mode and report the results of such detected cells quickly, UE not always need to perform cell detection when transitioning to idle mode and the undesirable UE latency in idle mode can be reduced, the active RF may be known in time domain, the already known timing can be re-utilized, and the lowest probability of network configuring a less suitable SCell when entering connected mode can be provided

Proposal 8: The applicablity of valid EMR reporting for the Rel-19 fast SCell activation requirement is for known cell only

2.4 Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

The 2400 ms is L3 measurement period, 2400= number of sample (5)* 160 ms* CSSF factor (3). The background of 160 ms is that if the latest measurement sample is more than 160ms ago, then UE may need additional time for AGC setting. However, the measurement type of eEMR is not periodic

Observation 4: Single measurement is defined for the eEMR of the SCell candidates

And in idle mode, no CSSF factor is considered.

From this, if 2400 ms can be changed based on valid eEMR results, we wonder what’s the components and whether it will change the gate regarding when need to do AGC training for activation using Trs.

Proposal 9: If the SCell is known based on the eEMR based known condition and belongs to FR1, if 2400 ms can be modified to Y ms

- The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

2.5 Issue 1-2-4: Whether the indication to network is needed

For the time being, the necessity of introducing the indication to NW for us is not clear, since in the previous discussions, no related indication was introduced including in euCA, and CQI feedback can be deemed as the end of activation procedure. Anyway NW can carry out the quick SCell scheduling based on the instant when the accurate CQI sent by the UE if the Tactivation_time can be shorten based on eEMR.

Proposal 10: Prefer not to introduce certain activation fast or slow indication to the network.

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 fast SCell activation based on eEMR. The following observations and proposals are obtained:

Observation 1: For FR2 intra-band scenario, if the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, there is no extra room to reduce the SCell activation delay even the UE supports Rel-18 EMR

Observation 2: The valid measurement report via eEMR for the concern cell should be included, and for FR2, the SSB index shall be superimposed.

Observation 3: Longer period may cause the cell’s timing information stored is invalid in connected mode; Shorter period may cause difficulty to use potential SCell candidates obtained from eEMR efficiently and limit UE flexibility

Observation 4: Single measurement is defined for the eEMR of the SCell candidates

Proposal 1:

- There is no need to include the case when measIdleValidityDuration-r18 is not configured

- Prefer to include the case when measReselectionValidityDuration-r18 is configured

Proposal 2: For the target scenarios of fast SCell activation, suggest to focus on the following:

- The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1

- The SCell being activated belongs to FR1

Proposal 3: In order to make the SINR side condition consistent, it is reasonable to assume that the detected SCells in connected mode would also be assumed to be a detected cell after UE transitioning to IDLE mode

Proposal 4: For the known condition on during the period [X] s

- For FR1, suggest to use X=[5] s as baseline

- For FR2 PC3, suggest to use X=[3] s as baseline

Proposal 5: It is feasible to introduce a new dedicated eEMR based fast SCell activation indication

- The corresponding MAC-CE command could be provided together with MAC-CE for SCell activation

Proposal 6:

The SCell in FR1 is known provided the following conditions are met for the SCell:

-	During the last [5]seconds before the reception of the direct SCell configuration command:

-	the UE has sent a [valid measurement report via Rel-18 EMR] for the SCell being directly activated, and

-	the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3,

-	the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

-	[the SSB measured during the period equal to [5] seconds also remains detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

Otherwise, the SCell is unknown.

Note: FFS Whether to modify the existing 4.4.2.1

Proposal 7:

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to [4]s for UE supporting power class 1/5 and [3]s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a [valid L3-RSRP measurement report via Rel-18 EMR with SSB index], and

-	[Rel-18 EMR based SCell fast activation command] is received [after L3-RSRP reporting via Rel-18 EMR] and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from [L3-RSRP reporting via Rel-18 EMR ]to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

-	[The reported SSBs with indexes remain detectable after UE has entered Idle/Inactive mode and during Idle/Inactive mode specified in clause 4.4.2.1]

Note: FFS Whether to modify the existing 4.4.2.1

Proposal 8: The applicablity of valid EMR reporting for the Rel-19 fast SCell activation requirement is for known cell only

Proposal 9: If the SCell is known based on the eEMR based known condition and belongs to FR1, if 2400 ms can be modified to Y ms

- The basic components to define Y and the subsequent impact on AGC training gate for activation shall be discussed.

Proposal 10: Prefer not to introduce certain activation fast or slow indication to the network.

4 References

[1] R4-2414041, “WF on NR_RRM_Ph5_Part2 - fast SCell activation”, Apple