3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2415576

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:               General discussion on AI mobility regarding testability and interoperability

Agenda item:			6.18.3

Document for:	Discussion

1 Introduction

In RAN#102 meeting, the study on AI/ML for mobility in NR was approved, and in the subsequent RAN#105 meeting, the SID was revised. In this paper, we would like to give some initial discussions on the second topic on AI mobility regarding testability and interoperability

2 Discussion

2.1 Generalization / adaptability of the models

AI Model generalization is a very important and challenging issue in deep learning, which refers to the ability of the trained model to perform well on new inputs unseen during the training. From our understanding, compared to the PHY prediction, the generalization/adaptability of the AI models for mobility is more worth studying. The reasons are:

- For AI mobility, various deployment scenarios (e.g., UMa, UMi, etc.), channel environments, UE movement patterns and speeds, and frequencies exist. Meanwhile, different RRC configurations and conditions for measurement (events) need to be considered in practice. All of them will impact generalization performance of an AI/ML model.

- Multiple cells would be considered in L3 AI mobility WI. While, across different cells may capture variations in NW conditions and local radio conditions. In this sense, the risk of model overfitting to specific cell characteristics may arise, which may decrease the adaptability to different cells

- Since the prediction measurement results are L3 filtered. Different L3 filtering factors may incur different prediction accuracy. While, when training datasets collected with fixed L3 filtering factor, the corresponding AI models may have worse generalization capability in different cells, especially.

- The measurement event prediction may benefit of cell-, site- or area-specific models and the study on AI model generalization for measurement event prediction is needed

In summary, it is more important and necessary to verify whether the AI/ML model trained in one communication case can be implemented in a different one directly, i.e., without re-training under mobility environment. Solving such issues can decrease the number of models deployed and increase adaptability and robustness in diverse and realistic radio communication scenarios.

Proposal 1: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Follow the conclusions on generalization achieved in AI/ML for NR air interface as much as possible

- Study depends on sufficient research progress in RAN2

2.2 LCM aspects

An AI/ML model needs to be developed, deployed, and managed during the entire lifecycle. Taking a look at the 3GPP AI/ML functional framework for the NR air interface, below, we can find that 1) Core functions of the framework contain: data collection, model training, management, inference, and model storage; 2) The AI/ML model LCM procedures need the following model operations: Data collection, Model monitoring, Model selection/deactivation/switching/fallback, Model inference, Model training/update, Model transfer/delivery and Model storage. Besides, there are two distinct methods for managing LCM: functionality-based LCM and model identity based-LCM. Both of them are considered in Rel-18 AI/ML RRM.

Fig 1. 3GPP AI/ML functional framework for the NR air interface.

The overall procedure of AI/ML model LCM was discussed in Rel-18 AI/ML BM RAN1, and potential AI mobility specific enhancement, e.g., LCM was agreed in RAN2 scope.

In Rel-18 AI/ML for NR air interface, the necessity and feasibility of defining requirements/tests for LCM was extensively discussed and confirmed. And during the discussions, the agreements on LCM related requirements/functional tests were achieved sequentially. Based on the WFs agreed in Rel-18 [1], the following LCM related requirements are agreed to define based on the specific BM and positioning use cases:

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

And in the latest meeting, companies agreed that the LCM would be tested [2].

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

From this, although the above LCM related requirements and functional tests are AI/ML PHY use cases specific, the aim, necessity, feasibility and even the principle of defining the LCM related requirements and functional tests should be inherited to AI/ML mobility use cases.

Proposal 2: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

- RAN4 study should be aligned with the agreements in RAN1 and RAN2

2.3 AI/ML functionalities for RAN4 test

Theoretically, based on Fig.1, testability issues for all the basic functionalities, i.e., Data collection, Model training, Model inference, Model monitoring can be studied in RAN4 RRM. While, as a starting point, we suggest to prioritize Model inference test verification.

Proposal 3: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case.

- Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality.

From our understanding, the testing can be taken as a popular technique for ensuring the reliability of AI model.

- RRM measurement prediction

The goals agreed in RAN2 for the whole evaluation scenarios are duplicated as below

| No. | Prioritization | Evaluation scenario combination | Deployment of the cell for measurement and the cell for prediction | Target study goal |
| --- | --- | --- | --- | --- |
| 1 | High | FR1 to FR1 intra-frequency intra-cell temporal domain case B (i.e., Intra_F_C_T_Case B) |  | 1st goal |
| 2 | High | FR1 to FR1 inter-frequency inter-cell (frequency domain) (i.e., Inter_F_C ) | - Co-located (Start from) - Same sector | 1st goal |
| 3 | High | FR2 to FR2 intra-frequency intra-cell temporal domain case A (i.e., Intra_F_C_T_Case A) |  | 2nd goal |
| 4 | Middle | FR2 to FR2 intra-frequency intra-cell spatial domain |  | 1st goal |
| 5 | Low | FR1 to FR1 intra-frequency TBD temporal domain case A |  | 2nd goal |
| 6 | Low | FR2 to FR2 intra-frequency TBD temporal domain case B |  | 1st goal |

The test objectives can be divided into two parts corresponding to the two goals, separately. Combine RAN2 conclusions and our understanding:

- 1st goal：

  - Test Property: Measurement overhead reduction

    - Verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

  - Metrics [TBD]:

    - Measurement reduction rate: MRRT for temporal domain; MRRS for spatial domain

    - Intermediate KPI (Accuracy): Average L3 cell RSRP difference; Average L1-RSRP difference (dB) (optional)

    - FFS: Prediction window

- 2nd goal：

  - Test Property: HO performance enhancement

    - Verify whether the HO performance gain of AI/ML model can be obtained

  - Metrics [TBD]:

    - System KPI: Performance KPIs defined in TR 36.839 as baseline for mobility performance KPIs

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Measurement event predictions

RAN2 announced that Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

- HO failure/RLF prediction (it was agreed to take RLF as the highest priority)

Two ways can carry out HO failure/RLF prediction. For the two ways, the Metrics are distinct. Combine RAN2 conclusions and our understanding:

- Direct prediction: Directly RLF prediction by AI/ML models

  - Test Property: HO performance enhancement

    - Verify whether the HO performance gain of AI/ML model can be obtained

    - Verify whether the successful/failure HO is classified correctly in datasets with class imbalance cases

  - Metrics [TBD]:

    - Classification intermediate KPI: F1 score

    - Performance KPI: Missed RLF detection, False RLF detection

- Indirect prediction: RLF prediction based on the temporal domain serving cell measurement predictions

  - Test Property:

    - Verify whether the prediction accuracy performance of AI/ML model is reliable

    - Verify whether the HO performance gain of AI/ML model can be obtained

  - Metrics [TBD]:

    - Intermediate KPI (Accuracy): Average L3 cell SINR difference

    - Performance KPI: Missed RLF detection, False RLF detection

Proposal 4: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

- RRM measurement prediction

  - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

  - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

- HO failure/RLF prediction

  - Directly RLF prediction by AI/ML models

    - To verify whether the HO performance gain of AI/ML model can be obtained

    - To verify whether the successful/failure HO is classified correctly in datasets with class imbalance cases

  - Indirectly RLF prediction based on the temporal domain serving cell measurement predictions

    - To verify whether the prediction accuracy performance of AI/ML model is reliable

    - To verify whether the HO performance gain of AI/ML model can be obtained

- FFS how to design the AI mobility specific test cases including test setup

- FFS the definition of ground truth

- Metrics/KPIs subject to RAN2’s progress and agreements

3 Conclusion

In this contribution, we provided our initial viewpoints to General discussion on AI mobility regarding testability and interoperability. The following observations and proposals are obtained:

Proposal 1: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Follow the conclusions on generalization achieved in AI/ML for NR air interface as much as possible

- Study depends on sufficient research progress in RAN2

Proposal 2: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

- The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

  - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

- RAN4 study should be aligned with the agreements in RAN1 and RAN2

Proposal 3: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case.

- Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality.

Proposal 4: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

- RRM measurement prediction

  - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

  - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

- HO failure/RLF prediction

  - Directly RLF prediction by AI/ML models

    - To verify whether the HO performance gain of AI/ML model can be obtained

    - To verify whether the successful/failure HO is classified correctly in datasets with class imbalance cases

  - Indirectly RLF prediction based on the temporal domain serving cell measurement predictions

    - To verify whether the prediction accuracy performance of AI/ML model is reliable

    - To verify whether the HO performance gain of AI/ML model can be obtained

- FFS how to design the AI mobility specific test cases including test setup

- FFS the definition of ground truth

- Metrics/KPIs subject to RAN2’s progress and agreements

4 References

[1] R4-2410570, “WF on NR ALML air interface”, Qualcomm Incorporated

[2] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm Incorporated