3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2415577

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:               General discussion on RRM requirement impact analyses on AI mobility

Agenda item:			6.18.2

Document for:	Discussion

1 Introduction

In RAN#102 meeting, the Study on AI/ML for mobility in NR was approved, and in the subsequent RAN#105 meeting, the SID was revised with the following objectives, the objective related to RAN4 is highlighted in yellow

< RP-242393 > The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

2 RAN2 progress and key points

Corresponding to the SID, the motivation of the prediction is for handover. Three aspects (/use cases) are mainly discussed: RRM measurement prediction, Measurement event predictions and HO failure/RLF prediction. The detailed information on RAN2 agreements can be found in Annex. It is worth noting that Two-sided model is not included. That means Model Training/Inference/Management/Monitoring can be performed at UE side or NW side.

2.1 RRM measurement prediction

A． During the discussions in RAN2, at least the following three sub-use cases for cell level measurement prediction model had been proposed and agreed

- Sub-use case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

- Sub-use case 2: To directly predict cell level results based on cell level results;

- Sub-use case 3: To directly predict cell level results based on beam level results

The inspiration of the three sub-use cases is from the measurement model defined in TS 38.300, below

For the above three RRM sub-use cases, all the cell level measurement result(s) refers to L3 filtered cell level measurement. While, regarding how to produce filtered L1 and L3 RSRP, i.e., per sample period or per measurement period for FR1 and the detailed value of sampling period for FR2, is pending on further discussion in RAN2. Beside of that, the L3 filtered beam level predictions can be performed in the particular case of FR2-to-FR2 intra-frequency temporal domain prediction case A but with lower priority.

Observation 1: RAN2 give first priority to L3 cell level measurement predictions, and put L3 beam level measurement predictions as low priority. For no matter FR1 or FR2, how to produce filtered L1 and L3 RSRP is still suspending

B． Based on the SID, the agreed sub-use cases can be categorized in diverse domains: FR1-to-FR1, FR2-to-FR2; space domain, time domain, frequency domain and their combinations; Intra-frequency and inter-frequency; Inter-cell and intra-cell. By means of down selection, the following scenario combinations were considered as higher and priority and evaluated for different goals.

| No. | Evaluation scenario combination | Deployment of the cell for measurement and the cell for prediction | Target study goal |
| --- | --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency intra-cell temporal domain case B (i.e., Intra_F_C_T_Case B) |  | 1st goal |
| 2 | FR1 to FR1 inter-frequency inter-cell (frequency domain) (i.e., Inter_F_C ) | - Co-located (Start from) - Same sector | 1st goal |
| 3 | FR2 to FR2 intra-frequency intra-cell temporal domain case A (i.e., Intra_F_C_T_Case A) |  | 2nd goal |
| 4 (Middle) | FR2 to FR2 intra-frequency intra-cell spatial domain (i.e., Intra_F_C_S) |  | 1st goal |

More specifically,

- 1st goal is to study AI/ML measurement prediction algorithm that can enable measurement overhead reduction

- 2nd goal is to study AI/ML measurement prediction algorithm that can enhance HO performance. And it is agreed that for the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead”

- Pure temporal domain case A/B. The BM case A and case B defined in AI/ML PHY TR 38.843 can be imitated as the baseline. However, further clarification of intra-frequency of the two temporal domain cases in RRM prediction is needed.

Besides,

- RAN2 agreed that Intra_F_Inter_C prediction, which refers to both co-located and non-colocated neighbouring cell prediction, is feasible but will be not be evaluated at least in early stage.

- So far, Intra_F_Cluster prediction has not clarified clearly

Observation 2: The following scenarios are considered in RAN2 for AI/ML RRM measurement prediction

- FR1-to-FR1

  - Intra-frequency intra-cell in temporal domain prediction for the purpose of measurement overhead reduction

  - Inter-frequency inter-cell in frequency domain prediction for the purpose of measurement overhead reduction

  - Other domain including: Intra-frequency temporal domain with 2nd goal is low prioritization

  - Cluster approach for intra-frequency prediction need to be further clarified

- FR2-to-FR2

  - Intra-frequency intra-cell in temporal domain prediction for the purpose of HO performance enhancement

  - Other domains including: Intra-frequency temporal domain with 1st goal and intra-frequency intra-cell spatial domain with 1st goal are low and middle prioritization, separately

  - Cluster approach for intra-frequency prediction need to be further clarified

- Furthermore, regarding KPI, simulation assumption and evaluation

- KPIs:

  - Average L3-RSRP difference (dB) and Average L1-RSRP difference (dB)(optional) are designed to examine the predicted capability of AI model

  - Measurement reduction rate (MRRT and MRRS)

  - FFS other KPIs

- Simulation assumption: R2-2407886

- Data collection:

  - Synthesized datasets based on 3GPP agreed channel model and deployment for evaluation. Field one is optional

  - Reuse current RAN1’s simulation assumptions AMAP by extending data generation to neighbouring cells

  - The use of random seeds in between the training and test dataset is considered

2.2 Measurement event predictions

Based on the measurement events prediction, NW could detect the potential problems for UE mobility and make the right handover decision for UE in advance. Delay the discussion until RRM measurement prediction discussions are more concrete and stable, the following observations can be obtained based on current RAN2 agreements

Observation 3: Regarding measurement event predictions

- Both direct measurement event prediction and indirect prediction are allowed

  - At least measurement event evaluation based on RRM measurement prediction result will be studied

- Start with A3 as a baseline

2.3 HO failure/RLF prediction

HOF and RLF are two important KPIs for the mobility procedures evaluation in the mobile networks. Although the rate of HOFs and RLFs are rare, occurrence of such failures could significantly deteriorate the NW QoS and UE QoE, especially in FR2 high speed scenario. Based on the AI/ML algorithm design, UE/NW can foresee the further (e.g., the quality of serving cell decrease, the time/probability of RLF/HOF occurrence in the future, etc), then to avoid the unexpected RLF/HOF. In this topic, RAN2 mainly focus on AI/ML algorithm design including direct/indirect measurement prediction, input/output of the AI/ML model corresponding to different prediction schemes, and the KPIs. And RAN2 agreed that FR2 study will be prioritized for RLF prediction

3 Discussion

3.1 Differences between AI/ML based beam management and AI/ML mobility

From our understanding, there are at least three main differences from high-level perspective:

- Motivation and KPIs

- Rel-18 AI/ML beam management

  - During Rel-18 study on AI/ML for NR air interface, RAN1 studied the performance of AI/ML prediction on beam management. The motivation of PHY BM prediction can be to help the NW to select the beam with good quality to allocate UE RACH resources and to improve UE throughput by using AI-assisted prediction to find the best or top-K beam(s) among a set of beams.

  - The Top-1/K (%) beam prediction accuracy can also be considered as KPI.

- Rel-19 AI/ML mobility

  - The ultimate motivation of AI/ML aided mobility can be to improve the system handover performance. While, since HO decision is always made by the NW, it relies on UE RRM measurement results to some extent. One main intention of intermediate AI-assisted RRM measurement prediction is to reduce UE measurement effort while simultaneously maintaining the system performance.

  - Need comprehensive consideration of intermediate KPIs and system KPIs. The former one is to evaluate the AI algorithm prediction capability and the later one is to exam the benefits.

  - The measurement reduction rate can be considered as one KPI

- Multiple domains prediction

- Rel-18 AI/ML beam management

  - During Rel-18 study on AI/ML for air interface, RAN1 studied and evaluated the spatial and temporal beam predictions based on smaller set of L1 beam measurements or historic measurements, only serving cell (intra-cell) prediction was considered.

  - The usage of frequency domain prediction was not studied due to the absence of specific use case support

- Rel-19 AI/ML mobility

  - For mobility, neighbouring cells are of the same-level significance as the serving cell since handover is a multi-cell related behaviour. The cross-cell RRM measurement contains inter-frequency and intra-frequency should be studied with certain spatial consistency.

  - For RRM measurement predication, the temporal, spatial, and frequency domain predictions are all in the scope of the SI

- Cell/beam level measurement prediction

- Rel-18 AI/ML beam management

  - During Rel-18 study on AI/ML for air interface, RAN1 studied L1 beam measurements. The predicted L1-RSRP is in BM use cases

- Rel-19 AI/ML mobility

  - On top of L1-RSRP, L3-RSRP considers L3 filtering. For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result.

  - L3 filtered beam level prediction cases are lower priority

Observation 4: There are at least three main differences between AI/ML beam management and AI/ML mobility from high-level perspective

- Motivation and KPIs

- Multiple domains prediction

- Cell/beam level measurement prediction

  - L3 filtered beam level prediction cases are lower priority in AI mobility

3.2 RRM measurement prediction

    - General aspects

In our opinion, the pivotal role of AI in BM and mobility shall be the same, that is be trained to learn the correlation in features and to predict future measurements, so the most general issues/requirements discussed in Rel-18 AI/ML RAN4 also need to be considered in Rel-19 AI mobility RAN4, for example the discussions on, baseline/benchmark performance, requirements and tests for training/inference definition, feasibility of requirements/tests for LCM evaluation, requirements for data collection, high level testing framework, AI/ML model generalization, general definition of model complexity. From this,

Proposal 1: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible

- RAN4 to discuss what general aspects could be reused to avoid the duplicate study

- RAN4 to discuss what general aspects are Rel-19 AI mobility specific, and need to be defined accordingly

    - Specific issues related to use cases for AI/ML mobility

RRM measurement prediction is a key component of AI/ML aided mobility. As the Section 2 mentioned, the first three sub-use cases for the prediction were agreed in RAN2 as higher priority, and the corresponding evaluation simulations are discussed in parallel.

Proposal 2: For RRM measurement prediction, it is suggested to consider all the three high prioritization sub-use cases for RAN4 discussion at current stage

We would like to give some analyses for the following two:

- Sub-use case 1: To predict beam level results, then generate cell level results based on the predicted beam results

The crucial link is very similar to Rel-18 AI/ML L1-RSRP per beam prediction with exception that the subsequent filtering processing is needed, while AI model is not impacted. In this sense, from our understanding,

- The UE side AI/ML model may make use of a Rel-18 AI/ML model for beam measurement predictions to obtain the predicted L1-RSRP of the same cell (serving cell-serving cell and neighbouring cell-neighbouring cell)

- Non-AI baseline of temporal domain prediction (1st goal) can reuse the approach considered in Rel-18 AI/ML BM, i.e., sample and hold based on the previous measurements specified in TR 38.843

- The framework/simulation platform for AI-PHY beam management can be utilized as much as possible

Proposal 3: For RRM measurement prediction sub-use case 1, consider AI-PHY beam management prediction framework/methodologies/requirements as starting point to study

- Sub-use case 2: To directly predict cell level results based on cell level results.

In this sub-use case, the input could be L3 filtered cell level measurement result. While, the controversial part is how to generate the L3 filtered results (i.e., how to determine the L3 filtering period). The L3 filtering period may bring effect to the datasets generation for AI model training, the number of observation instances and will influence the prediction accuracy. However, since RAN2 has not yet decided the period of L1/L3 filtering, the core and/or performance requirements, and test to be defined for sub-use case 2 could be deferred until RAN2’s conclusions

Proposal 4: For RRM measurement prediction sub-use case 2, the core and performance requirements, and test to be defined for such sub-use case could be deferred until RAN2’s conclusion

Actually, if all the possible scenarios are accounted, about 36 combinations shall be evaluated, the workload and time consumption are so huge. In this sense, RAN2 did a down-selection, Finally, they agreed:

- The modelling should include inter-cell on top of intra-cell scenarios when both serving and neighbouring beam/cell measurements are considered. Cluster approach will be also introduced if the number of input cells is more than one

- The modelling should include inter-frequency on top of intra-frequency scenarios. Measurement prediction shall be performed for the cells in the same frequency as the UE’s serving cell or for the cell in a different frequency, which correspond to intra-frequency and inter-frequency handover.

- Not only temporal (case A and case B) and spatial domain predictions, which were discussed in Rel-18 AI/ML BM, frequency domain prediction also should be considered.

Later on, three scenarios should be studied with higher priority, accordingly. From this, to achieve alignment on RAN2 scope and assumptions, we propose

Proposal 5:

For RRM measurement prediction, RAN4 RRM requirement’s study would be better to focus on the following with high priority:

- L3 measurement enhancement

- Temporal domain FR1 intra frequency

  - UE measurement reduction

- Frequency domain FR1 inter frequency

  - UE measurement reduction

- Temporal domain FR2 intra frequency

  - Handover performance enhancement

- Other scenarios should wait for more progress from RAN2

RRM requirement impacts are to be studied for core requirements, performance requirements as well as testability and interoperability requirements

Furthermore, based on the observations, a lot of issues for cell level measurement prediction, e.g., non-AI benchmark, the evaluation methodology of certain scenarios, KPIs for measurement prediction, the dataset generation, and input/output of the direct/indirect AI model had been discussed in RAN2, some of them were solved with clear conclusions. Even the simulation results evaluation for different cases based on simulation assumptions has begun in RAN2. To avoid the overlapping of similar discussions in RAN2/RAN4, from our understanding,

Proposal 6: For cell level measurement prediction, at least the following issues related to sub-use cases should subject to RAN2’s process and agreements

- Non-AI benchmark

- The evaluation methodologies for different scenarios

- KPIs for measurement prediction (e.g., prediction accuracy)

- Dataset generation for AI model training/test

- Input/output of the direct/indirect AI model

Since the simulation assumption including channel models, simulation results alignment and evaluation methodology are extensively discussed in RAN2, we propose

Proposal 7: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress

In the scope of the SID, both UE side model and NW side model measurement predictions are included. From our understanding,

- For the UE side model case, NW may configure UE to perform measurement prediction and to report the predicted measurement results (i.e., the output of the AI/ML model). In this case, the AI/ML model inference will be performed by the UE.

- For the NW side model case, NW may configure AI specific signalling. In this case, the AI/ML model inference will be performed by the NW, but the necessary input data of the AI model (e.g., historical measurement results and/or assistance information) should be generated by UE and reported to NW.

Obviously, the following alt.s can be investigated:

- AI/ML model training and inference at NW side

- AI/ML model training and inference at UE side

Since no concrete conclusions are achieved for the time being. From this,

Proposal 8: Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4

3.3 Measurement event predictions and HO failure/RLF prediction

From our understanding, one of the purposes of this discussion is to mitigate the occurrence of unintended events (e.g., ping-pong HO, too late/early/wrong HO, short ToS, RLF due to too late HO, etc.) in mobility by foreseeing the RRM status of UE. The other one is to obtain the HO performance gain especially for FR2 and/or high speed movement scenarios. In such scenarios, the legacy L3 HO mechanism provides a relatively long HO complete latency, which may also require a huge measurement effort, and even result in unnecessary event happens. Two prediction methods were agreed in RAN2, direct prediction and indirect prediction.

Furthermore, from our understanding, comparing to the legacy HO mechanism that NW decides HO after Event A3 trigger conditions are fulfilled during TTT, using AI/ML event predictions may reduce the HO execution time to some extent.

Proposal 9: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

Furthermore, based on RAN2’s conclusion, they decided to postpone Measurement event prediction discussion until RRM measurement prediction is more stabilized. RAN4 can also follow their decision at the current. From this, we propose

Proposal 10: It is suggested for RAN4 to delay the Measurement event prediction related requirements discussion until more stable conclusions are achieved in RAN2

4 Conclusion

In this contribution, we provided our initial viewpoints to RRM requirement impact analyses on AI mobility. The following observations and proposals are obtained:

Observation 1: RAN2 give first priority to L3 cell level measurement predictions, and put L3 beam level measurement predictions as low priority. For no matter FR1 or FR2, how to produce filtered L1 and L3 RSRP is still suspending

Observation 2: The following scenarios are considered in RAN2 for AI/ML RRM measurement prediction

- FR1-to-FR1

  - Intra-frequency intra-cell in temporal domain prediction for the purpose of measurement overhead reduction

  - Inter-frequency inter-cell in frequency domain prediction for the purpose of measurement overhead reduction

  - Other domain including: Intra-frequency temporal domain with 2nd goal is low prioritization

  - Cluster approach for intra-frequency prediction need to be further clarified

- FR2-to-FR2

  - Intra-frequency intra-cell in temporal domain prediction for the purpose of HO performance enhancement

  - Other domains including: Intra-frequency temporal domain with 1st goal and intra-frequency intra-cell spatial domain with 1st goal are low and middle prioritization, separately

  - Cluster approach for intra-frequency prediction need to be further clarified

Observation 3: Regarding measurement event predictions

- Both direct measurement event prediction and indirect prediction are allowed

  - At least measurement event evaluation based on RRM measurement prediction result will be studied

  - Start with A3 as a baseline

Observation 4: There are at least three main differences between AI/ML beam management and AI/ML mobility from high-level perspective

- Motivation and KPIs

- Multiple domains prediction

- Cell/beam level measurement prediction

  - L3 filtered beam level prediction cases are lower priority in AI mobility

Proposal 1: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible

- RAN4 to discuss what general aspects could be reused to avoid the duplicate study

- RAN4 to discuss what general aspects are Rel-19 AI mobility specific, and need to be defined accordingly

Proposal 2: For RRM measurement prediction, it is suggested to consider all the three high prioritization sub-use cases for RAN4 discussion at current stage

Proposal 3: For RRM measurement prediction sub-use case 1, consider AI-PHY beam management prediction framework/methodologies/requirements as starting point to study

Proposal 4: For RRM measurement prediction sub-use case 2, the core and performance requirements, and test to be defined for such sub-use case could be deferred until RAN2’s conclusion

Proposal 5:

For RRM measurement prediction, RAN4 RRM requirement’s study would be better to focus on the following with high priority:

- L3 measurement enhancement

- Temporal domain FR1 intra frequency

  - UE measurement reduction

- Frequency domain FR1 inter frequency

  - UE measurement reduction

- Temporal domain FR2 intra frequency

  - Handover performance enhancement

- Other scenarios should wait for more progress from RAN2

RRM requirement impacts are to be studied for core requirements, performance requirements as well as testability and interoperability requirements

Proposal 6: For cell level measurement prediction, at least the following issues related to sub-use cases should subject to RAN2’s process and agreements

- Non-AI benchmark

- The evaluation methodologies for different scenarios

- KPIs for measurement prediction (e.g., prediction accuracy)

- Dataset generation for AI model training/test

- Input/output of the direct/indirect AI model

Proposal 7: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress

Proposal 8: Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4

Proposal 9: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

Proposal 10: It is suggested for RAN4 to delay the Measurement event prediction related requirements discussion until more stable conclusions are achieved in RAN2

5 Annex

Relevant RAN2 Agreements for RRM measurement prediction

<RAN2 #125-bis> Agreements - For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction： RSRP difference to the actual measurement Temporal prediction：RSRP difference to the actual measurement measurement reduction rate as one KPI - As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed <RAN2 #126> Agreements on simulation assumptions 1 For 2nd study goal i.e. to enhance handover performance, evaluation exercise will focus on FR2 to FR2 intra-frequency scenario. 2 For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases 4 for RRM sub case 1, it is up to company to report L1 RSRP difference 5 Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured 6 For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation. 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS. 9 Intra-frequency intra-cell temporal domain prediction can be applied for all RRM sub cases. And it is up to company to report applied RRM sub case together with simulation result. 10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS. 11 For both Intra-frequency and inter-frequency inter-cell prediction, the measurement on cell for measurement should not be reduced in both temporal and spatial domain 12 For Inter_F_C (inter-frequency inter-cell), RAN2 start evaluation from co-located scenario 13 for Inter_F_C(inter-frequency inter-cell), RAN2 should focus on the case where cell for measurement and cell for prediction are in the same sector. 14 FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub cases. And it is up to company to report applied RRM sub cases together with their simulation result. 16 Intra_F_Inter_C (intra-frequency inter-cell) prediction will not be evaluated at least in early stage. Intra_F_Inter_C(Intra-frequency inter-cell) refers to both co-located and non-colocated neighbouring cell prediction. 17 No traffic model is simulated 18 During simulation UE is dropped 100% outdoor 19 It is up to company’s implementation to select how to distribute the UE 20 Fast fading is necessary for RRM sub case 1 and 3. FFS case 2 21 To agree not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8) and blockage (7.6.4) for channel modelling (38.901) 22 LOSsoft is optionally modelled in the channel modelling 23 To agree on following parameters for FR2: 1, 30GHz as central frequency 2, 200m as ISD 3, UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 as baseline channel modelling 24 For FR1, following parameters are agreed: 1, Table 6.2.1-1 template is taken as starting point ( to be updated by individual proposal later on) 3, to set up 2-tier model (7 sites, 3 sectors/cells per site) (20/20) 4, 500m as ISD 5, channel modelling is UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 6, 20MHz as bandwidth 7, The recommendated value in yellow in table 2.3.3-1 ( by removing wording “At least for BM-Case1,”) 25 Section 7.6.5 in 38.901 is taken as baseline for inter-frequency correlation model. Whether inter-frequency correlation model is used is optional and companies can report what they use. FFS on the understanding shadowing correlation in inter-freq. For now companies should report what assumption they have made 26 Following RRC parameters need be aligned as simulation parameters: (17/20) - RRC parameters for measurement consolidation - RRC parameters for L3 filtering (filter coefficient, measurement period) 27 We apply same L1 sampling period for both intra and inter (i.e. we don’t simulate existence of measurement gap as starting point) 28 Simulation assumptions discussed in section 2.3.1~2.3.3 is taken as baseline also for use cases other than RRM measurement prediction. Any update is subject to further discussion on other use cases. 29 The sample period(s) are aligned among companies for intra-frequency intra-cell temporal domain prediction. We can start with 20ms for FR2 and 40ms for FR1. Measurement period: FFS – suggestion from rapporteur is to start with 480ms for FR2 and 200ms for FR1 30 Simulation parameters in table 2.3.4-1 (by removing Table A.2.1-7 and Table 2.1-10) are taken as starting point for both UE sided model and network sided model for FR2. The number of beams could be left for company to report. FFS Use 100Mhz for channel BW? 31 For FR1, following parameters are agreed: 2, {4GHz,30KHz} as frequency for intra-frequency scenario and {2GHz, 15KHz(FDD)/30GHz(TDD)} as another frequency for inter-frequency scenario Agreements 1 average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged 2: Measurement reduction rates e.g. 1/2, for both intra-frequency intra-cell temporal domain prediction case B and spatial domain prediction. Revision in RAN2#127 is open. 3: One prediction window for FR1 and FR2 respectively as starting point. The detail value to be decided in the post email discussion. 4: UE trajectory option is up to company’s implementation and report 5: UE trajectory boundary processing is up to company’s implementation and report 6: For study goal 2, the candidate speeds are 60,90,120 km/h and company can report UE speed along with simulation result 7: For study goal 1, the candidate speeds are 30,60,90km/h and company can report UE speed along with simulation result 8: To decide on the values in table 2.4-2/3/4 in post email discussion. L3 filtering parameter for both FR1 and FR2; Recommended value. FilterCoefficient; 4. Table 2.4-2 Measurement period; Recommended value. FR1 to FR1 intra-frequency w.o. gap; 200ms. FR1 to FR1 inter-frequency with gap; 120ms. FR2 to FR2 intra-frequency w.o. gap; 480ms. Table 2.4-3 Consolidation parameter; Recommended value. nrofSS-BlocksToAverage for FR1; 1. nrofSS-BlocksToAverage for FR2; 3. absThreshSS-BlocksConsolidation; FFS. Table 2.4-4 9: To agree on 80MHz as bandwidth for FR2 10: To agree on 10m for BS antenna height for FR2 11: Fast fading is necessary for RRM sub case 2 too. 12: The shadowing fading of two different frequency layers can be reported by company 13: To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain Agreements The following is agreed Case number; Prioritization; Evaluation scenario combination; target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell. Agreements - For the cell level measurement prediction, start with consider a fixed value for L3 filtering in simulation. FFS which fixed value - For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result <RAN2 #127> Agreements => companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected. => For the agreed frequencies for inter-frequence case, only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h. Companies can consider other speeds for other frequencies if they chose to simulate them. => For temporal domain case B prediction the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s). Agreements - To keep two filtering options on the table and up to company to report. 2 One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. 3 In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement 4 continue to discuss following issues in the post email discussion: 1, Further clarification of intra-frequency of temporal domain case A and case B. 2, The set of observation vs prediction window parameters for intra-frequency temporal domain case A and case B 3, The number of TX and RX for FR1 and FR2 4, Filtering co-efficient for beam level prediction Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

Relevant RAN2 Agreements for Measurement event predictions

<RAN2 #125-bis> Agreements - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed - Clarifications on what is being as input should be provided with results - Start with A3 as a baseline. - Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

Relevant RAN2 Agreements for RLF HO failure prediction

<RAN2 #125-bis> Agreements - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed - Clarifications on what is being as input should be provided with results - Start with A3 as a baseline. - Measurement event prediction study can start after some further progress on RRM measurement prediction has been made <RAN2 #126> Agreements - Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). - Study Direct: Directly RLF prediction by AI/ML models. - FR2 study will be prioritized for RLF prediction. - The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. - HOF prediction is downprioritized in our study. NO simulations/evaluations should be done/submitted - RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case. - No evaluation/simulations are expected for August meeting for RLF - Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible). <RAN2 #127> Agreements - Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations Agreements on KPI For indirect - SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction. Direct - missed RLF detection, false RLF detection, F1 score. - Whether the RLF will happen based on the following methodology. Above a probability threshold we assume that RLF will happen. This is then compared with true RLF. - For the time being we don’t need HO procedure simulation in RLF simulation - FFS full buffer and assumption that all the cells are fully loaded. We will not simulate traffic. - Simulation results are not expected before February

Relevant RAN2 Agreements for Simulation assumption and evaluation methodology

<RAN2 #125-bis> Agreements to start evaluations - FR1-to-FR1 - Focus on intra-frequncy in time domain prediction for the purpose of measurement reduction - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible). - FR2-to-FR2 - Focus on intra-frequency - Perform evaluation both in time and spatial domain Agreements - AI mobility SI uses synthesized datasets based on 3GPP agreed channel model and deployment for evaluation. Field data is optional - Reuse current RAN1’s simulation assumptions as much as possible by extending data generation to neighbouring cells. - Once a set of simulation parameters and assumptions per each sub-use case (e.g., propagation scenario, deployment topology, channel modelling, UE trajectories, etc.) are settled, it should be used for baseline case (i.e. without AI/ML model), training (e.g. data set generation), validation, and inference etc. - Clarify and document the use of random seeds in between the training and test dataset, simulation drops/runs at least for channel modelling and UE trajectory. - Alignment of simulation assumptions is necessary, but explicit result calibration (e.g., as in TR 36.839) is not expected. Companies can independently report their gains achieved by AI/ML with detailed evaluation descriptions for cross-checking purposes. - For FR1, band n77/n78 is considered with 4GHz as the central frequency. FFS any other band - For FR2, only FR2-1 is considered, e.g., band n257. 30GHz central frequency can be adopted to reuse RAN1’s work as much as possible. FFS any other band - focus on Urban Macro (UMa) for FR1 and Umi for FR2 - RAN2 takes hexagonal regular topology as the starting point. - Take baseline simulation assumptions from Table 6.3.1-1 in TR 38.843 for FR2 as the starting point for channel modelling, e.g., BS/UE antenna configuration, BS Tx power, and BS/UE antenna height. UE rotation is excluded in the initial phase of evaluation. - UE trajectory model uses options 1-3 in TR 38.843 section 6.3.1 as the starting point. Down-selection to be discussed in email discussion - AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found.

6 References

[1]