# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415579

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	6.1.1.2.1

Document for:	Approval

# Background

In RAN4#112, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed which mainly focuses on the approach that converts full RB allocation in UE CBW to “inner RB allocation” with an new emission bandwidth.

In this contribution, we provide our further views on the MPR reduction via relaxed ACLR/SEM/spurious emission modification.

# Mechanism for MPR reduction via a new emission BW

In RAN4#112, the approach of defining a “new emission BW” to scale the emission limit was discussed intensively. In principle, the applicable frequencies of emission limits (both in-band and out-of-band) in 3GPP that depend on the UE-specific CBW shall be scaled according to this “new emission BW,” including IBE, SEM, ACLR, and SE, as illustrated in Fig. 1.

Figure. 2. Illustration of Rel-19 single CC MPR reduction approach based on a new emission bandwidth

In our view, such an approach can effectively reduce the MPR via the transfer of the outer/edge RB allocation in legacy UE-specific CBW to the inner RB allocation of the new emission BW.

Observation 1: The MPR can be reduced via the transfer of the outer/edge RB allocation in legacy UE-specific CBW to the inner RB allocation of a “new emission BW”.

Proposal 1: To reduce the MPR, define a new emission BW, and scale the applicable frequencies of general emission limits, including IBE, SEM, ACLR, and SE, according to this new emission BW.

# Applicable of the new emission BW

If the new emission BW edge exceeds the BS CBW edge, it may cause potential issues where out-of-band protection can not be guaranteed. Therefore, it is proposed that the new emission BW should not go beyond the BS CBW.

Observation 2: Allowing extended UE CBW go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection.

Proposal 2: To ensure the protection out of the frequency band, the new emission CBW should not exceed the BS CBW.

Meanwhile, emission limits that currently do not depend on UE CBW, for example, the spurious emission for co-existence or emission limits due to regulatory requirements, shall not be changed to ensure the 3GPP requirements can provide sufficient protection between different frequency bands and be compatible with regional regulatory requirements.

Proposal 3: No changes to spurious emission for co-existence or emission limits due to regulatory requirements with the new emission BW.

Then, the next question is how small the new emission limit bandwidth needs to be so that the UE can achieve sufficient MPR reduction. By looking at the MPR currently specified for PC3 and PC2, it can be seen that about 1 to 1.5 dB gain can be obtained by transfer to the edge/outer RB to inner RB while there is no gain or very limited gain (~ 0.5 dB) by transfer the edge RB to outer RB allocations. Therefore, it is reasonable to focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated. In other words, the “new emission BW” shall be at least 1.5* (legacy UE CBW).

Proposal 4: Focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated, in which the “new emission BW” is at least 1.5* (legacy UE CBW).

Meanwhile, RAN4 can consider that if the “new emission BW” is smaller than the 1.5* (UE CBW), UE shall apply the emission limit according to the legacy UE CBW.

Proposal 5: the UE shall apply the emission limit according to the legacy UE CBW if the “new emission BW” is smaller than 1.5* (UE CBW).

Another aspect regarding the applicable BW of the “new emission BW” is that the “new emission BW” can be totally transparent to the UE as long as if “new emission BW” is one of the CBW that the UE can support. The network can always configure the UE with a UE channel BW but with limited RB allocations, and it will be equivalent to the case illustrated above as a “new emission BW”. On the other hand, new UE behaviour, e.g., MPR reduction, needs to be specified if the new emission BW is wider than the maximum UE CBW it can support.

Proposal 6:  No specification impact of the “new emission BW” if it is one of the CBWs that the UE can support.

Proposal 7:  RAN4 focuses on the case that “new emission BW” is wider than the maximum UE CBW that it can support.

# Signalling of the new emission BW

With having defined the operation mechanism and applicable bandwidth of the new emission BW above,  how to signal this “new emission BW” to UE is still an open issue. In RAN4#112, two approaches have been proposed: one is to define a new “extended UE CBW” while the other is to adopt the BS CBW, which has already been included in SIB1 as carrierBandwidth. Both approaches have pros and cons, and we summarize our analysis in Table I.

Table I. The pros and cons of a different definition of the “new emission BW.”

| The “new emission BW” definition | Pros | Cons |
| --- | --- | --- |
| extended UE CBW | It can be used more flexibly, which can support the scenario that the operator spectrum is wider than a single BS CBW | Additional signaling overhead and specification work, such as the concept of extended UE CBW, does not exist as of today. |
| BS CBW | Lower signaling overhead and minimal spec impact as BS CBW has already been indicated in SIB1 as carrierBandwidth | The MPR reduction scenario is limited to a single BS CBW. |

Observation 3: From a signaling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

RAN4 shall take into the the pros and cons into consideration and determine how to define the new emission BW.

# Performance gain

In addition to the methodology study, the actual performance gain of supporting this reduced MPR also needs to be evaluated. Focus on the scenario that a narrower UE channel BW within wider BS bandwidth, allocating a device with narrow bandwidth, e.g., Redcap and eRedcap, towards the inner location within an operator’s spectrum block may not be a common scenario in real life to avoid spectrum fragmentation from the network aspect. In addition, since such an operation scenario requires the operator spectrum block to be wide enough, it may not be feasible for most FDD bands, especially those at sub-1GHz.

Observation 4: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 5: The frequency bands are usually small in FDD bands in sub 1 GHz, which makes it  not being feasible to move the UE away from the edge of the band.

Proposal 8: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

Figure. 2. The illustration of UE spectrum emission requirements in NR

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. As background information, the UE out-of-band emission limits are illustrated in Fig. 2. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Observation 6: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 9: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Observation 1: The MPR can be reduced via the transfer of the outer/edge RB allocation in legacy UE-specific CBW to the inner RB allocation of a “new emission BW”.

Observation 2: Allowing extended UE CBW go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection.

Observation 3: From a signaling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

Observation 4: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator’s spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 5: The frequency bands are usually small in FDD bands in sub 1 GHz, which makes it  not being feasible to move the UE away from the edge of the band.

Observation 6: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 1: To reduce the MPR, define a new emission BW, and scale the applicable frequencies of general emission limits, including IBE, SEM, ACLR, and SE, according to this new emission BW.

Proposal 2: To ensure the protection out of the frequency band, the new emission CBW should not exceed the BS CBW.

Proposal 3: No changes to spurious emission for co-existence or emission limits due to regulatory requirements with the new emission BW.

Proposal 4: Focus on the case that the “new emission BW” is large enough to accommodate the legacy UE CBW to be fully inner allocated, in which the “new emission BW” is at least 1.5* (legacy UE CBW).

Proposal 5: the UE shall apply the emission limit according to the legacy UE CBW if the “new emission BW” is smaller than 1.5* (UE CBW).

Proposal 6:  No specification impact of the “new emission BW” if it is one of the CBWs that the UE can support.

Proposal 7:  RAN4 focuses on the case that “new emission BW” is wider than the maximum UE CBW that it can support.

Proposal 8: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

Proposal 9: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability.

# References

- R4-2412818	Topic summary for [112][116] NR_ENDC_RF_Ph4_part1	Moderator(Huawei)

- R4-2414441	WF on power domain enhancement		Huawei

- R4-2408794 	Discussion on power domain enhancements for single carrier	ZTE Corporation, Sanechips

- RP-233279	MediaTek Views on RAN4 Rel-19: RF and OTA	MediaTek Inc.