# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415582

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	6.23.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were discussed in RAN4 #112 [1], and a WF was agreed [2]. According to the WF, the link level simulation assumption for ACS/ASCS evaluation has been agreed upon, but some prominent issues for REFSENS, which includes reference architecture, NF, and IM, still need to be further discussed.

In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- REFSENS for LP-WUR

## Reference architecture for LP-WUR

## Baseline architecture to derive the minimum requirement

In RAN4#112, it has been agreed to use the zero-IF receiver as a baseline architecture for envelope-based (ED-based) LP-WUR, while for the OFDM-based architecture, it is still pending for RAN1’s conclusion on the OFDM-based LP-WUR [2]. The agreement aligns with the progress of the WI, but in our view, it may not be necessary to consider both types of receivers when it comes to minimum requirement discussion.

Per the outcome of the study item phase in TR38.869, the NF of zero-IF based ED receiver is in the range of 12-16 dB, which is significantly higher than the NF of OFDM receiver that was typically assumed for UE in FR1 (in the range of 9-12 dB). Therefore, it can be expected that the REFSENS of the ED-based receiver will be worse than the OFDM-based receiver, and RAN4 may consider focusing on the ED-based receiver to derive the minimum requirement.

Proposal 1: RAN4 may consider focusing on the ED based receiver to derive a single set of minimum requirements on REFSENS for both ED type and OFDM type of receivers.

## Impact on REFSENS due to Antenna sharing

Another issue that has been brought up in the last couple of meetings is the consideration of antenna sharing [3], where a reference architecture is shown in Fig. 1.

Fig. 1. The reference architecture of zero-IF LP-WUR with antenna sharing with the MR.

In our view, antenna sharing would be necessary if the RRM measurement needs to be offloaded to the LP-WUR. Otherwise, the measured results would not be valid due to the possible different channels observed by different antennas. In such a case, an antenna switching circuit needs to be implemented, where additional insertion loss may need to be considered, and the NF that is going to be used for deriving the REFSENS requirement shall take this into account.

Observation 1: Antenna sharing would be needed if the RRM measurement from the MR needs to be offloaded to the LP-WUR.

Proposal 2: When deriving the NF of LP-WUR, the insertion loss of antenna switching for antenna sharing with MR needs to be taken into account.

Practically, the antenna switching is already taken into account in the MR REFSENS. Therefore, such an antenna-sharing scheme should not affect the performance of MR, and thus, no impact on the MR performance should be considered.

Proposal 3:  No impact on MR requirements should be considered.

## SNR definition to derive the REFSENS

One more open issue regarding the general receiver requirement is how to define the SNR value to derive the REFSENS. As we know, the REFSENS can be calculated as in (1) with respect to a threshold SNR level. Therefore, it is critical that RAN4 agrees on the definition of SNR in the equation first.

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM                                         (1)

The interface on defining SNR to derive the REFSENS should be carried by the input at the digital BB processing, which is illustrated in Fig. 1. This is the minimum SNR value that the digital BB can detect the incoming sequence correctly with a given MDR and FAR rate.

Proposal 4: The SNR by the input at the digital BB processing should be used to derive the REFSENS requirement.

## REFSENS performance metric

## REFSENS performance metric

In RAN4#110bis, the following agreements for the performance metric of REFSENS have been made:

Although the miss detection rate (MDR) has been agreed as the sole metric for LP-WUS Rx RF requirement, the MDR value is still for FFS. Besides the 1% MDR that has been used as a simulation assumption for determining SNR during the study phase, a higher percentage MDR, e.g., 5% or 10%, was also proposed in the discussion.

The reason for choosing a higher percentage instead of a lower value (e.g., 1%) is to facilitate testing, in our understanding. To obtain a 1% test results, it is intuitive to understand that at least 100 LP-WUS sequences are required to be measured in the conformance test. Moreover, based on the rule of thumb, the number of tests is preferred to be even ten times the least required number of sequences, e.g., 1000 LP-WUS sequences may be needed to obtain the performance of 1%MDR. Therefore, a high percentage, e.g., 10%, might be more feasible from the testability of view.

Observation 2	Adopting a relatively small percentage value on the MDR may lead to an excessive test time.

However, one concern regarding adopting a larger percentage in the core requirement, which needs to be discussed further, is how to ensure that the REFSENS requirement defined in RAN4 fulfills the coverage target of LP-WUS. It is our understanding that the LP-WUS should reach the same coverage of Msg. 3 with 1% MDR. Therefore, one possibility is still to define the core requirement according to 1% MDR and scale the corresponding REFSENS level to 10 % MDR (e.g., scaling according to different SNR levels of 1% and 10% MDR) in the conformance test.

Observation 3	The target coverage of LP-WUS is to meet Msg. 3 coverage with 1% MDR.

Proposal 5	Use 1% MDR as REFSENS metric for LP-WUR as a starting point to define the core requirement and further study if a higher percentage can be used while fulfilling the coverage target of LP-WUS.

Proposal 6	RAN4 may consider adopting a higher percentage MDR value, e.g., 5 % or 10 %, in the conformance test by scaling the REFSENS level accordingly.

## Calibration of simulation results

One particular issue discussed in RAN4#112 but without any conclusion is whether the SNR condition in the LLS simulation shall first calibrate with a fixed FAR value (e.g., 0.1% FAR for all simulations) among all the companies. For a given sequence, a false wake-up happens when the UE falsely detects noise or  sequences for other UEs as its own wake-up sequence. Physically, this would happen only when the unintended sequence is interrupted so that it appears to be very similar to the correct wake-up sequence. With a sequence with good auto- and cross- correlation properties, it can be expected that the probability of such a scenario is very low, and thereby the FAR iw very low, e.g., 0.1%, 0.01%. In addition, with improved sequence design and detection algorithm, the FAR value can be even further reduced.

Although it is possible to vary the FAR values with different interference and noise levels, the resulting SNR level may become unrealistic depending on the sequence and detection algorithm design to meet a target FAR value. Therefore, we don’t think it is a proper way to align the FAR value for the purpose of simulation calibration; rather, we should focus on defining an upper limit of the FAR value to ensure the MDR simulation results are valid.

Observation 4: Enforcing the FAR value to a certain level may result in a totally unrealistic SNR condition for some sequence and/or receiver design.

Proposal 7: Only define an upper limit FAR value for simulation to ensure the MDR simulation results are valid.

A more proper way to align the simulation results, especially the SINR condition, would be looking into the bit error rate (BER), whose performance has nothing to do with the sequence or the detector design.

Observation 5: If calibration on SINR condition would be needed, it would be better to consider the BER that is irrelevant to the sequence or receiver design.

## One or two sets of requirements

In addition, the number of requirements has also been discussed, and one proposal is to define sets of requirements to accommodate receivers for both ED-based and OFDM-type LP-WUS. In general, both types of WUS signal and the corresponding receivers need to support sufficient coverage of LP-WUS (as PUSCH of Msg 3). In addition, as we already discussed in section 2.1.1, the OFDM-based LP-WUR is unlikely to perform worse than ED-based LP-WUR due to improved NF and threshold SNR. Having in mind that RAN4 only defines minimum requirements, it should be sufficient to define only one set of requirements covering all types of receivers.

Observation 6 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Proposal 8	RAN4 should aim to define one set of minimum requirements covering all types of LP-WUS receivers.

- ACS and ASCS Requirement

## Requirement for ACS and ACSC

From the network co-existence aspect, the LP-WUS is expected to co-exist with other signals and suffer the same level of interference as a normal NR receiver. Thus, setting the same ACS level for the LP-WUS receiver as MR would be reasonable.

Observation 7	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Proposal 9	Define the ACS requirement for LP-WUS as 33 dB.

Meanwhile, ACSC's requirement is to tolerate the in-band interference that the network can control, so it could potentially be more relaxed than ACS. Therefore, RAN4 should further investigate if ACSC requirements should be set like ACS requirements.

One proposal on ACSC from RAN4#112 is to only define the test case but no ACSC requirement value. For legacy ACS requirements, although a requirement on ACS value is defined (33 dB), but since it is difficult to test the ACS value directly, the core requirement is actually verified by two test cases with low and high interference level but without testing the ACS level itself.

Therefore, for ASCS, it is a question if RAN4 still needs to put effort on defining an ASCS value itself or just focus on defining one or two test cases to ensure the LP-WUR receiver performance.

Proposal 10	considering the ACS is not measurable but its performance is verified indirectly with a few test cases on MR, it is a question if RAN4 needs to define a value of ACSC or rather focus on defining a test case.

Once RAN4 agrees on the level of ACS (or potentially also ASCS pending on the discussion above), further simulation and analysis can be done to derive the required number of guard RB with some practical filter assumptions.

Proposal 11 	RAN4 shall derive the number of guard RB based on some practical filter assumption once the ACS/ASCS requirement is agreed.

## Initial link-level simulation results

In this section, we present initial simulation results for ASCS analysis with OOK-4 waveform. Our simulations are based on the simulation assumption for link level simulation (LLS) agreed during RAN4#111, shown below:

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Chip rate | M=4 |
| WUS duration | 126 bits ~ 1mSec |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0 |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 |
| WUS RB | - Fixed 11RB ~ 3.96MHz |
| Position within channel | - Center of the channel |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth |
| Filter | - 5th Order Butterworth matching fixed 3.96MHz RF bandwidth |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same average PSD with WUS signal |
| Sampling rate | 7.68MHz |
| ADC bit width | Ideal ADC |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Power boosting | No power boosting |
| Channel Model | TDL-C 300 and AWGN |
| Performance metric | 1% MDR with false alarm rate below 1% |

As indicated in the table, the LP-WUS waveform is generated based on OOK-4 principles and for M = 4. To allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity, it is beneficial that the LP-WUS includes a preamble prior to its data information. Another advantage of adding such a preamble is reduced power consumption as well false detection at the UE while monitoring for LP-WUS. The LP-WUR first needs to only look for the preamble and only continues to detect the data if the preamble is detected. Fig. 2 shows the general signal structure used in this contribution containing sequences of length K and L bits, representing preamble and data, respectively, where data part is to indicate the subsequent action by the UE after LP-WUS detection and/or identity of (group) UE. Figure 3 illustrates a general block diagram of the digital baseband processing used to detect the LP-WUS in this contribution.

Fig. 2  LP-WUS structure, consisting of two fields, preamble used for synchronization and possibly cell identity purposes and data part to indicate the potential subsequent action or UE (group) identity.

Fig. 3  Digital baseband processing.

In our simulation, LP-WUS design has the following characteristics:

- Kasami sequences of equal length of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 1 msec.

- Overlaid OFDM sequence with length 16 consisting of an m-sequence of length 15 and a random single bit. The overlaid OFDM sequence is generated based on option 1-1 agreed in RAN1, i.e., before DFT/LS block.

- The LP-WUS occupies 16 OFDM-symbols, 11RBs (1 RB GB at each side of the LP-WUS bandwidth) with subcarrier spacing of 30 kHz.

We have used ideal ADC but it is expected that the ADC performance would not degrade our MDR performance significantly. The order of lowpass Butterworth is set to 5 and we assume no frequency and timing error.

We study the results, BER and MDR, for AWGN and TDL-C 300 channel for two guard band settings of 0 RB and 1 RB on each side of the LP-WUS bandwidth. The achieved false-alarm rate for the studied scenarios is below 0.1%, thanks to LP-WUS structure, sequence design, and DBB processing.

Fig. 4  Bit error rate (BER) for AWGN (left) and TDLC (right) channel models.

Fig. 5  Miss detection rate (MDR) for AWGN (left) and TDLC (right) channel models.

Based on this initial simulation results, it can be observed that the number of guard RB does not affect the MDR performance significantly.  Additionally, the processing gain from the sequence design can compensate for performance loss of the LP-WUR due to its ultra-low power design, allowing to reach the target coverage level even at relatively high BER.

Observation 8	The number of guard RB may not affect the performance significantly in certain receiver designs.

Observation 9	To compensate for performance loss (high NF) of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low misdetection and false-alarm error probabilities.

We can also observe that the performance under TDL-C channel is worse than the AWGN channel, which may be caused by the frequency dispersive response under TDL channel so that more NR signals fall into the WUS RB allocations. In RAN4#112, it was agreed to use AWGN channel to define the RF requirements for LP-WUR, which is reasonable considering the actual test condition is more AWGN like. However, if RAN4 also needs to define the number of guards RB that could be used in the real field, it may not be sufficient to consider only the AWGN channel condition.

Observation 10	The performance of LP-WUR is worse in TDL channel than AWGN channel.

Proposal 12	RAN4 shall determine which channel model is more appropriate to be used to derive the number of guard RBs that can be used in real life.

- OBB/IBB and other receiver requirements

Similar logic to ACS, it is expected that the blocking signal level in IBB and OBB should be the same as the MR. The exact location of the blocking signal can be further discussed. Assuming the LP-WUS receiver is capable of meeting the MR ACS requirement with configured guard RBs, it is proposed that the configured guard RB should also be configured in the OBB and IBB test., e.g., the offset of blocking signal for LP-WUS OBB/IBB test is shifted further away from the LP-wanted signal with the number of guard RB than in the MR case.

Proposal 13	Define the blocking signal level the same as the MR for LP-WUR, and further study if the offset of the blocking signal to the wanted signal should be shifted with guard RB.

- Side Condition and Testability of LP-WUS

The side condition of the LP-WUS requirement has also been brought up in RAN4#111, and whether RAN4 should consider both idle and connected mode conditions in the LP-WUS requirement was discussed. For most of the devices, it is our understanding that the LP-WUS is targeted mainly for idle mode. Meanwhile, the connected mode of LP-WUS is designed to support XR devices due to its unique traffic model, and it is unclear how this would affect the RF requirements. Therefore, it is suggested that RAN4 focus on the idle mode and FFS if the connected mode needs to be addressed separately later on and if any impact on the RF requirement of LP-WUR is identified.

Observation 11	Connected mode LP-WUS is mainly for XR devices.

Proposal 14	RAN4 can focus on the idle mode first and FFS if the connected mode needs to be addressed separately later on if any impact on the RF requirement of LP-WUR would be identified, at least in FR1.

For the testability, the following agreement has been made in RAN4#112:

Issue 4-1-1: General framework on LP-WUS testing

Agreements:

  - RAN4 further discusses the high-level approach how to achieve LP-WUS performance verification.

From the testability aspect, different from the legacy REFSENS or ACS test for the MR, there will be no ACK/NACK for the LP-WUS receiver, and therefore, how the test equipment (TE) shall be able to count the missed detection rate is unclear. One possible solution is to implement a dedicated test mode to allow the UE to record whenever it detects an intended wake-up sequence and report this value to TE once the test is done.

Observation 12	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

As an alternative, 3GPP may also investigate whether succeeding transmissions (e.g., PRACH) on the main radio can be detected by the TE once the UE has been woken up. For the sake of test time, it would be preferred that no connection mode needs to be created. However, it would be a question of how the UE LP-WUR would behave if there is no response from the TE on the MR after a few attempts.

Proposal 15	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1: Antenna sharing would be needed if the RRM measurement from the MR needs to be offloaded to the LP-WUR.

Observation 2	Adopting a relatively small percentage value on the MDR may lead to an excessive test time.

Observation 3	The target coverage of LP-WUS is to meet Msg. 3 coverage with 1% MDR.

Observation 4: Enforcing the FAR value to a certain level may result in a totally unrealistic SNR condition for some sequence and/or receiver design.

Observation 5: If calibration on SINR condition would be needed, it would be better to consider the BER that is irrelevant to the sequence or receiver design.

Observation 6 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Observation 7	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Observation 8	The number of guard RB may not affect the performance significantly in certain receiver designs.

Observation 9	To compensate for performance loss (high NF) of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low misdetection and false-alarm error probabilities.

Observation 10	The performance of LP-WUR is worse in TDL channel than AWGN channel.

Observation 11	Connected mode LP-WUS is mainly for XR devices.

Observation 12	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

Proposal 1: RAN4 may consider focusing on the ED based receiver to derive a single set of minimum requirements on REFSENS for both ED type and OFDM type of receivers.

Proposal 2: When deriving the NF of LP-WUR, the insertion loss of antenna switching for antenna sharing with MR needs to be taken into account.

Proposal 3:  No impact on MR requirements should be considered.

Proposal 4: The SNR by the input at the digital BB processing should be used to derive the REFSENS requirement.

Proposal 5	Use 1% MDR as REFSENS metric for LP-WUR as a starting point to define the core requirement and further study if a higher percentage can be used while fulfilling the coverage target of LP-WUS.

Proposal 6	RAN4 may consider adopting a higher percentage MDR value, e.g., 5 % or 10 %, in the conformance test by scaling the REFSENS level accordingly.

Proposal 7: Only define an upper limit FAR value for simulation to ensure the MDR simulation results are valid.

Proposal 7: Only define an upper limit FAR value for simulation to ensure the MDR simulation results are valid.

Proposal 8	RAN4 should aim to define one set of minimum requirements covering all types of LP-WUS receivers.

Proposal 9	Define the ACS requirement for LP-WUS as 33 dB.

Proposal 10	considering the ACS is not measurable but its performance is verified indirectly with a few test cases on MR, it is a question if RAN4 needs to define a value of ACSC or rather focus on defining a test case.

Proposal 11 	RAN4 shall derive the number of guard RB based on some practical filter assumption once the ACS/ASCS requirement is agreed.

Proposal 12	RAN4 shall determine which channel model is more appropriate to be used to derive the number of guard RBs that can be used in real life.

Proposal 13	Define the blocking signal level the same as the MR for LP-WUR, and further study if the offset of the blocking signal to the wanted signal should be shifted with guard RB.

Proposal 14	RAN4 can focus on the idle mode first and FFS if the connected mode needs to be addressed separately later on if any impact on the RF requirement of LP-WUR would be identified, at least in FR1.

Proposal 15	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

- References

R4-2412835		Topic summary for [112][133] NR_LPWUS_UERF	Moderator(VIVO)

R4-2414309		WF on LP-WUS UE RF requirements	Vivo

R4-2407894		Discussion on REFSENS for LP-WUR	Samsung